3GPP TSG RAN WG4 #92bis	R4-1912749

# Chongqing, China, 14th Oct – 18th Oct, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on corrections for MIMO Correlation Matrices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-10-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - α should be α1 and α2 before the table B.2.3.2.2-1. - In LTE, Medium Correlation for XP 1D was defined as Medium-A and for XP 2D was defined as Medium correlation. As we have unified the definition in 38.101-4, we should just define one Medium correlation with both α1 and α2 defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - α is replaced with α1 and α2 before the table B.2.3.2.2-1. - Medium-A is changed to Medium correlation with α2 defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MIMO correlation matrices for cross polarized antennas will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### B.2.3.2.2	MIMO Correlation Matrices using cross polarized antennas

The values for parameters α1, α2, β and γ for the cross polarized antenna models are given in Table B.2.3.2.2-1.

Table B.2.3.2.2-1: The α and β parameters for cross-polarized MIMO correlation matrices

| Correlation Model | 1 | 2 |  |  |
| --- | --- | --- | --- | --- |
| Medium Correlation | 0.3 | 0.3 | 0.6 | 0.2 |
| High Correlation | 0.9 | 0.9 | 0.9 | 0.3 |
| Note 1: Value of α1 applies when more than one pair of cross-polarized antenna elements in first dimension at gNB side. Note 2: Value of α2 applies when more than one pair of cross-polarized antenna elements in second dimension at gNB side. Note 3: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |  |  |

For the 1D cross polarized antenna array at gNB side, the correlation matrices for high spatial correlation and medium correlation are defined in Table B.2.3.2.2-2 and Table B.2.3.2.2-3 as below.

The values in Table B.2.3.2.2-2 have been adjusted to insure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

or

Where the value “a” is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 8x2 high spatial correlation case, a=0.00010.

Table B.2.3.2.2-2: MIMO correlation matrices for high spatial correlation

| 4x2 case |  |
| --- | --- |
| 8x2 case |  |

<< End of changes >>