3GPP TSG-RAN WG4 Meeting #92bis	R4-1912753

# Chongqing, China, 14- 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | --- | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on demodulation performance requirements for EN-DC including FR1 and FR2 CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no demodulation performance requirements specified for EN-DC including both FR1 and FR2 NR CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify FR1-FR2 EN-DC demodulation performance requirements following the existing principles to define EN-DC (only verifying the NR CC performance) and FR1-FR2 band combination (only verifying FR2 NR CC performance) performance requirements. In that way the UE supporting EN-DC including FR1 NR and FR2 NR CCs shall be verified under the EN-DC fallback mode including FR1 NR CC only or FR2 NR CC only against the corresponding EN-DC performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for EN-DC including both FR1 and FR2 CCs are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1, 9.2B.1.3, 9.3B.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

9	Demodulation performance requirements for interworking

9.1	General

This clause covers the UE demodulation performance requirements for EN-DC, NE-DC, inter-band NR-DC between FR1 and FR2, and inter-band NR CA between FR1 and FR2.

9.1.1	Applicability of requirements

The following applicability rules are specified for demodulation performance requirements for interworking:

-	For UEs supporting both SA and NSA,

-	The performance requirements specified in Section 5 will be verified only for SA except for the sustained downlink data rate test specified in Section 5.5 and 5.5A.

-	The performance requirements specified in Section 7 will be verified only for SA except for the sustained downlink data rate test specified in Section 7.5 and 7.5A.

-	The sustained downlink data rate tests specified in Sections 5.5, 5.5A and 7.5, 7.5A for SA and in Section 9.4B for NSA are verified separately.

-	The FR1 EN-DC test cases with the NR TDD DL-UL configurations which are not aligned with LTE's can be tested on the corresponding EN-DC band combinations where UE supports simultaneous transmission and reception.

-	For UEs supporting NR FR1 CA and NR CA including FR1 and FR2, the requirements applicability is specified in Table 9.1.1-1.

Table 9.1.1-1: Requirements applicability for UEs supporting NR FR2 CA and NR CA including FR1 and FR2

| Supported scenarios | Requirements |
| --- | --- |
| NR FR2 CA | Section 7.5A |
| NR CA including FR1 and FR2 | Section 9.4A.1 |
| Both NR FR2 CA and NR CA including FR1 and FR2 | Section 7.5A |

-	For UEs supporting EN-DC including FR2 and EN-DC including FR1 and FR2, the requirements applicability is specified in Table 9.1.1-2.

Table 9.1.1-2: Requirements applicability for UEs supporting EN-DC including FR2 and EN-DC including FR1 and FR2

| Supported scenarios | SDR requirements | PDSCH requirements | PDCCH requirements |
| --- | --- | --- | --- |
| EN-DC including FR2 | Section 9.4B.1.2 | Section 9.2B.1.2 | Section 9.3B.1.2 |
| EN-DC including FR1 and FR2 | Section 9.4B.1.3 | Section 9.2B.1.3 | Section 9.3B.1.3 |
| Both EN-DC including FR2 and EN-DC including FR1 and FR2 | Section 9.4B.1.2 | Section 9.2B.1.2 | Section 9.3B.1.2 |

9.1.1.1	Applicability of requirements for optional UE features

For UE which supports optional UE features the additional performance requirements from Table 9.1.1.1-1 should be applied.

<< Next changes>>

9.2	PDSCH Demodulation

9.2A	PDSCH demodulation for CA

9.2A.1	NR CA between FR1 and FR2

(Void)

9.2B	PDSCH demodulation for DC

9.2B.1	EN-DC

9.2B.1.1	EN-DC within FR1

9.2B.1.1.1	PDSCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDSCH demodulation performance requirements for NR are specified in Section 5.2. During the test, only the PDSCH performance on the NR cell(s) shall be verified.

9.2B.1.2	EN-DC including FR2 NR carrier only

9.2B.1.2.1	PDSCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDSCH demodulation performance requirements for NR are specified in Section 7.2. During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers shall be verified.

9.2B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDSCH demodulation performance requirements for NR are specified in Section 9.2B.1.1 and Section 9.2B.1.2. During the test, only the PDSCH performance on the NR cell(s) on FR2 carriers shall be verified.

9.2B.2	NR DC between FR1 and FR2

9.3	PDCCH demodulation

9.3A	PDCCH demodulation for CA

9.3A.1	NR CA between FR1 and FR2

(Void)

9.3B	PDCCH demodulation for DC

9.3B.1	EN-DC

9.3B.1.1	EN-DC within FR1

9.3B.1.1.1	PDCCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDCCH demodulation performance requirements for NR are specified in Section 5.3. During the test, only the PDCCH performance on the single NR cell shall be verified.

9.3B.1.2	EN-DC including FR2 NR carrier only

9.3B.1.2.1	PDCCH

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDCCH demodulation performance requirements are specified in Section 7.3. During the test, only the PDCCH performance on the single NR cell shall be verified.

9.3B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PDCCH demodulation performance requirements are specified in Section 9.3B.1.1 and Section 9.3B.1.2. During the test, only the PDCCH performance on the NR cell(s) on FR2 carriers shall be verified.

9.3B.2	NR DC between FR1 and FR2

<< End of changes >>