3GPP TSG-RAN WG4 RAN4#92bis 	R4-1912785

Chongqing, China, October 14th – 18th, 2019

Agenda item:	8.5.4.2

Source: 	Qualcomm Incorporated

Title: 	Ad hoc meeting minutes on IAB RRM

Document for:	Discussion

# RRM/RLM Requirements for IAB Nodes

## Background

| Requirements | Comments | Is requirement needed? |
| --- | --- | --- |
| Cell reselection | - Intra-frequency - Inter-frequency | No |
| Handover |  | FFS - Yes - No |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | Yes |
| SCell activation/deactivation delays |  | No |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No |
| PSCell addition/release delays |  | No |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | No |
| TCI switching | TCI switching delay | FFS |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | FFS for PCell and/or PSCell |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based beam failure detection and candidate beam detection | FFS for PCell and PSCell |
| Measurements requirements (time duration, number of cells, etc.) |  | FFS for L1-RSRP evaluation period. Needs further discussion for L3-RSRP evaluation period. |
| Measurement accuracy requirements |  | FFS for L1-RSRP Needs further discussion for L3-RSRP |
| MT timing related requirements | - MT maximum receive timing difference - MT transmit timing - MT maximum transmit timing difference - Timing Advance | FFS: MT transmit timing Timing Advance MT maximum receive timing difference MT maximum transmit timing difference |
| Switching time between MT and DU | --- | FFS |
| DU timing related requirements | - +- 3 us requirement - T_delta command | Yes (+- 3 us requirement / cell phase synch) FFS for Tdelta command |

## Ad Hoc Discussion

| Requirements | Comments | Is requirement needed? |
| --- | --- | --- |
| Cell reselection | - Intra-frequency - Inter-frequency | No |
| Handover |  | FFS - Yes - No |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | Yes |
| SCell activation/deactivation delays |  | No |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No |
| PSCell addition/release delays |  | No |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | No |
| TCI switching | TCI switching delay | FFS |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | FFS for PCell and/or PSCell |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based beam failure detection and candidate beam detection | FFS for PCell and PSCell |
| Measurements requirements (time duration, number of cells, etc.) |  | FFS for L1-RSRP evaluation period. Needs further discussion for L3-RSRP evaluation period. |
| Measurement accuracy requirements |  | FFS for L1-RSRP Needs further discussion for L3-RSRP |
| MT timing related requirements | - MT maximum receive timing difference - MT transmit timing - MT maximum transmit timing difference - Timing Advance | MT transmit timing (Te and Tq) Timing Advance FFS: MT maximum receive timing difference MT maximum transmit timing difference |
| Switching time between MT and DU | --- | FFS |
| DU timing related requirements | - 3 us requirement - T_delta command | Yes (3 us requirement / cell phase synch) FFS for Tdelta command |

Question from moderator: Without L1-RSRP accuracy requirement, how can we ensure that L1-RSRP reporting works?

E//: We don’t need requirement because we have requirements for RRC reestablishment. Beam may not change frequently because it’s a fixed node.

QC: Without requirement, how can we ensure beam management, e.g. beam change, is happening properly?

Samsung: For FWA, we need these requirements. Why can’t we have beam management requirement for IAB nodes? We can’t say that beam switch will never happen.

QC: Fully agree with Samsung.

Nokia: Is Samsung commenting about mobility or beam management?

Samsung: For Rel-16, we want beam management related requirement. For Rel-17, we may need full set of RRM requirements.

E//: We shouldn’t delay deployment.

Huawei: Agree with Ericsson.

Qualcomm: Having requirements will ensure performance.

E//: We don’t have time

QC: If we keep postponing, we won’t have sufficient time.

NOK: What is the minimum level of requirement for beam mgmt.?

Samsung: Can we have a middle ground for beam mgmt. requirement?

Qualcomm: Agree with SS. We may not need measurement eval period and accuracy. But, we will need TCI switch, RLM (with relaxed timeline), CBD

E//: Is RLM related to regulatory requirement?

QC: Yes.

E//: Aperiodic RSRP support can be considered.

## Ad Hoc Agreements

Following agreements were obtained in addition to the agreements that were previously achieved in RRM session.

| Requirements | Comments | Is requirement needed? |
| --- | --- | --- |
| MT timing related requirements | - MT maximum receive timing difference - MT transmit timing - MT maximum transmit timing difference - Timing Advance | MT transmit timing (Te and Tq) Timing Advance FFS: MT maximum receive timing difference MT maximum transmit timing difference |
| DU timing related requirements | - 3 us requirement - T_delta command | Yes (3 us requirement / cell phase synch) FFS for Tdelta command |