3GPP TSG-RAN4 Meeting #92bis	R4-1912894

# Chongqing, China, October 14th -18th, 2019

Source:	Ericsson

Title:	TP to TR 38.883: Section 5 System level simulation

Agenda item:	8.12.2

Document for:	Approval

# 1.	Introduction

Revision of R4-1911802 to remove conclusion section based on online feedback discussion and decision to separate conclusion section in separate TP

# 2.	References

[1]	R4-1910479, "TP for TR 38.883: System level simulation results for the feasibility study of FR2 DL 256QAM" (China Telecom).

[2]	R4-1910478, “TR skeleton for TR 38.883: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2)” (China Telecom).

[Start of Text Proposal]

### 5.2.2	Simulation results

#### 5.2.2.x	Results from company x

The following results are based upon the assumptions outlined in Section A.x

It has been discussed that there are two scenarios that should focus the study for indoor and urban micro where both scenarios are LOS between UE and BS.  The rationale for studying only these two edge case scenarios is that these are the only possible scenarios for deployment for millimeter wave that may experience benefit of higher MCS.

With an observed user data rates higher than 170 Mbps, there is a 15% benefit due to higher modulation.  For UEs utilizing 256 QAM an SINR upwards of 30 dB is needed.  For higher modulation a small power back off 3dB is applied.

Figure 5.2.2.x-1: System-level macro base station scenario SINR distribution and observed user throughput

For the above simulation results, the average user throughput for users with 64 QAM is 77.2 Mbps and for 256 QAM is 83.7 Mbps this gives an overall system gain of 8% for those users utilizing the higher modulation.  The average user throughput considered is calculated by sum(all observed user throughput)/(total number of users observed).

The simulation results below come from the indoor scenario where the results yield no benefit of 256 QAM over 64 QAM.  Both user throughput and SINR distributions are equal.

Figure 5.2.2.x-2: System-level indoor base station scenario SINR distribution and average user throughput

The overall average cell throughput gains in the above simulations are 6% and 3% for macro and indoor solutions respectively.  Although the capacity gains are modest, there are limited circumstances that during peak data rates

[Unchanged Sections]

# Annex <A> (informative):
System level simulation assumptions

# A.x Assumptions from company x

# Macro scenario:

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Macro site distance |  | 200 m |  |
| Number of sites |  | 19 |  |
| Frequency range |  | 30GHz |  |
| Beamforming |  | Yes |  |
| Simulation bandwidth |  | 200MHz |  |
| Number of UEs in the network |  | 1 active UE/sector |  |
| Traffic Model | Full buffer |
| Channel Model | nr-uma from TR 38.900 |
| Propagation conditions | LOS |
| gNB Tx power |  | 33 dBm for macro. |  |
| gNB antenna height |  | 25m for macro cells |  |
| gNB receiver noise figure |  | 10 dB |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 1) |  |
| UE Tx power (dBm) |  | 22.4 dBm |  |
| UE noise figure (dB) |  | 10 |  |

Indoor:

| Parameters |  |
| --- | --- |
| Indoor layout | Open office 50x120m |
| Number of sites | 6 |
| Frequency range | 30GHz |
| Simulation bandwidth | 200MHz |
| Number of UEs in the network | 1 active UE/sector |
| Traffic Model | Full buffer |
| Channel Model | nr-inh-open from TR 38.900 |
| Propagation conditions | LOS |
| gNB Tx power | 23 dBm |
| gNB receiver noise figure | 10 dB |
| UE Tx power (dBm) | 22.4 dBm |
| UE noise figure (dB) | 10 |

[End of Text Proposal]