3GPP TSG-RAN WG4 Meeting #92bis	R4-1913001

# Chongqin, China, 14th– 18th Oct, 2019

Source: 	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	TP for TR 37.823: PRB Alignment

Agenda Item:	7.10.2

Document for:	Approval

# 1	Introduction

In last RAN4#92 meeting, a WF for Rel-16 LTE-MTC coexistence with NR is agreed [1], where it specifies that the PRB alignment should be captured in the TR. This paper proposes the TP for this topic.

# 2	References

[1] R4-1907849, WF for Rel-16 LTE-MTC coexistence with NR

# 3	Text Proposal

## <Start of Text Proposal>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1] 	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190770, “Revised WID for Additional MTC enhancements for LTE”

## [3]		     3GPP TS 36.211, Physical channels and modulation, v13.2.0

## [4]                        R1-1903885, Coexistence of LTE-MTC with NR, Ericsson, RAN1#96bis

## <Skip Unchanged Parts>

## 7.2 PRB alignment

### 7.2.1 Impact of LTE-MTC Narrowband Operation

LTE-MTC UEs reuse legacy LTE PSS/SSS/PBCH for initial cell selection, which occupies the centre 72 subcarriers in the system BW excluding the DC subcarrier. After MSG1 in the random access procedure, the UE may use only 6 PRBs in a narrowband for transmissions and receptions if no frequency-hopping is configured. Otherwise hopping between multiple narrowbands may be performed. The mapping between the narrowband and the PRB is defined in 3GPP TS 36.211 [3] Section 6.2.7. Figure 7.2-1 illustrates the mapping for system BW up to 5 MHz.

Figure 7.2.1-1 Mapping between LTE PRB and Narrowband

For up to at least Rel-15 LTE-MTC operation in NR in-band, the number of NR PRBs reserved for LTE-MTC should be large enough to accommodate all the PRBs within the LTE transmission BW configuration. Particularly, the PUCCH channel in the UL always uses the edge PRBs within the system BW.

### 7.2.2 Intuitive Analysis

Consider the case of SCS=15 kHz for NR. A total of +1 consecutive NR PRBs in the DL need to be reserved for LTE-MTC owing to the unused DC subcarrier. The set of allowed values for  is given by 3GPP TS 36.104. Depending on the parity of , the DC subcarrier is either in the middle of a LTE PRB or in between two LTE PRBs. It’s easily seen that LTE PRBs can be aligned with NR PRBs if the following conditions are satisfied, and at most half of the PRBs in the LTE system BW can be aligned:

- The LTE DC subcarrier coincides with

  - the lowest (highest) subcarrier, i.e. subcarrier index 0 (11) of a NR PRB if  is even;

  - the middle subcarrier, i.e. subcarrier index 6 (5) of a NR PRB if  is odd;

- The LTE PRB is on the lower (upper) side of the LTE DC subcarrier.

In the UL, the LTE DC subcarrier is a valid subcarrier that belongs to a PRB and a 7.5 kHz frequency shift is employed to mitigate the effect of carrier leakage. Hence, the LTE PRBs may be aligned with the NR PRBs only if the operating NR band also supports 7.5 kHz frequency shift. Furthermore, owing to the restrictions of TX-RX frequency separation as defined in 3GPP TS 36.101, the aforementioned conditions on the DL DC subcarrier should be met so that the LTE DL PRBs on the lower side of the DC subcarrier are aligned with the NR DL PRBs. As a result, all of the LTE UL PRBs can be aligned with the NR UL PRBs.

Figure 7.2-2 illustrates an example of 1.4 MHz LTE carrier inside a NR carrier. In the DL, the LTE DC subcarrier coincides with the subcarrier 0 of an NR PRB. As a result, LTE PRB 0, 1 and 2 are aligned with NR PRBs. In the UL, all LTE PRBs are aligned with NR PRBs after 7.5 kHz shift. Additionally, Figure 7.2-3 illustrates an example of 3 MHz LTE carrier inside a NR carrier. In the DL, the LTE DC subcarrier coincides with the subcarrier 6 of an NR PRB. As a result, LTE PRB 0-6 are aligned with NR PRBs. In the UL, all LTE PRBs are aligned with NR PRBs after 7.5 kHz shift.

Figure 7.2.2-1: Example PRB alignment for a 1.4 MHz LTE carrier (having one LTE-MTC NB) inside a NR carrier

Figure 7.2.2-2: Example PRB alignment for a 3 MHz LTE carrier (having two LTE-MTC NBs) inside a NR carrier

If neither condition #1 nor condition #2 is satisfied, the number of so-called “outlying” subcarriers in the DL LTE carrier would be more than one. The outlying subcarriers are those in the lower/upper edge of the LTE carrier and only partially overlap with a NR RB. Assuming that the LTE DC subcarrier coincides with subcarrier index r, (0≤r≤11) within a NR RB, the number of outlying subcarriers can be derived as:

Table 7.2.2-1: Number of outlying subcarriers in LTE-MTC DL

|  | Lower edge | Upper edge |
| --- | --- | --- |
| LTE is even | mod(12-r, 12) | mod(r+1, 12) |
| LTE is odd | mod(6-r, 12) | mod(r+7, 12) |

### 7.2.3 Quantitative Analysis

Let NRBNRand NRBLTE denote the maximum number of RBs in the NR and LTE carrier, respectively. Let FREFNR and FcLTE denotes the RF reference frequency for NR and carrier frequency for LTE, respectively. It’s known that:

- NR RF reference frequency FREFNR corresponds to subcarrier 0 of RB NRBNR2 if NRBNR is even, and it corresponds to subcarrier 6 of RB NRBNR2 if NRBNR is odd;

- LTE carrier frequency FcLTE corresponds to the DC subcarrier with extra 7.5 kHz offset in the UL. In the DL, it locates between two adjacent RBs if NRBLTE is even, and it locates in the middle of a RB if NRBLTE is odd.

In order to optimize resource utilization, it is desired to align LTE PRBs with NR PRBs (in fact at most half of the PRBs in the DL and all in the UL as described in Section 7.2.2). In the following, we further formulate the conditions described in Section 7.2.2 based on different NR channel raster sizes.

7.2.3.1 NR 100 kHz Channel Raster

For subcarrier alignment (SCS=15 kHz), the NR RF reference frequency and LTE carrier frequency should satisfy the following condition:

100m-100n=15k          (7.2.3.1-1)

with:

- m being an integer identifying the LTE carrier on the 100 kHz raster, i.e. FcLTE=100m kHz;

- n being an integer identifying the NR carrier on the 100 kHz raster, i.e. FREFNR=100n kHz;

- k being an integer representing the subcarrier offset from the NR RF reference frequency.

Re-write Eq. (7.2.3.1-1) as:

100(m-n)=15k                   (7.2.3.1-2)

For the sake of the variable k being an integer, the value of m-n has to be a multiple of 3, i.e., m-n=3p, with p being an integer. Therefore, Eq. (7.2.3.1-2) can be re-written as:

k=20p , with p being an integer 	(7.2.3.1-3).

This means that the offset of the LTE carrier from the NR RF reference frequency should be a multiple of 20 subcarriers (i.e., a multiple of 300 kHz). We also have k>0 if the LTE carrier is on the upper side of the NR carrier and k<0 if the LTE carrier is on the lower side. Next we further derive the LTE carrier position in terms of RB offset as well as subcarrier index within a RB.

If NRBNR is even, the NR RF reference frequency FREFNR corresponds to subcarrier 0 of RB NRBNR2. Hence, we have:

- q≜k12, the RB offset from the NR RB containing the subcarrier corresponding to FREFNR

- r≜mod(k,12),  0≤r≤11,  the index of the subcarrier corresponding to FcLTE

With these definitions, we re-write k=20p as:

20p=12q+r,            (7.2.3.1-4)

or r=4(5p-3q)	(7.2.3.1-5).

It can be seen that valid values for subcarrier index r are: 0, 4 and 8. Note that the value of RB offset q is limited by the NR channel bandwidth, i.e. -NRBNR2≤q≤NRBNR2. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.1-4). Further considering PRB alignment, the condition #1a as described in Section 7.2.2 may be met if additional constraint of r=0 is added. However, the condition #1b as described in Section 7.2.2 can never be met since r=6 or 5 is not valid. This is also indicated by the number of LTE outlying subcarriers corresponding to NR subcarrier index r=0, 4 or 8 as shown in Table 7.2.3.1-1, which matches the observation in RAN1 [4].

Table 7.2.3.1-1: Number of outlying subcarriers in LTE-MTC DL when the DC subcarrier coincides with NR subcarrier index r=0, 4 or 8 (NRBNR is even)

|  | Lower edge | Upper edge |
| --- | --- | --- |
| LTE is even | 0, 8, 4 | 1, 5, 9 |
| LTE is odd | 6, 2, 10 | 7, 11, 3 |

Note: The value shown in bold corresponds to the condition #1a in Section 7.2.2.

On the other hand, if NRBNR is odd, the NR RF reference frequency FREFNR corresponds to subcarrier 6 of RB NRBNR2.

Taking this offset into account, we have q≜k+612 for RB offset and r≜mod(k+6, 12) for subcarrier index. And Eq. (7.2.3.1-3) can be re-written as:

20p+6=12q+r,            (7.2.3.1-6)

or r=2*(10p-3*(2q-1))	(7.2.3.1-7).

It can be seen that valid values for subcarrier index r are: 2, 6 and 10. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.1-6). Further considering PRB alignment, the condition #1b as described in Section 7.2.2 may be met if additional constraint of r=6 is added. However, the condition #1a as described in Section 7.2.2 can never be met since r=0 or 11 is not valid. This is also indicated by the number of LTE outlying subcarriers corresponding to NR subcarrier index r=2, 6 or 10 as shown in Table 7.2.3.1-2, which matches the observation in RAN1 [4].

Table 7.2.3.1-2: Number of outlying subcarriers in LTE-MTC DL when the DC subcarrier coincides with NR subcarrier index r=2, 6 or 10 (NRBNR is odd)

|  | Lower edge | Upper edge |
| --- | --- | --- |
| LTE is even | 10, 6, 2 | 3, 7, 11 |
| LTE is odd | 4, 0, 8 | 9, 1, 5 |

Note: The value shown in bold corresponds to the condition #1b in Section 7.2.2.

7.2.3.2 NR 15 kHz Channel Raster

Corresponding to the E-UTRA bands that support LTE-MTC, most of the NR counterparts use 100 kHz channel raster. So far, only band n41 uses SCS-based channel raster. For the sake of PRB alignment, only SCS=15 kHz is considered. Considering subcarrier alignment, the NR reference frequency and LTE carrier frequency should satisfy the following condition:

100m-15n=15k,		(7.2.3.2-1)

with:

- m being an integer identifying the LTE carrier on the 100 kHz raster, i.e. FcLTE=100m kHz;

- n being an integer identifying the NR carrier on the 15 kHz raster, i.e. FREFNR=15n kHz;

- k being an integer representing the subcarrier offset from the NR RF reference frequency.

Rewrite Eq. (7.2.3.1-1) as:

100m=15(n+k).                   (7.2.3.2-2)

This means that the LTE carrier should be on a raster of 300 kHz, i.e. m=3p, with p being an integer. Together with Eq. (7.2.3.2-2), we have:

k=20p-n.                   (7.2.3.2-3)

Reusing the definition of RB offset q and subcarrier index r in Section 7.2.3.1, we have:

20p-n=12q+r, if NRBNR is even,           (7.2.3.2-4)

20p-n+6=12q+r, if NRBNR is odd.           (7.2.3.2-5)

Note that the values of p and q are limited by the NR channel bandwidth, e.g. -NRBNR2≤q≤NRBNR2. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.2-4) or Eq. (7.2.3.2-5), for a given NR reference frequency, i.e. fixed n. Further considering PRB alignment, the conditions on subcarrier index r as described in Section 7.2.2 should be met as additional constraints. Note that such a solution may not always exist.

### 7.2.4 Summary

### The conditions for PRB alignment between LTE-MTC carrier and NR carrier are discussed. Section 7.2.2 provides an intuitive analysis, which is confirmed by the quantitative analysis in Section 7.2.3. Additionally, more insights are acquired through the numerical analysis, which are summarized in the following tables.

Table 7.2.4-1: Conditions for subcarrier alignment between NR and LTE-MTC

| NR Channel Raster Size | Condition | Note |
| --- | --- | --- |
| 100 kHz | Eq (7.2.3.1-3) | The frequency offset between the LTE-MTC carrier and the NR carrier should be multiple of 300 kHz. |
| 15 kHz | Eq (7.2.3.2-3) | The LTE-MTC carrier frequency should be a multiple of 300 kHz. |

Table 7.2.4-2: Constraints on the location of LTE-MTC inside NR in terms of RB offset and subcarrier index

| NR Channel Raster Size | NRBNR is even | NRBNR is odd |
| --- | --- | --- |
| 100 kHz | Eq (7.2.3.1-4) | Eq (7.2.3.1-6) |
| 15 kHz | Eq (7.2.3.2-4) | Eq (7.2.3.2-5) |

Based on the above listed conditions, it is also found that the choice of the NR subcarrier index (within a RB) for placing the LTE-MTC carrier is limited. And different choices result in different number of outlying subcarriers in the LTE-MTC carrier, for which the minimization is usually desired in order to optimize spectrum utilization. Table 7.2.4-3 summarizes the observations for 100 kHz NR channel raster size.

Table 7.2.4-3: Observations on the NR subcarrier index suitable for LTE MTC and the corresponding number of outlying subcarriers (NR channel raster = 100 kHz)

|  | NRBNR is even | NRBNR is odd |
| --- | --- | --- |
| Subcarrier index within a RB | 0, 4, 8 | 2, 6, 10 |
| Number of outlying subcarriers | Table 7.2.3.1-1 | Table 7.2.3.1-2 |

## <End of Text Proposal>