3GPP RAN WG4 Meeting #92bis		R4-1913006

Chongqing, China, 14 – 18th October 2019

Agenda item:	10.4.5

Source:	Apple Inc.

Title:	TP to TR38.820: corrections of deployment scenarios

WI/SI:	FS_7to24GHz_NR

Release:	Rel-16

Document for:	Approval

# 1	Introduction

The study on the 7 – 24 GHz frequency range [1] has made progress in identifying the technological feasibility of NR operation in this frequency range and has captured the latest agreements on the potential deployment scenarios in TR38.820 v0.2.0 [2].

This contribution aligns the assumptions on the UE antenna configuration in the deployment scenarios section of the TR (Clause 5.6) with the summary of UE RF technology aspects (Clause 6.3.1) and also corrects some editorial errors in the clause.

# 2	References

- RP-182884, "New SID on 7 - 24 GHz frequency range"

- R4-1909280, TR38.820 "7 - 24 GHz frequency range," Huawei, 3GPP RAN4 #92

# 3	Text proposal

<< start of text proposal >>

## 5.6	Deployment scenarios

Editor’s note: Considerations on possible deployment scenarios in this range, considering technology capabilities. Set of deployment scenarios captured below is subject to the RF technology feasibility studies (both BS and UE) and shall not be considered as the conclusion of the SI.

Referring to the previous NR discussions, three high level use cases were defined for NR: eMBB, URLLC and mMTC. Considering NR operation in 7 – 24 GHz range, the following was observed:

- eMBB: as 7 – 24 GHz range is considered to be mostly a capacity booster, the eMBB is a valid use case for 7 – 24 GHz range NR operation.

- URLLC: this use case includes such advanced services like e.g. factory automation, autonomous driving, and remote surgery, including both indoor and outdoor case. Therefore applicability of the URLLC use cases in 7 – 24 GHz required case by case analysis, due to potential coverage limitation restriction.

- mMTC: this NR use case requires good penetration, including deep indoor and wide area coverage. Due to propagation characteristics of 7 – 24 GHz frequencies, the mMTC use case is not seen as well suited for 7 – 24 GHz due to potential coverage limitations.

Referring to the NR study item, range of deployment scenarios were considered and documents in TR 38.913 [x], including both FR1 and FR2 spectrum allocations. NR scenarios from TR 38.913 [x] were analysed for the 7 – 24 GHz applicability, considering the carrier frequency assumptions. Additionally, IAB scenario was included based on related Rel-16 work item.

For the 7 – 24 GHz deployment scenarios discussion it is valid to consider their potential dependencies from the existing incumbent users and spectrum environment in general. Therefore, for each of the potential deployment scenarios, it is essential to also consider information on the (regional) incumbent/neighbour systems information. For more information on the application and incumbent users, refer to subclause [x].

Below, brief description of the considered 7 -24 GHz deployment scenarios is captured:

### 5.6.1	Indoor hotspot

Indoor hotspot scenario addresses an indoor case with relatively small coverage, high capacity and high user throughput driven by high density of low-mobility users. For the NR study, the carrier frequencies of 4 GHz, 30 GHz and 70 GHz were considered. Therefore, the whole 7 – 24 GHz range (with all three sub-ranges being applicable for this scenario) is seen as suitable for this scenario. This scenario is expected to be addressed by the LA BS class.

### 5.6.2	Dense urban

Dense Urban scenario addresses outdoor and outdoor-to-indoor case in dense urban, interference-limited scenarios, based on macro grid coverage with continuous cellular layout, extended with randomly dropped micro sites, i.e. two-layer layout (i.e. macro + micro). This scenario is characterized by high user density of low and medium mobility, and high traffic loads. For the NR study, the carrier frequencies of 4 GHz and 30 GHz were considered.

This scenario is expected to be addressed by the WA and MR BS classes. This scenario is well suited for CA or DC application.

### 5.6.3	Urban macro

Urban macro scenario addresses an outdoor and indoor case with large cells and continuous coverage, serving users of low and medium mobility.

For the NR study, the carrier frequencies of 2 GHz, 4 GHz and 30 GHz were considered. Therefore, the whole 7 – 24 GHz range (with all three sub-ranges being applicable for this scenario) is seen as suitable for this scenario.

The difference to Dense urban scenario is that the Urban macro is a one layer macro layout.

This scenario is expected to be addressed by the WA BS class. It shall be noted, that WA BS class was defined also for FR2 operation.

High speed train: TBD

Highway scenario: TBD

Urban Grid for Connected Car: TBD

IAB: TBD

### 5.6.4	Summary

Table 5.6.4-1 below summarizes the NR deployment scenarios for the 7 – 24 GHz frequency range.

Table 5.6.4-1: NR deployment scenarios for 7 – 24 GHz frequency range

| NR scenario | Carrier frequency (GHz) | Maximum aggregate system BW (DL+UL) (MHz) | UE antenna configurations (Tx and Rx antenna elements) | BS antenna array size (Tx and Rx antenna elements) | Scenario layout | User characteristic | ISD (m) | Scenario description | BS class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot | 7÷24 | ≤[200÷1000] | TBD | ≤ [256] | Single layer, indoor only | 100% Indoor, 3km/h, 10 users/TRxP | 20 | Indoor, small coverage, high capacity, high user throughput, high user density | LA |
| Dense urban | 7÷24 | ≤[200÷1000] | TBD | ≤ [256] | Two layer, macro hex grid, random drop micro | Uniform/macro TRxP, 10 users/TRxP, Clustered/micro TRxP, 80% indoor (3km/h), 20% outdoor (30km/h) | 200 | Outdoor and outdoor-to-indoor, high user densities, high traffic loads, dense urban areas, continuous cellular layout, interference-limited | WA, WA+MR |
| Urban macro | 7÷24 | ≤[200÷1000] | TBD | ≤ [256] | Single layer, hex grid, macro | 20% Outdoor in cars: 30km/h, 80% Indoor: 3km/h 10 users per TRxP | 500 | large cells and continuous coverage, interference-limited | WA |
| High speed train | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Highway scenario | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Urban Grid for Connected Car | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| IAB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

NOTE 1: Summary of all parameters is not intended for the system level simulation purposes, rather for the scenarios characteristics comparison.

NOTE 2: 	Values of the aggregated system BW and BS antenna array sizes, as well as user characteristics were reused from the NR deployment scenarios in TR 38.913 [x].

<< end of text proposal >>