3GPP TSG-RAN WG4 Meeting #92bis	R4-1913021

# Chongqing, China, 14 – 18 October, 2019

Source: 	Huawei

Title:	TP to TR 38.820: NR BS transmitted signal quality requirements

Agenda Item:	10.4.8.3

Document for:	Approval

# Introduction

Based on related discussion in [1], TP to TR 38.820 on the transmitted signal quality requirements is provided for approval.

# References

[1]		R4-1912216			NR BS output power dynamics for 7-24 GHz range

[2]		 38.820, v0.2.0

# Draft TP to TR 38.820

In this section we provide Draft TP to TR 38.820.

------------------------------ Modified section ------------------------------

## 7.4	BS RF requirements

Editor’s note: Study RF characteristics for example frequencies including both TX and RX. In particular:

•	Study how to design TX emissions mask for these frequencies

•	Study the spurious region and its upper frequency limit for practical measurements

•	Study how to design TDD TX switching time requirement

•	Study need for requirement coverage; e.g. co-location related requirements (TX IMD etc.) for each example frequency

•	Study how to design requirements where approach differs between FR1 and FR2; e.g. RX sensitivity requirement, RX ACS, blocking, RX IMD requirements

### 7.4.1	Transmitter requirements

#### 7.4.1.1	Tx requirements overview

Summary of the conducted and radiated Tx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below.

Table 7.4.1.1-1: Overview of conducted Tx requirements for NR BS in 7 – 24 GHz range

| Tx requirement |  |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- | --- |
| Base station output power |  |  |  |  |
| Output power dynamics |  | RE power control dynamic range |  |  |
|  |  | Total power dynamic range |  |  |
| Transmit ON/OFF power |  | Transmitter OFF power |  |  |
|  |  | Transmitter transient period |  |  |
| Transmitted signal quality |  | Frequency error | Requirements can be reused from FR1. |  |
|  |  | Modulation quality | QPSK, 16QAM, 64QAM requirements levels can be reused from FR1. |  |
|  |  | Time alignment error | Requirements for the MIMO transmission and for the inter-band CA (with or without MIMO) can be reused from FR1. | Derive intra-band CA TAE requirements, based on the set of the supported SCS. |
| Occupied bandwidth |  |  |  |  |
| ACLR |  |  |  |  |
| Operating band unwanted emissions |  |  |  |  |
| Transmitter spurious emissions | General transmitter spurious emissions |  |  |  |
|  | Protection of the BS receiver of own or different BS |  |  |  |
|  | Additional spurious emissions requirements |  |  |  |
|  | Co-location with other base stations |  |  |  |
| Transmitter intermodulation |  |  |  |  |

Table 7.4.1.1-2: Overview of radiated Tx requirements for NR BS in 7 – 24 GHz range

| OTA Tx requirement |  |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- | --- |
| Radiated transmit power |  |  |  |  |
| OTA base station output power |  |  |  |  |
| OTA output power dynamics |  | RE power control dynamic range |  |  |
|  |  | Total power dynamic range |  |  |
| OTA transmit ON/OFF power |  | Transmitter OFF power |  |  |
|  |  | Transmitter transient period |  |  |
| OTA transmitted signal quality |  | Frequency error | Requirements can be reused from FR1/FR2. |  |
|  |  | Modulation quality | Requirements for QPSK, 16QAM and 64QAM can be reused from FR1/FR2. |  |
|  |  | Time alignment error | Requirements for the MIMO transmission and for the inter-band CA (with or without MIMO) can be reused from FR1/FR2. | Derive intra-band CA TAE requirements, based on the set of the supported SCS. |
| OTA occupied bandwidth |  |  |  |  |
| OTA ACLR |  |  |  |  |
| OTA operating band unwanted emissions |  |  |  |  |
| OTA transmitter spurious emissions | General transmitter spurious emissions |  |  |  |
|  | Protection of the BS receiver of own or different BS |  |  |  |
|  | Additional spurious emissions requirements |  |  |  |
|  | Co-location with other base stations |  |  |  |
| OTA transmitter intermodulation |  |  |  |  |

#### 7.4.1.x		Transmitted signal quality

##### 7.4.1.x.1		Frequency error

The (conducted and radiated) frequency error requirements for NR BS is the same for FR1 and FR2. Therefore the (conducted and radiated) frequency error requirement for the 7 – 24 GHz range is expected to be reused from FR1/FR2 as defined in TS 38.104 [x].

##### 7.4.1.x.2		EVM

The (conducted and radiated) EVM levels for NR BS operating with the QPSK, 16QAM, or 64QAM were defined to be the same for FR1 and FR2. Therefore the (conducted and radiated) EVM requirements for the 7 – 24 GHz range can be reused from FR1/FR2 as defined in TS 38.104 [x]. Some aspects differ between FR1 and FR2 for the EVM requirement e.g. EVM window length and the differences between them need to be considered for 7 - 24 GHz range.

Conducted and radiated requirements were also defined for 256QAM operation in FR1. For FR2, a Rel-16 WI on the 256QAM for FR2 was agreed to further evaluate its feasibility. Depending on the outcome of this WI, the following options are identified:

- In case 256QAM requirements would be defined for FR2: in this case the 256QAM would be defined for both FR1 and FR2, indicating that it is also feasible to define 256QAM EVM requirement for the 7 – 24 GHz range;

- In case 256QAM requirements would not be defined for FR2: in this case the 256QAM would be defined for FR1 bot not for FR2. Therefore their applicability for the middle range of 7 – 24 GHz would be an open issue, which is outside the scope of this study item.

##### 7.4.1.x.3		TAE

The TAE requirements for the following transmission modes were defined the same for FR1 (conduced and radiated) and FR2 (radiated) and therefore are considered to be reused for the 7 – 24 GHz range:

- MIMO transmission

- inter-band CA, with or without MIMO

For intra-band CA TAE requirements, the FR1 and FR2 requirements differ due to different set of supported SCS values. Once the set of SCS is decided for the 7 – 24 GHz range (may differ from FR1 and from FR2 set of SCS’), the intra-band CA TAE requirements are to be derived.

------------------------------ End of modified section ------------------------------