3GPP TSG-RAN WG4 Meeting #92bis	R4-1913050

Chongqing, China, 14th– 18th October 2019

Agenda item:	8.12.2	Implementation feasibility and performance benefit study 	[NR_DL256QAM_FR2]

Source: 		NTT DOCOMO, INC.

Title: 	Feasibility evaluation for NR FR2 DL256QAM

Document for:	Approval

- Introduction

In RAN4#92, we provided simulation results on static channel with antenna configuration 2x2 and Rank 2 [1]. During the meeting, feasibility of FR2 DL 256QAM was discussed based on evaluation results provided by various companies, however no consensus was reached.

In this contribution, we provide further link level and system level simulation results.

2.	Discussion

2.1	Link level simulation

Table 1 summarizes link level simulation assumption, based on the agreed simulation assumption [2].

Table 1: Simulation assumption for link level simulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) |
| CBW | 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | AWGN TDL-D 30ns delay spread, 35Hz Doppler frequency |
| MCS | fixed MCSs 64QAM: MCS 24, 25, 26, 28 256QAM: MCS 21, 23, 25, 27 |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; random PMI |
| Symbol type | CP-OFDM |
| HARQ | None |
| Antenna configuration | Fading channel: 2x2 for Rank2, Low correlation |
| Channel estimation | Practical for AWGN Ideal for TDL-D |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Ideal CPE compensation |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): PN model config1: example1 (BS) + example1(UE) |

Figure 1 and 2 show link level simulation results for static and TDL-D channel respectively.

Observation 1: From the link level simulation results on static and TDL-D channel with 2x2 and rank 2, FR2 DL 256QAM modulation has non negligible performance gain compared with 64QAM modulation.

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 1: Link level simulation results on static channel

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 2: Link level simulation results on TDL-D channel

2.2	System level simulation

Table 2 summarizes the simulation assumption for system level simulation in this paper, based on TR 38.802 and modified for this study.

Table 2: Simulation assumption for system level simulation

| Parameter | Value |
| --- | --- |
| Deployment | Single macro layer, Hex. Grid |
| ISD | 200m |
| Carrier frequency | 28GHz (n257) |
| CBW | 100MHz |
| SCS | 120kHz |
| Channel model | TR38.901 UMi-Street canyon |
| BS Tx power | 33 dBm |
| Num. of beams | 64 |
| BS antenna config | (M,N,P,Mg,Ng) = (8,16,2,1,1) (dH,dV,dHg,dVg) = (0.5,0.5,NA,NA)λ |
| BS antenna height | 10m |
| UE antenna height | 1.5m (all outdoor) |
| UE receiver NF | 13dB |
| UE distribution | 3 users per TRP |

Figure 3 shows system level simulation results.

Observation 2: From the system level simulation results, DL SINR distribution is high enough to obtain a performance gain from 256QAM in FR2 UMi scenario.

Figure 3: System level simulation results

2.3	Conclusion on feasibility study

From above observations, we can conclude that DL 256QAM in FR2 can provide certain system performance gain in realistic SNR. Therefore, we propose to introduce the requirement of FR2 DL 256QAM.

Regarding Tx EVM requirement for FR2 256QAM, the same required value as FR1 is sufficient for FR2 from simulation results. Therefore, we propose to define 3.5% as the core requirement for BS Tx EVM for 256QAM modulation.

Proposal 1: RAN4 conclude that FR2 DL 256QAM modulation has enough performance gain compared with 64QAM under the agreed evaluation assumption.

Proposal 2: Introduce RAN4 requirements for FR2 DL 256QAM modulation.

Proposal 3: Define 3.5% as a core requirement for BS Tx EVM with 256QAM modulation in FR2.

3.	Conclusion

In this contribution, we provide simulation results for the feasibility study of FR2 DL 256QAM. The following observation and proposals are obtained.

Observation 1: From the link level simulation results on static and TDL-D channel with 2x2 and rank 2, FR2 DL 256QAM modulation has non negligible performance gain compared with 64QAM modulation.

Observation 2: From the system level simulation results, DL SINR distribution is high enough to obtain a performance gain from 256QAM in FR2 UMi scenario.

Proposal 1: RAN4 conclude that FR2 DL 256QAM modulation has enough performance gain compared with 64QAM under the agreed evaluation assumption.

Proposal 2: Introduce RAN4 requirements for FR2 DL 256QAM modulation.

Proposal 3: Define 3.5% as a core requirement for BS Tx EVM with 256QAM modulation in FR2.

References

- R4-1908660, “Feasibility evaluation for NR FR2 DL256QAM”, RAN4 #92, August 2019

- R4-1907711, “Simulation assumption of feasibility study for FR2 DL 256QAM”, RAN4 #91, May 2019