3GPP TSG-RAN WG4 Meeting #92bis	R4-1913051

Chongqing, China, 14th– 18th October 2019

Agenda item:	8.12.2	Implementation feasibility and performance benefit study 	[NR_DL256QAM_FR2]

Source: 		NTT DOCOMO, INC.

Title: 	TP to TR 38.833 NR FR2 DL256QAM link level and system level simulation results

Document for:	Approval

- Introduction

In [1], we provided link level and system level simulation results for the feasibility study of FR2 DL 256QAM. This contribution is TP to TR 38.833 to capture the simulation results and based on [2].

References

- R4-1913050, “Feasibility evaluation for NR FR2 DL256QAM”, NTT DOCOMO, INC., RAN4 #92bis, October 2019

- R4-1911086, “Draft TR 38.883 for FR2 DL 256QAM v0.1.0”, China Telecom, RAN4 #92bis, October 2019

--------------Start of text proposal-------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-190760, “Add support of NR DL 256QAM for FR2”, China Telecom

[3]	3GPP TR 38.803: "Study on new radio access technology: RF and co-existence aspects".

[4]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[5]	R4-1909091, “Link level simulation results for the feasibility study of FR2 DL 256QAM”, China Telecom

[6]	R4-1909269, “Initial FR2 DL 256QAM link level simulation results”, Nokia

[7]	R4-1913050, “Feasibility evaluation for NR FR2 DL256QAM”, NTT DOCOMO

[8]	R4-1909186, “Simulation results on DL 256QAM”, Huawei

[9]	R4-1909401, “Initial Link Simulation Results on DL 256 QAM FR2”, Ericsson

[10]	R4-1908391, “Discussion on 256QAM for FR2”, CATT

[11] 	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[12] 	R4-1908144, “Views on feasibility of 256QAM for FR2”, Qualcomm

[13]	Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers,  IEEE Journal of Solid-State Circuits ( Volume: 53 , Issue: 7 , July 2018 )

[14]	R4-1909403, “Feasibility of UE demodulation testing”, Ericsson

[5.2.2.1-1] 	R4-1713125, “Evaluation on BS Tx EVM for mmWave”, Huawei, HiSilicon

[5.2.2.2-2] 	R4-1906014, “System level simulation for 256QAM DL in FR2”, Nokia, Nokia Shanghai Bell

[5.2.2.2-3]	T. Tuominen, N. Tervo, A.Pärssinen; Analyzing  5G  RF  System  Performance  and Relation to Link Budget for Directive MIMO,  IEEE Transactions on Antennas and Propagation ( Volume: 65 , Issue: 12 , Dec. 2017 )

[5.2.2.3-4]	R4-1908211, “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel

[A-1]	R4-1711777, WF on simulation assumption for NR BS EVM requirement, Huawei, HiSilicon

[A-2]	R4-1709666, Simulation assumptions on UE Tx EVM for mmWave, Huawei, HiSilicon

[A-3]	3GPP Radio Access Network Working Group. 2016. “E-UTRA and UTRA Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)”. 3GPP TR 37.842 V13.0.0.

[A-4]	Du Jinfeng, Chizhik Dmitry, Rodriguez Rodolfo Feick, Mauricio, Castro Guillermo, Valenzuela Reinaldo. A. 2018. “Suburban Fixed Wireless Access Channel Measurements and Models at 28 GHz for 90% Outdoor Coverage”, arXiv: 1807:03763.

[A-5]	3GPP Radio Access Network Working Group. 2018. “Study on channel model for frequencies from 0.5 to 100 GHz (Release 15)”. 3GPP TR 38.901 V15.0.0.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

--------------Omitted unchanged sections-------------

5.2.1.1	Simulation assumptions

The link level simulation assumptions are listed as in table 5.2.1.1-1, based on which, to evaluate the throughput difference between 64QAM and 256QAM. The study aims to identify conditions where DL 256QAM provides performance benefits.

Table 5.2.1.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257) and 39 GHz (n260) |
| CBW | 50 MHz, 100MHz |
| SCS | 60kHz, 120 kHz; |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: MCS 23, 24, 26, 28 in TS 38.214 Table 5.1.3.1-1, and other MCSs are not precluded 256QAM: MCS 21, 23, 25, 27 in TS 38.214 Table 5.1.3.1-2, and other MCSs are not precluded Baseline: fixed MCSs |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels and Section 7.5 for static channel; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (BS) + example1(UE) Option b): example2 (BS) + example2(UE) Option c): example2 (BS) + example2(BS) Option d):example2 (BS) + PN model config1: example1(UE) Option e): Other phase noise models, e.g. ones extracted from commercially available components or published results, are not excluded |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: [1.0%-5.0%], rxEVM: [1.0%-5.0%] Option 1: txEVM <= rxEVM; Option2: no restriction |
| Other parameters | follow assumptions in 38.101-4 Section 7.2 for fading channels (e.g., case 2-6) and Section 7.5 for static channels |

The assumptions adopted by each company are shown as following table 5.2.1.1-2 which are down-selected based on the table 5.2.1.1-1.

Table 5.2.1.1-2 link level simulation assumptions down-selected by companies

| Parameter |  |  |  | CTC[5] | Nokia[6] | Docomo[7] | Huawei[8] | Ericsson[9] | CATT[10] | Intel[11] | Qualcomm[12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz |  | 29 GHz |  |  |
| CBW |  |  |  | 50MHz | 100MHz | 100MHz | 50MHz |  | 50MHz | 50MHz | 100MHz |
| SCS |  |  |  | 120kHz | 60kHz | 120kHz | 120kHz |  | 60kHz | 60kHz | 120kHz |
| Allocated RBs |  |  |  |  |  |  |  |  |  |  |  |
| Propagation |  |  | TDL-A |  |  |  |  |  |  |  |  |
|  |  |  | TDL-D |  |  |  |  |  |  |  |  |
|  |  |  | Static |  |  |  |  |  |  |  |  |
| MCS |  |  | 64QAM | 28 | 26,28 | 24,25,26,28 | 23,24,26,28 | 23,24,26,28 | 23 | 23,24,26,28 | 26,27,28 |
|  |  |  | 256QAM | 27 | 21,23 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21 | 21,23,25,27 | 20,21,22 |
| Precoding |  |  |  |  |  |  |  |  |  |  |  |
| Symbol type |  |  |  |  |  |  |  |  |  |  |  |
| HARQ |  |  |  | None | None | None |  | None | 8 | 8 | 8 |
| Antenna configuration |  | Fading |  | 2x2 for Rank1 | 2x2 for Rank1 |  |  | 1x2 for Rank1 | 2x2 for Rank1 |  | 2x2 for Rank2 |
|  |  | Static |  |  |  | 2x2 for Rank2 |  | 1x2 for Rank1 | 1x2 for Rank1 |  | 2x2 for Rank2 |
| Channel estimation |  |  |  |  |  | Practical for AWGN Ideal for TDL-D |  |  |  |  |  |
| Receiver type |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH configuration |  |  |  |  |  |  |  |  |  |  |  |
| DMRS configuration |  |  |  |  |  |  |  | No additional |  |  |  |
| PTRS configuration |  |  |  | None |  |  |  |  |  |  |  |
| Phase noise compensation |  |  |  | None |  | Ideal |  |  |  |  |  |
| Phase noise model | Option a) |  |  |  |  |  |  |  |  |  |  |
|  | Option b) |  |  |  |  |  |  |  |  |  |  |
|  | Option c) |  |  |  |  |  |  |  |  |  |  |
|  | Option d) |  |  |  |  |  |  |  |  |  |  |
|  | Option e) |  |  |  |  |  |  |  |  |  | example1BS+example2UE internal PN model |
| txEVM + rxEVM excluding phase noise for 256QAM |  |  |  | Tx+Rx: 3%, 4% | txEVM: 3%, rxEVM: 3% | txEVM: 0%, 2%, 3%, rxEVM: 0%, 2%, 3% | txEVM: 1%-3%, rxEVM: 1%-3% |  |  | txEVM: 1%-3%, rxEVM: 1%-3% | Tx:3% Rx:internal |
| Other parameters |  |  |  |  |  |  |  |  |  |  |  |
| Note: The symbol of means selecting the parameters corresponding to table 5.2.1.1-1. |  |  |  |  |  |  |  |  |  |  |  |

--------------Omitted unchanged sections-------------

5.2.1.4	Results from NTT DOCOMO [7]

Figure 5.2.1.4-1 and 5.2.1.4-2 show link level simulation results compared between 64QAM and 256QAM. On static channel and TDL-D with antenna configuration 2x2 and Rank 2, with  3% Tx EVM and 3% Rx EVM, the performance gain compared to 64QAM modulation is obtained over 21dB and 25dB SNR, respectively.

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 5.2.1.4-1: Simulation results on static channel

a) Tx EVM 0% and Rx EVM 0%						 b) Tx EVM 2% and Rx EVM 2%

c) Tx EVM 3% and Rx EVM 3%

Figure 5.2.1.4-2: Simulation results on TDL-D channel

--------------Omitted unchanged sections-------------

5.2.2.x	Results from NTT DOCOMO

Figure 5.2.2.x-1 shows the system level simulation result [7]. From the result, around 25% of UEs can achieve more than 26dB SINR.

Figure 5.2.2.x-1: System level simulation results

--------------Omitted unchanged sections-------------

A.4 Assumptions from NTT DOCOMO

Table A.4-1 shows the simulation assumption for system level simulation, which is based on TR 38.802 and modified for the feasibility study.

Table A.4-1: Simulation assumption for system level simulation

| Parameter | Value |
| --- | --- |
| Deployment | Single macro layer, Hex. Grid |
| ISD | 200m |
| Carrier frequency | 28GHz (n257) |
| CBW | 100MHz |
| SCS | 120kHz |
| Channel model | TR38.901 UMi-Street canyon |
| BS Tx power | 33 dBm |
| Num. of beams | 64 |
| BS antenna config | (M,N,P,Mg,Ng) = (8,16,2,1,1) (dH,dV,dHg,dVg) = (0.5,0.5,NA,NA)λ |
| BS antenna height | 10m |
| UE antenna height | 1.5m (all outdoor) |
| UE receiver NF | 13dB |
| UE distribution | 3 users per TRP |

--------------End of text proposal-------------