3GPP TSG-RAN WG4 Meeting #94-e	R4-2000030

Online, 24 - 28 February, 2020

Source:	ZTE

Title:	Discussion on handover requirements

Agenda item:	6.10.6

Document for:	Discussion

# 1	Introduction

- The core requirements for handover are specified in clause 6.1 in TS 38.133 [1]. There are some unclear issues remaining in the specification, and it’s our intention to discuss these issues and provide our view.

2	Handover requirements

The following content is copied directly from TS 38.133:

6.1.1 NR Handover 6.1.1.1 Introduction The purpose of NR handover is to change the NR PCell to another NR cell. The requirements in this clause are applicable to SA NR, NE-DC and NR-DC. 6.1.1.2 NR FR1 - NR FR1 Handover The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR1 cell to NR FR1 cell. 6.1.1.2.1 Handover delay Procedure delays for all procedures that can command a handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command. Where: Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.2.2.

Observation 1: For NR to NR handover, Dhandover is defined as the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2] plus the interruption time.

In TS 38.331, the table in clause 12 is copied below:

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC Release | RRCRelease |  | NA |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | 10 |  |
| RRC resume | RRCResume | RRCResumeComplete | 6 or 10 | Value=6 applies for a UE supporting reduced CP latency for the case of RRCResume message only including MAC and PHY configuration, and no DRX, SPS, configured grant, CA or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRCResumeComplete and the data is transmitted over common search space with DCI format 0_0. In this scenario, the RRC procedure delay [ms] can extend beyond the reception of the UL grant, up to 7 ms. For other cases, Value = 10 applies. |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Other procedures |  |  |  |  |
| UE assistance information |  | UEAssistanceInformation | NA |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 80 |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

According to the table, the maximum RRC procedure delay is 80ms, for UE capability transfer, which is irrelevant to handover process. The intention of the statement should be “the maximum RRC procedure delay for procedures which can trigger a NR to NR handover”, and as defined in TS 38.331, up to Rel-16, the only RRC command which can trigger an  NR to NR handover is RRC reconfiguration. So in this case, in 38.133, the requirement can be clarified to the following.

In TS 38.133, change the requirement for NR to NR handover to:

“When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the RRC procedure delay of RRC reconfiguration defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.X.2.”

Agree on CR [] which captures the above proposals.

3	Conclusion

- For NR to NR handover, Dhandover is defined as the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2] plus the interruption time.

- Proposal 1: In TS 38.133, change the requirement for NR to NR handover to:

“When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover from the end of the last TTI containing the RRC command.

Where:

- Dhandover equals the RRC procedure delay of RRC reconfiguration defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.X.2.”

- Proposal 2: Agree on CR [] which captures the above proposals.

References

- TS 38.133

- TS 38.331