3GPP TSG-RAN WG4 Meeting #94-e	R4-2000058

Online, 24 - 28 February, 2020

Source:	ZTE

Title:	Discussion on PSCell addition delay in NR-U

Agenda item:	8.1.4.5

Document for:	Discussion

# 1	Introduction

In RAN4 93, one of the open issues remain on PSCell addition / release delay for NR-U is whether to extend the period for the known condition of target cells. The known condition for a target cell in Rel-15 is copied below:

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions: During the last 5 seconds before the reception of the PSCell configuration command: - the UE has sent a valid measurement report for the PSCell being configured and - One of the SSBs measured from the PSCell being configured remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being configured also remains detectable during the PSCell configuration delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

This paper intends to provide our view on this topic.

2	Condition for known cell

- In RAN4 93 [1], it’s agreed that the following two options need to be further investigated.

In our view, when adapting the current R15 requirements to the changes introduced by NR-U, the baseline should be that when the channel occupancy is low enough, e.g., no other RAT or UE is currently using the same band to communicate, the R16 requirements under NR-U should be the same as R15 requirements. In some other requirements, the time period is extended by counting unavailable SSBs at the UE, which means when all SSBs are available, the requirement for NR-U is exactly the same with R15 requirements. This makes sense to us because the added occasions is intended to count for the number of SSBs (or other DRS) which are not available due to LBT, which is a new feature compared to R15. Thus, by adding those signal occasions, the essential requirement should remain the same with R15 requirements.

Based on this reason, generally, we don’t think extending the time period by a fixed number would be of great help to address the impact of LBT failures.

Do not extend the time period in the known condition for the target PSCell under NR-U.

3	Conclusion

- Proposal 1: Do not extend the time period in the known condition for the target PSCell under NR-U.

References

- R4-1915777, WF on NR-U RRM Requirements