3GPP RAN WG4 Meeting #94-e		R4-2000086

Electronic meeting, 24th February – 6th March 2020

Agenda item:	9.23.2

Source:	Apple Inc

Title:	Introduction of LTE/NR spectrum sharing in band 48/n48 frequency range

WI/SI:	NR_n48_LTE_48_coex-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

Dynamic spectrum sharing is an important feature that allows for sharing existing spectrum between the LTE and NR carriers, thus enabling smoother transition from LTE and faster adoption of NR. Amongst a number of prerequisites for running LTE and NR within the same frequency band, either FDD or TDD, an operator has to ensure that both LTE and NR raster are aligned and that the NR synchronization blocks do not overlap with LTE reference signals. As an example, the following WI was agreed to enable dynamic spectrum sharing in band 41/n41 frequency range [1].

After the RAN#86 meeting, a new WI was agreed [2] aiming to analyse and introduce, if needed, changes to support dynamic spectrum sharing in band 48/n48 frequency range. In this discussion paper we elaborate further on the required changes needed to support the aforementioned functionality.

# 2	LTE/NR spectrum sharing in band 48/n48

From the RAN WG4 objective, the following aspects should be addressed:

-	Channel raster;

-	UL shift;

-	Sync pattern.

In the following subsections we present our further considerations in each of the aforementioned points.

## 2.1	Channel raster

To align LTE and NR carrier, both carriers have to be configured with the same centre frequencies, which in turn should be an integer multiple of the channel raster step of the corresponding LTE and NR bands. While the LTE band 48 has a channel raster of 100kHz, the corresponding NR band n48 follows so-called SCS based channel raster, which is either 15 or 30kHz (refer also to Annex B). Based on that, one of the solutions not requiring any standardization changes would be to consider FREF values at step of 300kHz for those carriers where DSS operation between LTE and NR is anticipated. For other carriers, using exclusively either LTE or NR, there will be no restrictions on how they can be configured.

Observation 1a:	300kHz raster does not require any further standardization changes and thus can be used by the SAS allocation entity to align LTE and NR centre frequencies for those carriers where DSS operation is needed.

Even though the solution with FREF values at step of 300kHz does not require any further standardization changes in 3GPP, it is worth noting that it will put restrictions on the SAS entity that is responsible for spectrum allocations as the latter will have to ensure that the allocated carriers are aligned as an integer multiple of 300kHz. Furthermore, the following problems might emerge:

-	Spectrum utilization. The preferred chunk of allocated spectrum on the CBRS band 48/n48 is 10MHz, but it can be also as small as 5MHz, which includes both data RBs and the carrier guard-bands. With 100kHz channel raster, the SAS entity can allocate flexibly required spectrum without any leftovers thus ensuring the best spectrum utilization of allocated resources. However, with 300kHz channel raster, not all the centre frequencies will be an integer multiple of 300kHz meaning that there can be some "leftovers" resulting in somewhat less efficient spectrum usage. It might be somewhat marginal problem for large carrier sizes, but the problem will amplify if small 5MHz carriers are allocated because each of them will have to be aligned with 300kHz raster.  As an example, if 10MHz channel have to be allocated for several operators, then the corresponding centre frequencies on the 100kHz raster can be 3555MHz, 3565MHz, 3575MHz, 3585MHz, etc. At the same time, to ensure that the channel raster is an integer multiple of 300kHz, centre frequencies should be 3555MHz, 3565.2MHz, 3575.4MHz, 3585.6 etc.

-	Spectrum re-allocations.  Another potential problem is when a particular carrier initially uses only LTE with 100kHz raster, but then an operator decides to enable DSS thus requiring 300kHz raster alignment. In this case, the corresponding LTE carrier centre frequency will shift potentially impacting other operators’ centre frequencies in the whole band 48/n48 spectrum range. As an example, 40MHz chunk of spectrum can be easily and fairly shared between four operators with 100kHz channel raster when only LTE is used, i.e. each operator will get 10MHz. However, once an operator decides to use NR and DSS requiring the channel raster to be an integer multiple of 300kHz, it will be impossible to accommodate four 10MHz channels without violating minimum guard band sizes.

Observation 1b:	As opposed to 100kHz, 300kHz raster might result in less efficient spectrum utilization and/or spectrum re-allocation problems when an operator decides to enable the DSS functionality.

Referring to Observation 1b, the same problem was acknowledged during the DSS related WI for band 41/n41 frequency range, and thus 100kHz raster was added to a new band n90 (refer also to Annex B).

Proposal 1a:	300kHz channel raster can be viewed as a baseline option for spectrum sharing between LTE and NR.

Proposal 1b:	100kHz channel raster can ensure more efficient (dynamic) spectrum utilization in band n48.

## 2.2	UL shift

As already mentioned in the Introduction part, one of the prerequisites for dynamic spectrum sharing between the LTE and NR carriers is the system ability to align sub-carrier grids of the LTE and NR systems. To align UL sub-carrier grids, an additional higher layer parameter was added to the system information to indicate UL channel raster shift. As can be seen from Figure 2.2-1, having a common and aligned sub-carrier grid, the base station can decide flexible how many RBs should be allocated for LTE or NR UEs. And if both LTE and NR use 15kHz SCS, then no inter-numerology guard-band is needed.

Figure 2.2-1: Exemplary sub-carrier grid with 15kHz LTE and 15kHz NR RBs.

However, as follows from TR 38.822 and TS 38.101-1 (see also Annex A), this parameter is mandatory only for the FDD and SUL bands. In other words, a UE camped on the TDD bands is not anticipated to act on the corresponding parameter if it is signalled by the network. The problem was firstly identified for bands 41/n41 for LTE/NR spectrum sharing WI [1], and RAN WG4 agreed to introduce a new band [2] to circumvent around the fact that channel raster shift is not a mandatory parameter for the NR TDD bands and to introduce new channel raster values. Referring to Table 5.2-1 from TS 38.101-1 (highlighted in yellow for the sake of clarity), band n90 was introduced which is effectively the same band as n41, but the network deploying LTE and NR on the same carrier will use band n90 to prevent legacy terminals from camping on that carrier. In turn, a UE supporting band n90 has to support UL channel raster shift parameter.

Referring to the agreements from the CBRS Alliance, 30kHz SCS is the mandatory supported configuration. In other words, dynamic spectrum sharing for the LTE and NR carriers should deal with the mixed numerology case. According to RAN WG4 specifications, there are no particular requirements for the mixed-numerology deployments, and as Figure 2.2-2 presents, a particular network implementation may follow different strategies. If the network uses the common sub-carrier grid spanning over LTE and NR allocations, then it would be quite easy to ensure that inter-numerology guard band is either 15kHz or 30kHz SCS RB, or even zero RBs. The latter case is also feasible because even though 15kHz and 30kHz numerologies are not orthogonal, it will just cause additional emission to the neighbouring RB. And since the scheduling is done by the base station, the latter will be aware of it. As an example, if LTE and NR transmissions do not take 100% of UL resources, then it is quite natural to make an inter-numerology guard-band as it comes for free. For the 100% load scenario, it can be up to the network implementation and scheduling whether to schedule all RBs catering for better spectrum utilization (e.g. best effort service) or avoid unnecessary emissions to the neighbouring RB. Thus, enabling UL shift on band n48 will allow sub-carrier grid alignment which in turn can be used by the network for better resource utilization. And even though it can be viewed as a marginal optimization for a large carrier size, it will be a more noticeable improvement for 5 or 10MHz carriers which can be allocated by the SAS entity, especially accounting for the fact that 30kHz SCS RB will provide worse spectrum utilization than 15kHz SCS.

Figure 2.2-2: Exemplary sub-carrier grid for 15kHz LTE and 30kHz NR RBs.

As a summary, our view is that it is possible to follow same principle as for FDD and SUL bands – mandate supporting the corresponding channel raster shift parameter also for band n48 (as a similar decision was made for band n41). Since a UE anyway has to support this feature for FDD and SUL bands, it will not create substantially new functional requirements.  It is also worth noting that an additional clarification was introduced after the RAN2#106 meeting specifying that UL shift parameter is applicable to both FDD and TDD bands [3] (see also Annex D). Thus, only the corresponding sub-clause 5.4.2.1 should be changed to make this parameter applicable to band n48.

Proposal 2:	Consider adding 7.5kHz UL shift capability for band n48.

## 2.3	Sync pattern

As discussed during the RAN4#92bis and RAN4#93 meetings, there can be overlaps between the LTE CRS and NR SSB transmissions [3][4]. As can be seen from the Figure 2.3-1 below, existing band n48 synchronization pattern C still can work for 1 and 2 port LTE transmission because the corresponding NR SSB can be transmitted in LTE OFDM symbols #1-2 and #8-9. However, if LTE uses 4 antenna port transmission, then all the sync pattern C NR SSB transmission opportunities overlap with the LTE CRS symbols. One solution to this problem is to introduce sync pattern B for band n48. Referring to the same Figure 2.3-1, the sync pattern B NR SSB can be transmitted in LTE OFDM symbols #2-3 even if the 4-port LTE transmission is used.

Figure 2.3-1: LTE CRS and NR SSB transmissions for 1ms sub-frame.

One of the potential solutions to avoid LTE CRS and NR SSB overlaps is to leverage LTE MBSFN sub-frames so that LTE CRS can be muted in those sub-frames where NR SSB is expected. However, the problem is that LTE MBSFN configuration allows to mute only specific sub-frames, which are not the ones where NR SSB can be sent. Figure 2.3-2 below presents TDD configurations #1 and #2 considered by the CBRS alliance for band 48/n48. The LTE MBSFN can be enabled for sub-frames #3-4 and #7-8; but the NR SSB can be transmitted only in the first symbols of the half-sub-frame.

Figure 2.3-2: LTE MBSFN and NR SSB transmissions for 10ms frame.

Observation 3a:	NR sync pattern C can work with 1-2 port LTE deployments, but 4-port LTE CRS transmission will always collide with NR SSB.

Observation 3b:	NR sync pattern B can work with 4-port LTE deployments.

Observation 3c:	Since candidate LTE MBSFN sub-frames do not overlap with OFDM symbols where NR SSB is transmitted, LTE MBSFN cannot be considered as a viable solution to avoid overlaps (unless some further changes are introduced impacting other WGs).

Based on the presented considerations, the most flexible solution would be to consider sync pattern B for band n48 so that 4-port LTE deployments can also be used. As discussed earlier in RAN WG4, the only concern is that band n48 overlaps with e.g. band n77, which uses sync pattern C. On the one hand, band n48 is only for US market and band n77 is deployed in EU region, so there should be no conflict. On the other hand, since a UE does not know band ID before it gets a sync and reads system information, it will have no choice but to test for both sync pattern B and C while scanning 3550–3700 MHz frequency range. Nevertheless, our understanding is that it will not cause any serious problem because there are other cases with overlapping bands where different patterns can be used. Having different sync patterns in the same frequency range might increase initial search time and cell selection process during the cold start, but once a UE camps on the operator serving cell, it will follow cell re-selection process and frequencies indicated by a particular operator. In the worst-case scenario, even if a UE fails to camp on e.g. band n48 or n77 because it does not find the synchronization signal, it will search for other bands and RATs.

Proposal 3:	Adopt sync pattern B for band n48 definition.

## 2.4	Other considerations

As for the backward compatibility analysis, it is worth noting that NR TDD band n48 was added after the RAN#84 meeting meaning that it is not anticipated to have legacy devices, which support band n48 but do not support channel raster shift. Thus, introducing further changes to band n48 does not break any backward compatibility.

# 3	Conclusions

In this discussion paper we have presented our further views on the three major aspects needed for the dynamic spectrum sharing between LTE and NR on band n48. As a summary of our paper, our proposals are:

Proposal 1a:	300kHz channel raster can be viewed as a baseline option for spectrum sharing between LTE and NR.

Proposal 1b:	100kHz channel raster can ensure more efficient (dynamic) spectrum utilization in band n48.

Proposal 2:	Consider adding 7.5kHz UL shift capability for band n48.

Proposal 3:	Adopt sync pattern B for band n48 definition.

# 4	References

RP-182883, "New WI proposal: LTE/NR spectrum sharing in Band 41/n41", KDDI Corporation

RP-192427, "New WID: LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

R4-1913331, "Further discussion on DSS in LTE re-farming bands", Samsung

R4-1915312, "Views on dynamic spectrum sharing between LTE band 48 and NR band n48", Google Inc.

Annex A (excerpt from TS 38.101-1)

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n303 | 2305 Mhz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band [n90] is in accordance with that for Band n41; a UE supporting Band [n90] shall meet the requirements for Band n41. |  |  |  |

Annex B (excerpt from TS 38.101-1):

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| [n90] | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |

Annex C (excerpt from TS 38.101-1):

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

For SUL bands, for the uplink of all FDD bands defined in Table 5.2-1 and for Band [n90]

FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz.

where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7].

The mapping between the channel raster and corresponding resource element is given in Section 5.4.2.2. The applicable entries for each operating band are defined in Section 5.4.2.3

Annex D (excerpt from TS 38.331):

#### FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2).

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15]. Value in dBm.

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise. |