3GPP TSG-RAN WG4 Meeting #94-e                         	     R4-2000124

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.14.1.1

Source:	vivo

Title:	Discussion on FR2 MPE mitigation

Document for:	Approval

- Introduction

- RAN2 based signaling solutions are sufficiently fast for the FR2 MPE purposes

- RAN4 shall request RAN2 to develop signaling for FR2 MPE purposes with the following assumptions;

  - RAN4 understands MAC-CE is suitable method

  - MPE event related assistance Information provided by the UE to the network

    - P-MPR is indicated to the network and is agreed in RAN4#93

    - Dynamic duty cycle will be further discussed in RAN4#94

    - Single entry PHR will be further discussed in RAN4 #94

  - Report should be configurable as periodic, or event triggered. Configurable periods and trigger conditions are FFS

- RAN4 will send LS to RAN2 in RAN4 #93 to inform RAN2 that MAC-CE signaling may be required for MPE solutions. RAN4 will inform RAN2 on the complete solution in RAN4 #94

In RAN4 #93 meeting, way forward on FR2 MPE Enhancement was agreed [1] and cited below for convenience. Use of MAC-CE and indication of P-MPR were agreed as suitable method, in addition to this, the necessity of Dynamic duty cycle, single entry PHR and trigger conditions were left for discussion and decision in RAN4#94. In this document we discuss our view on these remaining topics.

- Discussion

  - Dynamic duty cycle

The concept of “Static” maximum uplink duty cycle (maxUplinkDutyCycle-FR2) has been introduced in 38101-2, which indicate UE’s preferred maximum uplink duty cycle to meet the exposure compliance requirements. The motivation of more dynamic methods (P-MPR, dynamic duty cycle etc.) comes from related working scope of the new WI [2] which is “includes methods and solutions to avoid radio link failures (RLF) and connection releases due to unpredictable UE P-MPRs that can be caused by the FR2 UE RF exposure compliance reasons”, i.e. the intension is to introduce necessary information/reporting to deal with possible rapid change of UE environment (proximity to human body etc.) and prevent RLF.

Observation1: intension of WI is to deal with possible rapid change of UE environment (proximity to human body etc.) and prevent RLF.

As it is known that transmission power and duty cycle are two factors that impacted MPE jointly. And the assumption is these two factors can be leveraged equally for certain MPE threshold, e.g. 23dBm@20% duty generating similar MPE as 20dBm@40% duty. In another word, the instantaneous preferred “dynamic duty cycle” can be derived from P-MPR by referring to “static” UE capability reported in maxUplinkDutyCycle-FR2.which assume maximum output power (i.e. zero P-MPR).

During the previous discussion, P-MPR and dynamic duty cycle have been divided into 2 categories, i.e. “Rapid indication methods” or “Assistance information methods”, and implicitly been understood as “P-MPR is for short term” and “dynamic duty cycle is somehow for longer term”. However it does not have to be like this in real operation, e.g. P-MPR can also be used as “long term” purpose. UE can report lower P-MPR for higher exposure headroom within a MPE compliance period (e.g. 6 minutes in ICNIRP, 4 seconds in FCC for 24 to 42 GHz), and higher P-MPR for lower exposure headroom vice versa. Similar technique has already been used in UE product, like “time averaging SAR”. After all it is UE’s obligation for exposure compliance.

Observations 2: with UE static max duty cycle capability, “dynamic duty cycle” can be derived from P-MPR and does not provide additional information. P-MPR can also be used as “long term” method by UE implementation.

Proposal1: Dynamic duty cycle is not reported by UE based on the agreement that P-MPR is indicated to the network.

  - Single entry PHR

In R15, P-bit is only specified for multiple entry PHR which is used for the power management back off setting for each carrier in CA scenario. It was considered as not needed for non-CA case, i.e. single entry PHR.

During R16 current discussion, as we agreed to use MAC-CE for MPE mitigation purpose and P-MPR will be indicated together with P-bit. It is definitely applicable for single cell group reporting, i.e. single entry PHR.

Proposal2: introduce P-bit and P-MPR indicator in single-entry PHR. introduce P-MPR indicator in multi-entry PHR.

Regarding extension of the existing PHR MAC CE to indicate magnitude of the P-MPR. There are basically two options: 1) add a new byte, 2) use two reserved "R" bits.

Because this P-MPR information is used for RLF as stated in the WID, and RLF is large granularity situation (small granularity can be handled by gNB’ link adaptation [7]), two bits to indicate four possible values range should be enough to serve this purpose. One example could be as below. And the indicated range of 9dB is close to the capability range of maxUplinkDutyCycle-FR2 which is 15%~100% as in 38.331.

Proposal3: use two reserved "R" bits to indicate magnitude of the P-MPR, corresponding lookup table could be as below.

| 2 bits indicator | PMPR magnitude (dB) |
| --- | --- |
| 00 | 0~3 |
| 01 | 3~6 |
| 10 | 6~9 |
| 11 | >9 |

  - Trigger conditions

The trigger condition for PHR has been define in 38.321 including both periodic and event triggered reporting. By reviewing the value ranges of each parameter defined in 38.331 as below, seems they are also applicable for P-MPR in MPE mitigation study, e.g. phr-PeriodicTimer, phr-ProhibitTimer and phr-Tx-PowerFactorChange.

Because P-bit/P-MPR will be indicated in the same MAC-CE as PHR, it is straightforward to reuse the trigger condition for PHR.

Proposal4: reuse PHR trigger condition for P-bit/P-MPR reporting.

PHR-Config information element

-- ASN1START

-- TAG-PHR-CONFIG-START

PHR-Config ::=                      SEQUENCE {

phr-PeriodicTimer                   ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer                   ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange            ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR                         BOOLEAN,

dummy                               BOOLEAN,

phr-Type2OtherCell                  BOOLEAN,

phr-ModeOtherCG                     ENUMERATED {real, virtual},

...

}

-- TAG-PHR-CONFIG-STOP

-- ASN1STOP

3	Conclusions

In this contribution, we discuss the remaining topics of the necessity of Dynamic duty cycle, single entry PHR and trigger conditions.

Observation1: intension of WI is to deal with possible rapid change of UE environment (proximity to human body etc.) and prevent RLF.

Observations 2: with UE static max duty cycle capability, “dynamic duty cycle” can be derived from P-MPR and does not provide additional information. P-MPR can also be used as “long term” method by UE implementation.

Proposal1: Dynamic duty cycle is not reported by UE based on the agreement that P-MPR is indicated to the network.

Proposal2: introduce P-bit and P-MPR indicator in single-entry PHR. introduce P-MPR indicator in multi-entry PHR.

Proposal3: use two reserved "R" bits to indicate magnitude of the P-MPR, corresponding lookup table could be as below.

| 2 bits indicator | PMPR magnitude (dB) |
| --- | --- |
| 00 | 0~3 |
| 01 | 3~6 |
| 10 | 6~9 |
| 11 | >9 |

Proposal4: reuse PHR trigger condition for P-bit/P-MPR reporting.

- References

[1] R4-1916170 WF on FR2 MPE Enhancement, qualcomm, 3GPP RAN4#93, Nov 2019

[2] RP-191290, "NR RF requirements for FR2", Nokia, Nokia Shanghai Bell, 3GPP RAN#84, June 2019

[3] R4-1913057, “WF on MPE enhancement”, Qualcomm, 3GPP RAN4#92bis, Oct 2019

[4] R4-1913530, further considerations on the uplink duty cycle enhancements for the MPE scenario, apple, 3GPP RAN4#92bis, Oct 2019

[5] R4-1913947, Discussion on enhancement for RLF mitigation due to MPE on FR2, LG, 3GPP RAN4#92bis, Oct 2019

[6] R2-1817936 Power backoff issue in single entry PHR MAC CE Qualcomm Incorporated discussion Rel-15 NR_newRAT-Core

[7] R4-1914141 On the metrics for enhanced MPE indication, Ericsson, 3GPP RAN4#93, Nov 2019

Annex

TS 38.321

5.4.6	Power Headroom Reporting

The Power Headroom reporting procedure is used to provide the serving gNB with the following information:

-	Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell;

-	Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases);

-	Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell.

RRC controls Power Headroom reporting by configuring the following parameters:

-	phr-PeriodicTimer;

-	phr-ProhibitTimer;

-	phr-Tx-PowerFactorChange;

-	phr-Type2OtherCell;

-	phr-ModeOtherCG;

-	multiplePHR.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between.

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer;

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity:

4>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6];

4>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

4>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

5>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in clause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell;

3>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in clause 6.1.3.8 based on the values reported by the physical layer.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).