3GPP TSG-RAN WG4 Meeting #94-e	R4-2000194

# Electonic Meeting, 24th Febuary – 6th March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0192 | rev | - | Current version: | 15.8.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS38.101-1 CR: adding 30KHz SSB SCS for n40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For n40 only 15KHz SSB SCS is supported in the NR specification. But there would be three use cases where 30KHz SSB SCS is preferred. The first one is for the wider bandwidth deployment, for which 80MHz and 60MHz channel bandwidths are specified with 30KHz SCS in n40. The second one is LTE-NR co-existence use case, where 30KHz SSB SCS should be used to avoid the collision with LTE CRS like for the other bands including n66/n5/n90. The third one is CA case, where the NR CA with n40 is deployed. For it, 30KHz SCS would be preferred due to limitation of optional feature of simultaneous reception on separate CCs with different numerologies in CA. For those use cases, the configuration of 15KHz SCS SSB plus 30KHz SCS data would be problematic in the real deployment: - Firstly, the guard band is needed between SSB with 15KHz SCS and data with 30KHz SCS, which would cause the loss of spectrum efficiency, because SSB RE is not orthogonal with data RE and the duration of SSB and data symbols are unequal. - Secondly, UE has to stop reception of serving cell(s) on Band n40 when conducting measurement, which would lead to the long interruption and downlink throughput loss, because the simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement is an optional feature. - Thirdly, the implementation complexity would be increased for BS, UE and test equipment, which may delay the deployment on n40, because the additional hardware is needed to process SSB and data, e.g., single FFT does not work and dual FFT is needed. - Fourthly, the 30KHz SCS SSB cannot avoid the collision with LTE CRS in LTE-NR downlink co-existence scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 30KHz SSB SCS for n40 in the synchronization raster configuration table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The features including wider channel bandwidth, intra-frequency measurement, CA with 30KHz data SCS configured on all the CCs and LTE-NR co-existence cannot be well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6.3B.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change has no impact on ASN.1. Since n40 UE device is not in the market or to be in the market soon until Q1 2020, there could be no backward compatible issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n34 | 15 kHz | Case A | 5030 – <1> – 5056 |
| n38 | 15 kHz | Case A | 6431 – <1> – 6544 |
| n39 | 15 kHz | Case A | 4706 – <1> – 4795 |
| n40 | 15 kHz | Case A | 5756 – <1> – 5995 |
|  | 30 kHz | Case C | [5762]– <1> –[5989] |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n50 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8] |  |  |  |

<< End of change >>