TSG-RAN Working Group 4 (Radio) #94-e	 R4-2000197

Online, Feb. 24 – March 6, 2020

Source:	Interdigital Inc.

Title:	Remaining issues for MPE issues mitigation

Agenda item: 	8.14.1.1

Document for:	Approval

# Introduction

This contribution discusses the remaining issues for a pre-emptive approach solution regarding the MPE mitigation issues in FR2. We are outlining here for convenience the WF agreements and remaining issues:

- RAN2 based signaling solutions are sufficiently fast for the FR2 MPE purposes

- RAN4 shall request RAN2 to develop signaling for FR2 MPE purposes with the following assumptions;

  - RAN4 understands MAC-CE is suitable method

  - MPE event related assistance Information provided by the UE to the network

    - P-MPR is indicated to the network and is agreed in RAN4#93

    - Dynamic duty cycle will be further discussed in RAN4#94

    - Single entry PHR will be further discussed in RAN4 #94

  - Report should be configurable as periodic, or event triggered. Configurable periods and trigger conditions are FFS

- RAN4 will send LS to RAN2 in RAN4 #93 to inform RAN2 that MAC-CE signaling may be required for MPE solutions. RAN4 will inform RAN2 on the complete solution in RAN4 #94

To summarize:

RAN4 agreed:

- to have a MAC-CE signaling solution, as the MAC-CE latency deemed to be sufficient for this scope.

- report the P-MPR value in dB to the network

- have this report configurable, or event triggered.

RAN4 to further discuss:

- Dynamic duty cycle reporting as an assisted information

- Single entry MAC-CE (single entry meaning the kind of PHR related terminology.

# Discussion

Since we are supporting a preemptive approach, we propose to signal along the P-MPR value a duty cycle related information. In the following paragraph we will discuss the advantages of such approach.

## 2.1 MPE assistance information

As we know, the MPE is an averaged power density measurement over a certain period. As we can see in the Fig. 1 below, this MPE limit can be met at different power levels as in measurement Cycle 1 and then measurement Cycle 2 based on 2 different power levels.

Power

Cycle 1                                                                                 Cycle 2

0%                  25%                50%                75%                0%                 25%                 50%                75%       Time

Fig. 1 Two equivalent averaged measurement cycles

## In the above example, for Cycle 2, if the UE power would’ve been kept as per Cycle 1, the UE would’ve had to take a maximum P-MPR=3dB reduction on transmissions above that level in order to comply with the MPE limit.

These measurements must consider all the UL transmissions in a cell group that is served by the same RF chain in a band. That means we must account not only for an UL serving cell, but for all transmissions over all active serving cells in that averaging/measurement period in the cell group. Also, a Cell Group is relying to the same scheduling entity in the network.

Thus, we believe that a such report, shall be related to a cell group.

Proposal 1: The MAC-CE report shall report an aggregated measurement per Cell Group, and this can be a single-entry report.

In [2] was proposed to use the PHR as terminology for this report. Since PHR is typically defined per cell, and the above proposal suggest having this report per cell group, we propose to use a different name in order to avoid any confusion.

Proposal 2: The new MAC-CE report to be named Energy Headroom Report (EHR).

## Also, in [2] has been proposed to report P-MPR, PHR and an SDC (Sustainable Duty Cycle).  We believe that when the P-MPR takes a positive value, it is already an MPE limit situation corrected by the UE. In this case, the energy headroom report would be negative.

## In our view, the energy headroom report it is relative to the maximum duty cycle supported by the UE at the maximum power.

Thus, we propose to report the P-MPR and the Energy Headroom Report both when triggered by the proximity sensors.

Examples of reports:

Trigger level:  10% HER

First Report:

P-MPR = 0, EHR= 5%

Second Report:

P-MPR = 0 EHR= 0%

Third report:

P-MPR = 2dB EHR= -15%

After getting negative EHR, the network scheduler may react and reduce the duty cycle or configuration and the next report may be:

Fourth report:

P-MPR = 0, EHR= 5%

This reporting would continue until the proximity sensors will reset.

We believe that due to multiple possible corrective actions from the network side that would allow reducing the output power density, like grant size (in RBs), CC reconfigurations, and the cell group aggregated power averaging, a generic Energy Headroom is a more valuable information for the network.

Proposal 3: The Energy Headroom Report is defined as the unused/overused energy as a percentage relative to the total energy based on the maximum duty cycle at the maximum UE power that complies with MPE limit.

## 2.2 The Energy Headroom (EH) reporting and P-MPR ranges

The energy headroom values range is a useful information around the critical values that are around the UE MPE limit. For this reason, we believe that 64 values (6 bits) granularity going from below to - 30% to 33% and beyond would be enough. The negative values mean overshooting the UE MPE limit, while positive values mean available energy headroom.

The mapping can be as follows:

| Reported value | Measured quantity value (%) |
| --- | --- |
| ENERGY_HEADROOM_0 | EH  - 30 |
| ENERGY_HEADROOM_1 | -30  EH  -29 |
| ENERGY_HEADROOM_2 | -29  EH  -28 |
| ENERGY_HEADROOM_3 | -28  EH  -27 |
|  |  |
| ENERGY_HEADROOM_53 | 23  EH  24 |
| ENERGY_HEADROOM_54 | 24  EH  25 |
| ENERGY_HEADROOM_55 | 25  EH  26 |
| ENERGY_HEADROOM_56 | 26  EH  27 |
| ENERGY_HEADROOM_57 | 27  EH  28 |
| ENERGY_HEADROOM_58 | 28  EH  29 |
| ENERGY_HEADROOM_59 | 29  EH  30 |
| ENERGY_HEADROOM_60 | 30  EH  31 |
| ENERGY_HEADROOM_61 | 31  EH  32 |
| ENERGY_HEADROOM_62 | 32  EH  33 |
| ENERGY_HEADROOM_63 | EH ≥ 33 |

Proposal 4: Use 64 range values for energy headroom reporting values.

For the P-MPR reporting, we believe that 32 values are a wide enough range. This reported will express the estimated power back-off that the UE is supposed taking based on the current energy headroom measurement.

The mapping can be as follows:

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_CG_00 | PMP-R,cg,f < 1 | dBm |
| P-MPR_CG_01 | 1  PMP-R,cg,f < 2 | dBm |
| P-MPR_CG_02 | 2  PMP-R,cg,f < 3 | dBm |
| … | … | … |
| P-MPR_CG_29 | 29  PMP-R,cg,f < 30 | dBm |
| P-MPR_CG_30 | 30  PMP-R,cg,f < 31 | dBm |
| PCMAX_CG_31 | 31  PMP-R,cg,f | dBm |

Proposal 5: Use a 32 range values for P-MPR reporting values.

## 2.4 The triggering and reporting of the EHR

We propose to have the same set of configurable parameters that a single entry PHR would have. That means we can have an RRC configuration that would look like:

-- ASN1START

-- TAG-EHR-CONFIG-START

EHR-Config ::=                      SEQUENCE {

ehr-PeriodicTimer                   ENUMERATED {sf50, sf100,sf200, sf400,

sf600, sf800, sf900, infinity},

ehr-ProhibitTimer                   ENUMERATED {sf50, sf100,sf200, sf400,

sf600, sf800, sf900, sf1000},

ehr-threshold            			ENUMERATED {sf0, sf5, sf10, sf20, sf25, sf30},

ehr-ModeOtherCG                     ENUMERATED {real, virtual},

...

}

-- TAG-EHR-CONFIG-STOP

-- ASN1STOP

We propose to have a threshold that has a 5% granularity as in the above configuration example as ehr-threshold.

Also, the triggering threshold must be combined with the proximity detection, otherwise the UE can use all its UL power and capabilities, without any restriction.

2.4.1 Event based or periodic reporting

The event based, aperiodic reporting is configured by setting the ehr-PeriodicTimer parameter to infinity in the configuration.  Otherwise, the periodic reporting is done until the proximity detection is reset.

The prohibit timer has a similar interpretation as the PHR prohibit timer.

2.4.2 Priority of the EHR transmission

We believe that this report is very important, and thus should have a high priority among scheduled UL transmissions. Whilst this remains a RAN2 decision, we should mention this into the final LS.

Proposal 6: EHR transmission shall have a high priority.

# Conclusion

In this contribution we discussed and proposed a pre-emptive approach solution for MPE mitigation issues in FR2 along with specification impact and a latency analysis. Along with the following proposals, we have the specifications impact in the Appendix A, B and C at the end of this document.

Proposal 1: The MAC-CE report shall aggregate its measurement per Cell Group, and this can be a single-entry report.

Proposal 2: The new MAC-CE report to be named Energy Headroom Report (EHR).

Proposal 3: The Energy Headroom Report is defined as the unused/overused energy as a percentage relative to the total energy based on the maximum duty cycle at the maximum UE power that complies with MPE limit.

Proposal 4: Use 64 range values for energy headroom reporting values.

Proposal 5: Use a 32 range values for P-MPR reporting values.

Proposal 6: EHR transmission shall have a high priority.

# References

- ICNIRP Guidelines for limiting energy to time-varying electric, magnetic and electromagnetic fields (up to 300GHz),  http://www.icnirp.org/cms/upload/publications/ICNIRPemfgdl.pdf

- R4-1913230 P-MPR and headroom reporting for MPE, Qualcomm Incorporated, RAN4 93, November 2019

- R4-191316170 WF on MPE Enhancement, Qualcomm, RAN4 93, November 2019

# Appendix A: Energy headroom reporting impact on TS 38.133

The 38.133 impact consists of the introduction of a table for the energy headroom report as follows:

A sub-clause can be added under the power headroom report section as:

#### 10.1.17.1.2	Energy Headroom Report Mapping

The energy headroom reporting range is from -30 to 33 %. Table 10.1.17.2-1 defines the report mapping.

Table 10.1.17.1.2-1: Energy headroom report mapping

| Reported value | Measured quantity value (%) |
| --- | --- |
| ENERGY_HEADROOM_0 | EH  - 30 |
| ENERGY_HEADROOM_1 | -30  EH  -29 |
| ENERGY_HEADROOM_2 | -29  EH  -28 |
| ENERGY_HEADROOM_3 | -28  EH  -27 |
|  |  |
| ENERGY_HEADROOM_53 | 23  EH  24 |
| ENERGY_HEADROOM_54 | 24  EH  25 |
| ENERGY_HEADROOM_55 | 25  EH  26 |
| ENERGY_HEADROOM_56 | 26  EH  27 |
| ENERGY_HEADROOM_57 | 27  EH  28 |
| ENERGY_HEADROOM_58 | 28  EH  29 |
| ENERGY_HEADROOM_59 | 29  EH  30 |
| ENERGY_HEADROOM_60 | 30  EH  31 |
| ENERGY_HEADROOM_61 | 31  EH  32 |
| ENERGY_HEADROOM_62 | 32  EH  33 |
| ENERGY_HEADROOM_63 | EH ≥ 33 |

The P-MPR reporting range can be added using a table in a sub-clause 10.1.18.2 for P-MPR report mapping:

10.1.18.2	P-MPR Report Mapping

The P-MPR reporting range is defined from 1 dBm to 33 dBm with 1 dB resolution. Table 10.1.18.2-1 defines the reporting mapping.

Table 10.1.18.2-1 Mapping of P-MPR

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_CG_00 | PMP-R,cg,f < 1 | dBm |
| P-MPR_CG_01 | 1  PMP-R,cg,f < 2 | dBm |
| P-MPR_CG_02 | 2  PMP-R,cg,f < 3 | dBm |
| … | … | … |
| P-MPR_CG_29 | 29  PMP-R,cg,f < 30 | dBm |
| P-MPR_CG_30 | 30  PMP-R,cg,f < 31 | dBm |
| P-MPR_CG_31 | 31  PMP-R,cg,f | dBm |

# Appendix B: Energy headroom reporting impact on TS 38.331

The following 38.331 parameters are proposed as an example for energy headroom (EHR) report configuration:

The IE EHR-Config is used to configure parameters for energy headroom reporting.

EHR-Config information element

-- ASN1START

-- TAG-EHR-CONFIG-START

EHR-Config ::=                      SEQUENCE {

ehr-PeriodicTimer                   ENUMERATED {sf50, sf100,sf200, sf400,

sf600, sf800, sf900, infinity},

ehr-ProhibitTimer                   ENUMERATED {sf50, sf100,sf200, sf400,

sf600, sf800, sf900, sf1000},

ehr-threshold            			ENUMERATED {sf0, sf5, sf10, sf20, sf25, sf30},

ehr-ModeOtherCG                     ENUMERATED {real, virtual},

...

}

-- TAG-EHR-CONFIG-STOP

-- ASN1STOP

EHR-Config field descriptions

ehr-PeriodicTimer Value in number of subframes for EHR reporting as specified in TS 38.321 [3]. Value sf100 corresponds to 100 ms, and so on.

ehr-ProhibitTimer Value in number of subframes for EHR reporting as specified in TS 38.321 [3]. Value sf100 corresponds to 100 subframes, and so on.

ehr-threshold Value in number in percentage for EHR reporting as specified in TS 38.321 [3]. Value sf0 corresponds to 0% energy headroom, value sf5 corresponds to 5% energy headroom, value sf15 corresponds to 15% energy headroom, and so on.

ehr-ModeOtherCG If the UE is configured with only one cell group (no DC), it ignores the field. If DC is configured, one EHR per CG is configured.

# Appendix C: Energy headroom reporting impact on TS 38.321

The impact on TS 38.321 consists of the introduction an EHR MAC CE and the corresponding reporting procedure. An indicated P-MPR value can have a 32 dB range (5 bits) or 63dB (6bits). The MAC CE can be designed similar to the single-entry PHR MAC CE. For example:

### a.b.c	Energy Headroom Reporting

The Energy Headroom reporting procedure is maintained per MAC entity and includes the following information:

-	energy headroom: energy headroom report for Uplink transmissions evaluated over the configured period. For an aperiodically triggered report, a minimum period is assumed for averaging energy measurements.

-	power reduction: P-MPR report for Uplink transmissions in a CG.

RRC controls Energy Headroom reporting by configuring the following parameters:

-	ehr-PeriodicTimer;

-	ehr-ProhibitTimer;

-	ehr-ModeOtherCG;

-	ehr-threshold;

An Energy Headroom Report (EHR) shall be triggered if the UE requires MPE management (as allowed by TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) and any of the following events occur:

-	ehr-ProhibitTimer is not running and the measured energy has exceeded ehr-threshold for at least one activated Serving Cell of any MAC entity since the last transmission of a EHR in this MAC entity when the MAC entity has UL resources for new transmission;

-	ehr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the energy headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell configured in the MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or changed);

If the MAC entity has UL resources allocated for a new transmission, the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start ehr-PeriodicTimer;

1>	if the Energy Headroom reporting procedure determines that at least one EHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for EHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in subclause 5.4.3.1:

2>	obtain the value of the EH report from the physical layer for the corresponding cell group;

2>	obtain the value for the corresponding PMP-R,cg,f field from the physical layer;

2>	instruct the Multiplexing and Assembly procedure to generate and transmit the EHR MAC CE as defined in subclause 6.1.3.x based on the values reported by the physical layer.

2>	start or restart ehr-PeriodicTimer;

2>	start or restart ehr-ProhibitTimer;

#### 6.1.3.x	EHR MAC CE

The EHR MAC CE is identified by a MAC PDU subheader with LCID as specified in Table #.#.#.

It has a fixed size and consists of two octets defined as follows (figure 6.1.3.x-1):

-	R: Reserved bit, set to "0";

-	Energy Headroom (EH): This field indicates the energy headroom level. The length of the field is 6 bits. The reported EH and the corresponding energy headroom levels are shown in Table x.y.z-1 below (the corresponding measured values in dB are specified in TS 38.133);

-	PMP-R, cg: This field indicates the PMP-R, cg (as specified in TS 38.101). The reported PMP-R, cg values range are shown in Table x.y.z.2 (the corresponding measured values in dBm are specified in TS 38.133).

-	P: This field indicates whether the MAC entity will apply the estimated power back-off due to power management (as allowed by P-MPR, cg as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]). The MAC entity shall set P=1 when an imminent MPE event is about to happen and the corresponding reported P-MPR, cg field is the estimated power back-off to be applied after the expiration of the ehr-ProhibitTimer;

Figure 6.1.3.x-1: EHR MAC CE

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R | R | Energy Headroom |  |  |  |  |  |
| R | R | P | PMP-R, cg |  |  |  |  |

Table 6.1.3.x-1: Energy Headroom levels for EHR

| EH | Reported value |
| --- | --- |
| 0 | ENERGY_HEADROOM_0 |
| 1 | ENERGY _HEADROOM_1 |
| 2 | ENERGY _HEADROOM_2 |
| 3 | ENERGY _HEADROOM_3 |
| … |  |
| 60 | ENERGY _HEADROOM_60 |
| 61 | ENERGY _HEADROOM_61 |
| 62 | ENERGY _HEADROOM_62 |
| 63 | ENERGY_HEADROOM_63 |

Table 6.1.3.x-2: Effective power reduction for P-MPR

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| P-MPR_C_00 | PMP-R,c,f < 1 | dBm |
| P-MPR_C_01 | 1 PMP-R,c,f < 2 | dBm |
| P-MPR_C_02 | 2 PMP-R,c,f < 3 | dBm |
| … | … | … |
| P-MPR_C_30 | 30 PMP-R,c,f < 31 | dBm |
| P-MPR_C_31 | 32 PMP-R,c,f < 33 | dBm |
| PCMAX_C_32 | 33 PMP-R,c,f | dBm |