3GPP TSG-RAN WG4 Meeting #94-e                                                            R4-2000240

Online, 24th Feb-6th Mar, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_2-5_n66 and DC_2-5-5_n66

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_2-5_n66 and DC_2-5-5_n66 according to the request in [1].

2. Reference

- RP-192815, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.65	DC_2-5_n66 and DC_2-5-5_n66

### 5.1.65.1	Operating bands for DC

Table 5.1.65.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n66 | CA_2-5 | n66 | No |
| DC_2-5-5_n66 | CA_2-5-5 | n66 | No |

### 5.1.65.2	Configuration for DC

Table 5.1.65.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5A | n66A |
| DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5B | n66A |
| DC_2A-5A-5A_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5A-5A | n66A |

5.1.65.3	Coexistence studies

Based on co-existence studies of DC_2A_n66A in TR 37.863-01-01, the third order IMD may fall into band 2 and/or n66; the fifth order IMD may fall into band n66.

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, the second order IMD may fall into band 5.

5.1.65.4	∆TIB and ∆RIB values

For DC_2-5_n66 and DC_2-5-5_n66, the TIB,c and RIB,c values are derived from CA_2-5-66 in TS 36.101.

Table 5.1.65.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.65.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | n66 | 0.3 |

### 5.1.65.5	REFSENS requirements

The MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_2A_n66A and DC_5A_n66A) in TS 38.101-3. There is no specific MSD for 3DL2UL DC.

<End of Text Proposal>