3GPP TSG-RAN WG4 Meeting #94-e                                          R4-2000275

Electronic Meeting, 24th Feb– 6th Mar, 2020

Agenda Item:   8.5.3

Source:        Samsung

Title:          TP for TS38.174: IAB system parameters

Document for:  Approval

# Introduction

This is the resubmission of R4-1916001 to provide TP for TS38.174 regarding the system parameters. Since the status of latest TR skeleton [2] is not clear, this one is still based on the previous version of the TS skeleton.

# Reference

[1] R4-1912890, WF on IAB system parameter

[2] R4-19160000, WF on IAB specification structure and terminology

# TP for IAB TS 38.174

<Start of the TP>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[5] 	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

<Unchanged clause skipped>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IAB channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink

NOTE 1:	The IAB channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements. IAB channel bandwidth shall be defined for both IAB-DU and IAB-MT.

NOTE 2:	It is possible for the IAB-DU to transmit to and/or receive from one or more UE [or IAB-MT] bandwidth parts that are smaller than or equal to the IAB-DU transmission bandwidth configuration, in any part of the IAB-DU transmission bandwidth configuration.

NOTE 3:	The UE channel bandwidth definition shall be applied for IAB-MT unless state otherwise.

NOTE 4:	The BS channel bandwidth definition shall be applied for IAB-DU unless state otherwise.

## 3.2	Symbols

BWChannel	IAB channel bandwidth

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BW	Bandwidth

FR	Frequency Range

IAB           Integrated Access and Backhaul

NR	New Radio

<Unchanged clause skipped>

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and IAB channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and IAB channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which IAB can operate according to the present version of the specification are identified as described in table 5.1-1 in [5].

## 5.2	Operating bands

NR IAB is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

Table 5.2-1 NR IAB operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2469 MHz – 2690 MHz | 2469 MHz – 2690 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

Table 5.2-2: NR IAB operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

## 5.3	Channel bandwidth

### 5.3.1	General

The requirement specified in 5.3.1 of TS38.104 [5] applies for IAB-DU. And the requirement specified in 5.3.1 of TS38.101-1[2] applies for IAB-MT.

### 5.3.2	Transmission bandwidth configuration

The requirement specified in 5.3.2 of TS38.104 [5] for each BS channel bandwidth applies for each IAB channel bandwidth of IAB.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The requirement specified in 5.3.3 of TS38.104 [5] for each BS channel bandwidth applies for each IAB channel bandwidth of IAB.

### 5.3.4	RB alignment

The requirement specified in 5.3.4 of TS38.104 [5] applies for IAB-DU. And the requirement specified in 5.3.4 of TS38.101-1[2] applies for IAB-MT for NR FR1. The requirement specified in 5.3.4 of TS38.101-2[3] applies for IAB-MT for NR FR2.

### 5.3.5	IAB channel bandwidth per operating band

The requirements in this specification apply to the combination of IAB channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1 and in table 5.3.5-2 for FR2. For IAB, the transmission bandwidth configuration in 5.3.2 shall be supported for each of the IAB channel bandwidths within the IAB capability.

Table 5.3.5-1: IAB channel bandwidths and SCS per operating band in FR1

| NR band / SCS / IAB channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 302 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
| NOTE 1: This IAB channel bandwidth is optional for IAB-MT in this release of the specification. NOTE 2: This IAB channel bandwidth applies for IAB-DU only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: IAB channel bandwidths and SCS per operating band in FR2

| NR band / SCS / IAB channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

## 5.4	Channel arrangement

The requirements specified in 5.4 of TS38.104 [5] apply for IAB.

<End of the TP>