3GPP TSG-RAN WG4 Meeting #94-e                                           R4-2000283

Electronic meeting, 24th Feb– 6th Mar, 2020

Agenda Item:   8.5.4.2.2

Source:        Samsung

Title:          FR2 IAB receiver spurious emission

Document for:  Approval

# Introduction

In previous meeting it was proposed to use common RX spurious emission for IAB node in [1]. And it is captured in AH minutes [2] as “ Reuse Rx spurious emission from BS” for IAB.

Based on this agreement, the TP for IAB OTA RX spurious emission is provided as below.

# TP for TS38.174

<Start of TP>

## 10.7	OTA receiver spurious emissions

### 10.7.1 General

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

The metric used to capture OTA receiver spurious emissions for IAB is total radiated power (TRP), with the requirement defined at the [RIB].

### 10.7.2 Minimum requirement

The OTA RX spurious emissions requirement shall apply during the all IAB transmitters in OFF period only.

For RX only [multi-band RIB], the OTA RX spurious emissions requirements are subject to exclusion zones in each supported operating band.

For the IAB operating in FR1, the OTA RX spurious emissions requirement is that for each basic limit specified in table 10.7.2-1 of TS38.104[2], the power sum of emissions at the [RIB] shall not exceed limits specified as the basic limit + X, where X = 9 dB, unless stated differently in regional regulation.

For the IAB operating in FR2, the power of any RX spurious emission shall not exceed the limits in table 10.7.3-1 of TS38.104 [2].

<End of TP>

# Reference

[1] R4-1916161, IAB Ad Hoc meeting minutes