3GPP TSG-RAN WG4 Meeting #94-e                                           R4-2000284

Electronic meeting, 24th Feb– 6th Mar, 2020

Agenda Item:   8.5.4.2.2

Source:        Samsung

Title:           IAB receiver In channel selectivity

Document for:  Approval

# Introduction

In updated version for IAB TS skeleton in [1], the primary clause of Receiver in-channel selectivity is still subdivided in to two sub-clauses for IAB-DU and IAB-MT respectively. Even though we provide our consideration for this aspect in reflector, it seems not include in the final version. Hence in this contribution we share consideration on this issue further. And it is suggested that the receiver in-channel selectivity requirement should be dedicated for IAB-DU.

# Discussion

In channel selectivity is defined for BS to verify BS receiver capability to handle one target wanted signal with the other NR signal at higher PSD level existing in same channel which can be recognized as interference signal for the first signal. The example of ICS is illustrated as figure below.

The necessity of ICS requirement can be understood for BS, of which it may be possible to receive multiple User equipment transmission with different PSD level within channel bandwidth simultaneously. However, it can’t be recognized as typical case in view of DL reception in angle of User device. Usually it is expected that the DL transmission from gNB will have equal PSD within the channel bandwidth. Hence there is no such requirement in UE side. And this should be still valid assumption for IAB-MT DL reception. Consequently, the In-channel selectivity requirement should be dedicated for IAB-DU reception, but it is not needed for IAB-MT receiver. Furthermore, the IAB TS skeleton should be updated in line with this.

Observation 1: the motivation to include ICS requirement is not applicable for IAB-MT reception.

Proposal 1: In-channel selectivity requirement should be defined for IAB-DU only.

Proposal 2: IAB TS 38.174 should be updated to remove In-channel selectivity for IAB-MT related sub-clauses.

# 3 Conclusion

In this contribution we provide the view on necessity of ICS requirement for IAB-MT. And the proposals from our side are:

Proposal 1: In-channel selectivity requirement should be defined for IAB-DU only.

Proposal 2: IAB TS 38.174 should be updated to remove In-channel selectivity for IAB-MT related sub-clauses.

# 3 Reference

[1] R4-1916000, WF on IAB TS skeleton