3GPP TSG-RAN WG4 Meeting #94-e		            R4-2000303

Online, 24th February – 6th March, 2020

Agenda item:	8.17.2.1.2

Source:	Samsung

Title:	Simulation results for UE demodulation requirements under HST single tap

Document for:	Discussion

# Introduction

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved.

In this contribution, we provide the updated simulation results for HST -single tap channels based on the assumptions agreed in RAN4#93 found in [1] for further alignment.

# Common test setup

Table1: Simulation assumption for HST-single tap channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 1×2; 1×4 (baseline) 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | HST-single tap |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=300m, Dmin=2m |  |
| Rank | Rank =1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - Option 1: 1250Hz - Option 2: 875Hz | 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

Note: the following simulation results are based on the sample changed HST-single tap channel.

## FDD Simulation results

Figure 3-1:FDD Simulation results for NR PDSCH under HST single tap for 1x2 and 1x4 antenna configuation

## TDD Simulation results

Figure 3-2:TDD Simulation results for NR PDSCH under HST- single tap with 1x2 and 1x4

## Summary of @ 70% of max throughput

Table 1: Ideal and impairment simulation results for NR PDSCH under HST- single tap channel with 1x2 and 1x4

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR（dB）@70% Max TP (ideal) | SNR（dB）@70% Max TP (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 1250 | 1Tx 2Rx ULA Low | 13 | 3.9 | 6.4 |
| 2 |  |  |  | 17 | 7.7 | 10.2 |
| 3 |  |  | 1Tx 4Rx ULA Low | 13 | 0.8 | 3.3 |
| 4 |  |  |  | 17 | 4.7 | 7.2 |
| 5 |  | 875 | 1Tx 2Rx ULA Low | 13 | 3.6 | 6.1 |
| 6 |  |  |  | 17 | 7.2 | 7.7 |
| 7 |  |  | 1Tx 4Rx ULA Low | 13 | 0.6 | 3.1 |
| 8 |  |  |  | 17 | 4.1 | 6.6 |
| 9 | 40MHz/30kHz | 1667 | 1Tx 2Rx ULA Low | 13 | 3.5 | 6.0 |
| 10 |  |  |  | 17 | 6.6 | 9.1 |
| 11 |  |  | 1Tx 4Rx ULA Low | 13 | 0.6 | 3.1 |
| 12 |  |  |  | 17 | 3.7 | 6.2 |

# Conclusion

In this contribution we provide simulation results for HST- single tap channels.

# Reference

- R4-1915926 , Draft WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC

- R4-1913374, Simulation results for UE demodulation requirements under HST single tap,Samsung