3GPP TSG-RAN WG4 Meeting #94-e              	R4-2000307

Online, 24th Feb– 6th Mar, 2020

Agenda item:	8.17.2.2.2

Source:	Samsung

Title:	Ideal and impairment simulation results for NR HST PRACH

Document for:	Discussion

# 1	Introduction

During the last NR HST discussion in RAN#93b meeting, the requirements of PRACH under high speed scenario were further discussed. The simulation assumption with PRACH was approved and captured in WF [2].

In this contribution, the ideal and impairment simulation results are provided for alignment purpose.

# 2	Simulation results

In this section, the ideal and impairment results for PRACH format 0 with restricted set type A and type B, as well as short sequence format are provided. The test preamble configurations for format 0 and short sequence formats are specified as follows:

Table 1: test preambles for high speed Mode restricted set type A and B

| Burst format | Restricted set | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | Type A | 15 | 384 | 0 |
|  | Type B | 15 | 30 | 30 |

Table 2: test preambles for short sequence format with 15KHz and 30KHz SCS

| Burst format | SCS | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Table 3: Ideal and impairment results for High Speed Mode restricted set type A for Format 0

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | SNR [dB] Ideal | SNR[dB] Impairment |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 625Hz | -14.36 | -12.36 |
|  |  | AWGN | 1340Hz | -16.09 | -14.09 |
|  |  | TDLC300-100 Low | 400Hz | -7.32 | -5.32 |
|  | 4 | AWGN | 625Hz | -16.77 | -14.77 |
|  |  | AWGN | 1340Hz | -18.54 | -16.54 |
|  |  | TDLC300-100 Low | 400Hz | -12.60 | -10.60 |
|  | 8 | AWGN | 625Hz | -19.09 | -17.09 |
|  |  | AWGN | 1340Hz | -20.99 | -18.99 |
|  |  | TDLC300-100 Low | 400Hz | -16.87 | -14.87 |

Table 4: Ideal and impairment results for High Speed Mode restricted set type B for Format 0

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | SNR [dB] Ideal | SNR [dB] Ideal |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 625Hz | -13.92 | -11.92 |
|  |  | AWGN | 2334Hz | -15.80 | -13.80 |
|  |  | TDLC300-100 Low | 400Hz | -6.97 | -4.97 |
|  | 4 | AWGN | 625Hz | -16.31 | -14.31 |
|  |  | AWGN | 2344Hz | -18.28 | -16.28 |
|  |  | TDLC300-100 Low | 400Hz | -12.34 | -10.34 |
|  | 8 | AWGN | 625Hz | -18.55 | -16.55 |
|  |  | AWGN | 2334Hz | -20.63 | -18.63 |
|  |  | TDLC300-100 Low | 400Hz | -16.58 | -14.58 |

As frequency offset for short sequence format, it is agreed to align with PUSCH maximum Doppler shift in WF [2] as follows:

- 15KHz SCS: 1740 Hz

- 30KHz SCS: 3334 HZ

Table 5: Ideal and impairment results for short sequence format for 15 KHz

| Format | Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | 1 | 2 | AWGN | 1740Hz | -13.23 | -11.23 |
| A2 |  | 4 | AWGN |  | -15.47 | -13.47 |
| A2 |  | 8 | AWGN |  | -17.59 | -15.59 |
| B4 |  | 2 | AWGN | 1740Hz | -16.73 | -14.73 |
| B4 |  | 4 | AWGN |  | -18.84 | -16.84 |
| B4 |  | 8 | AWGN |  | -20.80 | -18.80 |
| C2 |  | 2 | AWGN | 1740Hz | -13.25 | -11.25 |
| C2 |  | 4 | AWGN |  | -15.48 | -13.48 |
| C2 |  | 8 | AWGN |  | -17.53 | -15.53 |

Table 6: Ideal and impairment results for short sequence format for 30 KHz SCS

| Format | Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | SNR [dB] Ideal | SNR [dB] Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | 1 | 2 | AWGN | 3334HZ | -13.10 | -11.10 |
| A2 |  | 4 | AWGN |  | -15.39 | -13.39 |
| A2 |  | 8 | AWGN |  | -17.45 | -15.45 |
| B4 |  | 2 | AWGN | 3334HZ | -16.70 | -14.70 |
| B4 |  | 4 | AWGN |  | -18.74 | -16.74 |
| B4 |  | 8 | AWGN |  | -20.65 | -18.65 |
| C2 |  | 2 | AWGN | 3334HZ | -13.13 | -11.13 |
| C2 |  | 4 | AWGN |  | -15.38 | -13.38 |
| C2 |  | 8 | AWGN |  | -17.45 | -15.45 |

# 3	Conclusion

In this contribution, the ideal and impairment results of HST PRACH are presented for requirement finalization.

# References

R4-1915914 Way forward for NR PRACH HST demodulation requirements, Samsung

R4-1915886 Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon