3GPP TSG-RAN WG4 Meeting #94-e              	R4-2000312

Online, 24th February– 6th March, 2020

Agenda item:	7.11.4

Source:	Samsung

Title:	View on BS demodulation requirement for LTE additional enhancement for NB IoT

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the work scope of performance part related with LTE Rel-16 WI additional enhancement for NB-IoT is discussed. The generally issue to identify the features that related BS demodulation requirement is captured in WF [1] as:

- BS demodulation requirement - FFS whether to introduce the BS demodulation performance requirement(s) to verify the following features - NPUSCH configured with multi-TB scheduling - Coexistence of NPUSCH and NR - When NB-IoT transmission is postponed by subframe(s) for NR UL transmission

In this contribution, the view on additional enhancement NB-IoT performance requirement is presented.

# 2	Discussion

## 2.1	Multi-TB scheduling

Similarly eMTC, LTE Rel-16 additional enhancement for NB IoT WI can also support the feature of multi-TB scheduling with a single DCI, in order to reduce the control channel traffic load. The TBs can support both interleaved and non-interleaved transmission.

As agreed in RAN1#98 meeting

For unicast, for a Rel-16 UE configured with multiple TB scheduling: - When one TB is scheduled by the DCI, the repetitions for one transport block are contiguously transmitted - When multiple TBs are scheduled by the DCI - The repetitions for one transport block can be either contiguously transmitted or interleaved. - Interleaving is an eNB configured feature - FFS: Interleaving feature is a UE optional feature

As mentioned, multi-TB with interleaving is eNB configured feature. It will be configured based on practical scenario. During the RAN1 discussion, the gain of multi-TB with interleaving is verified under high Doppler frequency. In the typical NB-IoT deployment, most UE are stationary or have low speed, thus UE will rarely suffering high Doppler frequency. Compared with eMTC, there are only up to 2TB can be scheduled in single DCI, therefore the gain of interfaced transmission will be further limited. The diversity gain can be achieved with repetition and frequency hopping. So, in our view, the typical scheduling scenario will be non-interleaving transmission, configured by eNB.

In terms of demodulation requirement, since each TB corresponds to one HARQ process, therefore the multi TB requirement can be verified with existing Rel-14 NB-IoT requirement.

For NB IoT, both single tone and multi-tone transmission can be supported. In terms of interleaving granularity, the following is agreed in RAN1#99 meeting

For the uplink multi-tone case, interleaving granularity is N* NRU *NULslots, where the NRU is the number of RUs, NULslots is the number of slots occupied by 1 RU. - When the repetition is less than N, interleaving is not supported. - N=4 For uplink single-tone, the granularity is based on N* NRU *NULslots , with N =1

The interleaving granularity is similar with as the NPUSCH transmission mapping principle as follows

After mapping to slots, the slots shall be repeated additional times, before continuing the mapping of to the following slot, where

My understanding, LTE NB-IoT has already supported the similar non-continuously transmission. In case of the resource reserved is overlapping, the NPUSCH transmission in overlapped  slots is postponed until the next. In case of large number of repetition, the whole transmission will be divided two parts with gap.

Therefore, for each TB transmission, in our view, the demodulation requirement can be verified existing requirement with valid-subframe configuration in FDD and UL-DL configuration in TDD. We donot think this feature will require the new BS demodulation requirement.

Observation 1: The interleaving granularity can be verified with valid-subframe configuration similarly

Proposal 1: No BS demodulation requirement for multi-TB scheduling.

## 2.2	Coexistence of PUSCH and NR

RAN1 has studied the aspects of FDD/TDD NB-IoT coexistence with NR, and introduce DL/UL resource reservation in NB-IoT for as least FDD/TDD NB-IoT non anchor carriers to avoid resource overlap between NR and NB-IoT when NB-IoT is deployed within an NR carrier.

The resource reservation in FDD/TDD NB IoT considers the following aspects:

- Subframe/slot/symbol level granularity

- Dropped/postponed resource associated to a certain granularity

Regarding the coexistence with NR, the following are agreed in RAN1 #99 meeting

- For unicast, NPUSCH scrambled by C-RNTI or SPS-C-RNTI that would fall into the reserved resource is postponed for subframe-level reserved resources. - For unicast, NPUSCH scrambled by C-RNTI and SPS-C-RNTI that would fall into the reserved - resource is dropped for symbol and slot-level reserved resources. - For slot-level reservation in UL NB-IoT, DMRS transmission in a reserved slot is dropped. - (for 15kHz) - For symbol-level reservation in UL NB-IoT, DMRS symbols can be reserved - For 10msec periodicity, if only subframe level granularity: a bitmap with length 10 bits is used for indicating reserved subframes

As mentioned, in case of subframe level based reserved, NB-IoT transmission will be postponed until the next uplink subframe that is not fully reserved, with a length 10bits bitmap indicated.

As for slot-level reservation, DMRS transmission in a reserved slot is dropped.  So, only the channel estimation will be slightly impacted. As mentioned, most UE are stationary or have low speed deployed in NB-IoT scenario. Thus, we do not think the demodulation requirement will be large different with existing requirements, considering the cross-frame channel estimation could be applied in NB-IoT.

Based on above analysis, we do not think there is no necessary to specify the new BS demodulation requirement for coexistence of PUSCH and NR.

Proposal 2: No new BS demodulation requirement for coexistence of PUSCH and NR.

# Conclusion

In this contribution, the initial results are provided to analyze the URLLC with high reliability requirement.

Observation 1: The interleaving granularity can be verified with valid-subframe configuration similarly

Proposal 1: No BS demodulation requirement for multi-TB scheduling.

Proposal 2: No new BS demodulation requirement for coexistence of PUSCH and NR.

# References

R1-1915909, Way forward on LTE UE and BS performance requirements for additional enhancements for NB-IOT, Huawei