3GPP TSG-RAN WG4 Meeting #94-e                                                           R4-2000320

Electrical Meeting, Feb.24th – Mar.6th

Source:	Samsung

Title:	Test case design for Enhanced Type II codebook

Agenda item:			8.11.3.3

Document for:	Approval

- Introduction

As agreed in RAN#85, Rel-16 NR eMIMO WI has below work objectives for RAN4 core part and performance part:

- Specify core requirements associated with the items specified by RAN1 [RAN4-Core]

  - Specify RRM requirements

  - Identify impact on RF requirements for the reduced PAPR pi/2-BPSK DMRS and, if needed, specify RF requirements

  - Identify impact on RF requirements for the uplink full power transmission and, if needed, specify RF requirements

- Specify necessary performance requirements for the specified enhancements. [RAN4-Performance]

In [1], we provide brief analysis for performance impact of Rel-16 e-MIMO WI:

| Sub-items |  | BS Demod | UE Performance |  |
| --- | --- | --- | --- | --- |
|  |  |  | UE Demod | CSI |
| Item 1: CSI enhancement for MU-MIMO |  | No | Yes | Yes |
| Item 2: Multi-TRP/Panel transmission |  | No | Yes | No |
| Item3:Beam management enhancement | Item3a: L1-SINR measurement | No | No | No |
|  | Item3b: BFR for Scell | No | No | No |
|  | Item3c: DL/UL beam indication with reduced latency and overhead | No | No | No |
| Item4: Full TX power UL transmission |  | No | No | No |
| Item5: Low PAPR RS |  | FFS | FFS | No |

In this contribution, test case design for PMI test cases with enhanced Type II codebook provided.

- Discussion

Rel-16 Type II codebook construction

Rel-16 Type II  Precoders for a layer is given by size-P×N3 matrix W=W1W2WfH

- P=2N1N2= #SD dimensions

- N3= #FD dimensions

- Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank)

- Spatial domain (SD) compression - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected - Compression in spatial domain using W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II) - Frequency-domain (FD) compression - Compression via Wf=fk0 fk1…fkM-1, where fkmm=0M-1 are M size-N3×1 orthogonal DFT vectors - Number of FD-components M is configurable - Linear combination by W2 (for a layer) - W2=c0,0⋯c0,M-1⋮⋱⋮c2L-1,0⋯c2L-1,M-1 is composed of K=2LM linear combination coefficients

Rel-16 Type II codebook construction consists of below parameters:

- n1-n2-codebookSubsetRestriction-r16 : (N1,N2), (O1, O2), PCSI_RS

  - Total number of antenna ports P = 2N1N2 which supporting 4, 8, 12,16,24 and 32 ports for Type II codebook

  - (N1,N2), (O1,O2) as in table below

Table 5.2.2.2.1-2: Supported configurations of and

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |
| 8 | (2,2) | (4,4) |
|  | (4,1) | (4,1) |
| 12 | (3,2) | (4,4) |
|  | (6,1) | (4,1) |
| 16 | (4,2) | (4,4) |
|  | (8,1) | (4,1) |
| 24 | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,1) |
| 32 | (4,4) | (4,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,1) |

- numberOfPMISubbandsPerCQISubband: R =1 or 2

- paramCombination-r16:L, β and pυ

  - L: number of beams

  - β : compression factor in spatial domain

  - pυ : compression factor in frequency domain

Table 5.2.2.2.5-1: Codebook parameter configurations for L,  β and pυ

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |
| 7 | 6 | ¼ | - | ½ |
| 8 | 6 | ¼ | - | ¾ |

In order to take a comparable performance compared to Rel-15 NR Type I and Type II PMI test cases, we proposed to use 16 CSI-RS ports with (N1,N2) = (4,2) as starting points. For number of PMI sub-bands perCQI band, we suggest to take R =2. Regarding paramCombination-r16, we recommend to take value 6, with this configuration, we can maximize number of candidate codebooks. From UE processing respective, this requires maximum UE calculation complexity and acts like a pressure test.

Proposal 1: Introduce PMI test cases with enhanced Type II codebook with below parameters:

- Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1, O2)  = (4,4)

- numberOfPMISubbandsPerCQISubband: R =2

- paramCombination-r16: 6, with L =4, pv =1/2, β=1/2

Test metric

Proposal 2: For test metric, two alternatives can be considered：

- Alt 1: Relative Throughput ration between follow PMI and random PMI

- Alt 2： Relative throughput ratio with following PMI between enhanced Type II codebook and Rel-15 Type II codebook

Beam-steering model

For beam-steering model, the multi-beam steering approach as specified in B.2.3B.4A of TS36.101 can be used as starting point to further extend applied for L beams

- beam index

- ， relative power of the l beam compared to first beam

- , total power scaling factor

For simplicity, the power of beams can be fixed as equivalent to first beams then beam steering model can as follow

Proposal 3: Taking beam steering approach as specified in B.2.3B.4A of TS36.101 as staring point with further extension applicable for number of L beams

MCS&Rank

Proposal 4: 16QAM ½ with rank2 can be selected as starting point.

Other test parameters

For other test parameters, we can reuse test parameters from Rel-15 NR PMI test cases as starting points.

Proposal 5: Reuse test parameters from Rel-15 NR PMI test cases as starting points.

3. Conclusion

In this contribution, test case design for PMI test cases with enhanced Type II codebook provided.

Proposal 1 (Codebook construction): Introduce PMI test cases with enhanced Type II codebook with below parameters:

- Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1, O2)  = (4,4)

- numberOfPMISubbandsPerCQISubband: R =2

- paramCombination-r16: 6, with L =4, pv =1/2, β=1/2

Proposal 2 (Test metric): two alternatives can be considered：

- Alt 1: Relative Throughput ration between follow PMI and random PMI

- Alt 2： Relative throughput ratio with following PMI between enhanced Type II codebook and Rel-15 Type II codebook

Proposal 3 (Beam steering model): Taking beam steering approach as specified in B.2.3B.4A of TS36.101 as staring point with further extension applicable for number of L beams

- beam index

- ， relative power of the l beam compared to first beam

- , total power scaling factor

Proposal 4 (MCS and Rank): 16QAM ½ with rank2 can be selected as starting point.

Proposal 5 (other test parameters): Reuse test parameters from Rel-15 NR PMI test cases as starting points.

4. Reference

[1] R4-2000319, “Overview on performance requirements for Rel-16 eMIMO WI “, Samsung