3GPP TSG-RAN WG4 Meeting #94-e                                                           R4-2000321

Electrical Meeting, Feb.24th – Mar.6th

Source:	Samsung

Title:	Initial simulation results for PMI reporting with enhanced type II codebook

Agenda item:			8.11.3.3

Document for:	Discussion

- Introduction

As agreed in RAN#85, Rel-16 NR eMIMO WI has below work objectives for RAN4 core part and performance part:

- Specify core requirements associated with the items specified by RAN1 [RAN4-Core]

  - Specify RRM requirements

  - Identify impact on RF requirements for the reduced PAPR pi/2-BPSK DMRS and, if needed, specify RF requirements

  - Identify impact on RF requirements for the uplink full power transmission and, if needed, specify RF requirements

- Specify necessary performance requirements for the specified enhancements. [RAN4-Performance]

In [1], we provide brief analysis for performance impact of Rel-16 e-MIMO WI:

| Sub-items |  | BS Demod | UE Performance |  |
| --- | --- | --- | --- | --- |
|  |  |  | UE Demod | CSI |
| Item 1: CSI enhancement for MU-MIMO |  | No | Yes | Yes |
| Item 2: Multi-TRP/Panel transmission |  | No | Yes | No |
| Item3:Beam management enhancement | Item3a: L1-SINR measurement | No | No | No |
|  | Item3b: BFR for Scell | No | No | No |
|  | Item3c: DL/UL beam indication with reduced latency and overhead | No | No | No |
| Item4: Full TX power UL transmission |  | No | No | No |
| Item5: Low PAPR RS |  | FFS | FFS | No |

In this contribution, initial simulation results for Type II codebook PMI test cases provided.

- Simulation assumption

- Table 1 Simulation assumptions for Type II PMI (FDD)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLC 300ns,5Hz |
| Antenna configuration |  |  | - High XP 16 x 2，16x4 (N1,N2) = (4,2) (O1,O2) = (4,4) |
| MCS |  |  | 13 |
| Rank |  |  | 1/2/3/4 |
| Beamforming Model |  |  | As specified in Annex B2.3B.4A [2] |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3 (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3 ) |  | Row 12 (0,2,4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (12,-) |
|  | CSI-RS interval and offset |  | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | PMI feedback Fix MCS, RI |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | Type I, Type II, Enhanced Type II |
|  | NumberOfBeams |  | 1,2,4,6 |
|  | NPSK |  | 4,8 |
|  | SubbandAmplitude |  | ‘ON’ |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the eNB downlink before slot#(n+3). Note 3: Randomization of the principle beam directions shall be used as specified in Annex B.2.3B.4A of TS 36.101 with Relative power ratio (p) equals to 1. |  |  |  |

Table 2: paraCombination_r16 configuration (layer v =1, 2)

| parameter | paraCombination_r16=1 | paraCombination_r16=4 | paraCombination_r16=6 |
| --- | --- | --- | --- |
| L | 2 | 4 | 4 |
| pυ | 1/4 | 1/4 | 1/2 |
| β | 1/4 | 1/2 | 1/2 |
| K0 | 2 | 8 | 16 |

- Initial simulation results

16x2 rank1

Figure 1 and table 1 below summarize Absolut throughput performance under 16Tx rank1 cases with different codebook configurations.

Figure 1: PMI performance with Type I, Type II and Type II enhance codebook (16Tx Rank1 case)

Table 1: SNR points summary for 16QAM Rank1 case

|  | Type I | Type II | Enhanced Type II with ParaCombination_r16=1 | Enhanced Type II with paraCombination_r16=4 |
| --- | --- | --- | --- | --- |
| SNR point at 60% throughput | -2.0 | -0.5 | -3.1 | -- |
| SNR point at 70% throughput | -0.8 | 0.9 | -1.7 | -3.8 |
| SNR point at 80% throughput | 0.8 | 2.9 | -0.5 | -2.1 |

16x2 rank2

Figure 2 and table 3 below summarize Absolut throughput performance under 16Tx rank2 cases with different codebook configurations.

Figure 2 PMI performance with Type I, Type II and Type II enhance codebook (16Tx Rank2 case)

Table 2: SNR points summary for 16QAM Rank2 case

|  | Type I | Type II | Enhanced Type II with ParaCombination_r16=1 | Enhanced Type II with paraCombination_r16=4 | Enhanced Type II with paraCombination_r16=6 |
| --- | --- | --- | --- | --- | --- |
| SNR point at 60% throughput | 4.1 | 7.5 | 5.3 | 4.1 | 1.2 |
| SNR point at 70% throughput | 5.6 | 9.2 | 7.6 | 6.1 | 3.1 |
| SNR point at 80% throughput | 7.8 | 11.2 | 9.5 | 8.0 | 4.8 |

- Conclusion

In this contribution, we provide preliminary simulation results for Type II enhanced codebook.

We can observe that with Enhanced Type II with paraCombination_r16=6 achieve better performance than other case meanwhile UE processing complexity and reporting overhead increased.

4. Reference

[1] R4-2000319, “Overview on performance requirements for Rel-16 eMIMO WI “, Samsung