3GPP TSG-RAN WG4 Meeting #94-e		R4-2000360

E-meeting, 24 February – 6 March, 2020

Agenda item:	8.18.1.1

Source:	Intel Corporation

Title:	Simulation results for Normal CA requirements

Document for:	Discussion

# Introduction

In RAN4 #93 paper with simulation assumptions for Normal CA requirements was approved [1].

In this paper we provide simulation results for Normal CA scenarios.

# Discussion

## FR1 15 kHz simulation results

In this section we provide alignment and impairment results for FR1 FDD and TDD scenarios with 15 kHz SCS. Figure 1 illustrates PDSCH throughput curves for different BW configurations and number of UE receive antennas.

| 2 Rx UE, FDD |  |
| --- | --- |
| 4 Rx UE, FDD |  |
| 2 Rx UE, TDD |  |
| 4 Rx UE, TDD |  |
| Figure 1. PDSCH simulation results for FR1 15 kHz. |  |

Summary of alignment simulation results for FR1 scenarios with 15 kHz SCS is provided in Table 1.

Table 1. Alignment FR1 15 kHz simulation results

| Duplex mode | Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2 Rx | 10.7 | 10.8 | 10.8 | 10.9 | 11.1 | 10.8 | 11.2 | 11.7 |
|  | 4 Rx | 6.0 | 6.1 | 6.2 | 6.1 | 6.3 | 6.1 | 6.3 | 6.5 |
| TDD | 2 Rx | 10.9 | 11.1 | 11.2 | 11.1 | 11.3 | 11.1 | 11.6 | 12.0 |
|  | 4 Rx | 6.2 | 6.2 | 6.3 | 6.2 | 6.4 | 6.2 | 6.4 | 6.6 |

Summary of impairment simulation results for FR1 scenarios with 15 kHz SCS is provided in Table 2.

Table 2. Impairment FR1 15 kHz simulation results

| Duplex mode | Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2 Rx | 12.7 | 12.8 | 12.8 | 12.9 | 13.1 | 12.8 | 13.2 | 13.7 |
|  | 4 Rx | 8.0 | 8.1 | 8.2 | 8.1 | 8.3 | 8.1 | 8.3 | 8.2 |
| TDD | 2 Rx | 12.9 | 13.1 | 13.2 | 13.1 | 13.3 | 13.1 | 13.6 | 14.0 |
|  | 4 Rx | 8.2 | 8.2 | 8.3 | 8.2 | 8.4 | 8.2 | 8.4 | 8.6 |

## FR1 30 kHz simulation results

In this section we provide alignment and impairment results for FR1 TDD scenarios with 30 kHz SCS. Figure 2 illustrates PDSCH throughput curves for different BW configurations and number of UE receive antennas.

| 2 Rx UE |  |
| --- | --- |
| 4 Rx UE |  |
| Figure 2. PDSCH simulation results for FR1 30 kHz. |  |

Summary of alignment simulation results for FR1 TDD scenarios with 30 kHz SCS is provided in Table 3.

Table 3. Alignment FR1 30 kHz simulation results

| Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 Rx | 10.7 | 10.8 | 10.8 | 11.0 | 10.9 | 10.8 | 11.2 | 11.5 | 11.3 | 11.8 | 11.6 | 12.1 |
| 4 Rx | 6.3 | 6.0 | 6.0 | 6.3 | 6.2 | 6.0 | 6.2 | 6.5 | 6.4 | 6.6 | 6.5 | 6.7 |

Summary of impairment simulation results for FR1 TDD scenarios with 30 kHz SCS is provided in Table 4.

Table 4. Impairment FR1 30 kHz simulation results

| Number of Rx Antenna | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 Rx | 12.7 | 12.8 | 12.8 | 13.0 | 12.9 | 12.8 | 13.2 | 13.5 | 13.3 | 13.8 | 13.6 | 14.1 |
| 4 Rx | 8.3 | 8.0 | 8.0 | 8.3 | 8.2 | 8.0 | 8.2 | 8.5 | 8.4 | 8.6 | 8.5 | 8.7 |

## FR2 simulation results

In this section we provide alignment and impairment results for FR2 scenarios with 120 kHz SCS. Figure 3 illustrates PDSCH throughput curves for different BW configurations and FRCs.

| Rank 1, MCS 13 | Rank 2, MCS 10 |
| --- | --- |
| Figure 3. PDSCH simulation results for FR2 120 kHz. |  |

Summary of alignment simulation results for FR2 scenarios with 120 kHz SCS is provided in Table 5.

Table 5. Alignment FR2 120 kHz simulation results

| FRC | 50 MHz | 100 MHz | 20 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| Rank 1, MCS 13 | 4.8 | 4.8 | 4.7 | 4.8 |
| Rank 2, MCS 10 | 8.1 | 7.9 | 8.0 | 8.1 |

# Conclusion

In this paper we provided alignment and impairment simulation results for FR1 Normal CA scenarios and alignment simulation results for FR2 Normal CA scenarios.

# References

- R4-1915863 “Simulation assumptions for NR normal CA UE performance requirements”, Intel, RAN4 #93, November 2019.