3GPP TSG-RAN WG4 Meeting #94-e		R4-2000370

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:       8.9.1.1

Source:                Intel Corporation

Title:                     Discussion on test feasibility and methodology for URLLC

Document for:     Discussion

# Introduction

In RAN4#93 the test methodology for testing high reliability metric of 99.999% was discussed and agreements were captured in [1]. The agreements related to the test methodology are summarized below

The two methods are optimizations of the existing RAN5 framework: - SINR set to target 10^-5 BLER. RAN5 test methodology adapted so that pass/fail decision is evaluated every N error reports instead of every error report - SINR set to target BLER much lower than 10^-5. RAN5 test methodology re-used with early pass expected. Potentially allow for early pass even with zero error reports, after sufficient sub-frames observed. - This kind of test is observing lack of error floor, not testing SNR vs BLER - Other optimizations not precluded as long as they are in line with the existing methodology. Baseline parameters for methodology 1: N (i.e. pass/fail criterion reported after observing N errors): [1, 10, 100] M (Bad device factor): 1.5 Baseline parameters for methodology 2: Targeted “real” BLER: [zero, 10^-7, 10^-6, 0,5*10^-5] M (Bad device factor): 1.5

In this contribution we present our evaluation and views on the test methodologies.

# Discussion

## Methodology 1

Methodology 1 is based on targeting testing at BLER of 10-5 with target overall confidence level of 99.999%. We present results from evaluation of the methodology. The baseline parameters used for evaluation are:

M (bad DUT factor): 1.5

N (number of errors observed for pass/fail): Shown in table

Method for deriving pass/fail limits: 34.121-1

DUTs considered: Limit and Marginal DUTs

Table 1: Testing time analysis for Method 1

| D (Per step error prob.) | N | Overall CL | Max number of errors | Max number of samples | Min number of samples | Testing time for 15 KHz SCS (hrs) |  | Testing time 30KHz SCS (hrs) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Max | Min | Max | Min |
| 8.00E-05 | 1 | 99.815% | 348 | 28191815 | 628899 | 7.8311 | 0.1747 | 3.9155 | 0.0873 |
|  | 10 | 99.959% |  |  | 1768220 |  | 0.4912 |  | 0.2456 |
|  | 20 | 99.978% |  |  | 2762107 |  | 0.7673 |  | 0.3836 |
|  | 50 | 99.994% |  |  | 5413468 |  | 1.5037 |  | 0.7519 |
|  | 100 | 99.999% |  |  | 9482332 |  | 2.6340 |  | 1.3170 |
| 4.00E-06 | 1 | 99.991% | 455 | 39461341 | 828615 | 10.9615 | 0.2302 | 5.4807 | 0.1151 |
|  | 10 | 99.997% |  |  | 2053690 |  | 0.5705 |  | 0.2852 |
|  | 20 | 99.999% |  |  | 3104613 |  | 0.8624 |  | 0.4312 |
| 4.00E-07 | 1 | 99.999% | 595 | 48217743 | 982121 | 13.3938 | 0.2728 | 6.6969 | 0.1364 |

Based on the results, our observations are that the following combinations can achieve overall confidence level of 5-9s. The maximum and minimum testing times for 15KHz SCS are summarized.

- With D=8e-5 and N=100

  - Max testing time: ~ 8hrs

  - Minimum testing time: ~2.6 hrs

- With D=4e-6 and N=20

  - Max testing time: ~11 hrs

  - Min testing time: < 1hr

- With D=4e-7 and N=1

  - Max testing time: ~13.4 hrs

  - Min testing time: ~20 minutes

By increasing the step size (number of errors observed per step) overall confidence level of 5-9s can be achieved with smaller per step error probability. The disadvantage of increasing the step size is that a minimum of N errors needs to be observed before determining pass/fail.

Observation #1: By increasing N, overall CL of 5-9s can be achieved with lower value of D and smaller maximum testing time, at the same time minimum testing time increases.

With smaller value of D for e.g., 4e-7, overall CL of 5-9s can be achieved with N=1. While the maximum testing time is high with small D, the minimum testing time is also greatly reduced.

Observation #2: Overall CL of 5-9s can be achieved with very small D (4e-7) and testing at every error observed.

Methodology 1 tests for SNR vs BLER performance. It is critical that performance requirements are introduced to test the high reliability BLER target of 1e-5. The concern raised in previous meeting on defining test cases with 1e-5 and high confidence level is the lengthy testing time. Based on the analysis shown in Table 1, it can be seen that the maximum testing time is rather long, but that is the worst case. Good DUTs should be able to complete the test in a much shorter time using early pass criteria. Moreover, while defining SNR vs BLER performance tests, a margin is added to the requirement such that in most cases the BLER at the requirement SNR is lower than the BLER tested for. In the next section we provide some analysis on average test time when the BLER tested is lower than 1e-5.

Observation #3: The BLER achieved at SNR specified in performance tests is usually lower than requirement

Another way to decrease the testing time is to introduce tests with 30KHz SCS rather than 15KHz even for FDD mode. The testing time in reduces by half with 30KHz SCS and can reduce the test time drastically for high reliability target test cases.

Proposal #1: Introduce test cases with 30KHz SCS for TDD and FDD mode to reduce test time

Based on the analysis and observation, we believe that it is feasible to introduce performance test at 1e-5 BLER and the parameters for statistical testing can be chosen to achieve confidence level of 99.999%. Hence, we recommend introducing performance tests with target BLER of 1e-5.

Proposal #2: Introduce performance requirements for testing 10-5 BLER for URLLC

For the parameters to achieve 99.999% overall confidence level, we recommend using D=4e-7 and N=1. While the maximum testing time is long with this combination, we believe that the actual test time will be much shorter for good DUTs.

Proposal #3: To achieve overall confidence level of 99.999% define statistical testing methodology based on D=4e-7 and N=1

## Methodology 2

The 2nd methodology a.k.a. boosted SNR method to test at a SINR lower than the SINR required to achieve 1e-5 BLER such that early pass can be declared based on smaller number of observed errors. In order to understand the number of samples required to complete the test, we simulate DUTs at different BLER levels to model performance of good and bad DUTs. The pass/fail limits are derived based on methodology in TS 34.121-1 for the combinations of D and N that can achieve overall CL of 5-9s.

Table 2: Simulation based testing time for CL of 99.999%

|  | Target BLER | Testing time for 15 KHz SCS (hrs) |  |  | Testing time for 30 KHz SCS (hrs) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min |
| D=8E-05 N=100 | 0.001 | 1.8495 | 1.8495 | 1.8495 | 0.9247 | 0.9247 | 0.9247 |
|  | 0.0001 | 1.8495 | 1.8495 | 1.8495 | 0.9247 | 0.9247 | 0.9247 |
|  | 1.00E-05 | 3.1755 | 6.8489 | 2.6340 | 1.5878 | 3.4245 | 1.3170 |
|  | 5.00E-06 | 2.6340 | 2.6340 | 2.6340 | 1.3170 | 1.3170 | 1.3170 |
|  | 1.00E-06 | 2.6340 | 2.6340 | 2.6340 | 1.3170 | 1.3170 | 1.3170 |
|  | 1.00E-07 | 2.6340 | 2.6340 | 2.6340 | 1.3170 | 1.3170 | 1.3170 |
| D=4E-06 N=20 | 0.001 | 0.1637 | 0.1637 | 0.1637 | 0.0819 | 0.0819 | 0.0819 |
|  | 0.0001 | 0.1637 | 0.1637 | 0.1637 | 0.0819 | 0.0819 | 0.0819 |
|  | 1.00E-05 | 3.0746 | 6.6874 | 0.8624 | 1.5373 | 3.3437 | 0.4312 |
|  | 5.00E-06 | 0.9057 | 1.8719 | 0.8624 | 0.4529 | 0.9359 | 0.4312 |
|  | 1.00E-06 | 0.8624 | 0.8624 | 0.8624 | 0.4312 | 0.4312 | 0.4312 |
|  | 1.00E-07 | 0.8624 | 0.8624 | 0.8624 | 0.4312 | 0.4312 | 0.4312 |
| D=4E-07 N=1 | 0.001 | 0.0018 | 0.0039 | 1 sample | 0.0009 | 0.0019 | 1 sample |
|  | 0.0001 | 0.0302 | 0.1028 | 0.0039 | 0.0151 | 0.0514 | 0.0019 |
|  | 1.00E-05 | 3.2798 | 6.4780 | 0.2728 | 1.6399 | 3.2390 | 0.1364 |
|  | 5.00E-06 | 0.6872 | 1.6467 | 0.2728 | 0.3436 | 0.8233 | 0.1364 |
|  | 1.00E-06 | 0.2968 | 0.4914 | 0.2728 | 0.1484 | 0.2457 | 0.1364 |
|  | 1.00E-07 | 0.2731 | 0.3270 | 0.2728 | 0.1365 | 0.1635 | 0.1364 |

From the results shown in Table 2 we observe that for target BLER lower than 1e-5, the testing time is much lower based on early pass criteria. This suggests that the SNR boost method would give indeed results in overall lower testing time.

Observation #4: The testing time for target BLER < 1e-5 is shorter than target BLER of 1e-5

We also observe that the average testing time for 1e-5 BLER is reasonable and the max testing time derived from simulation is smaller than that calculated using statistical method in Table 1.

Observation #5: The average testing time for 1e-5 BLER is reasonable and max testing time is smaller than that derived from statistical method

The boosted SNR method would indeed lower the testing time for high reliability target of 5-9s, but at the same time it is a test for DUT error floor and can be categorized as a functional rather than performance test.

Observation #6: Method 2 would test for error floor rather than performance and would be a functional test.

With the boosted SNR method, one unknown is how much the SNR should be boosted as to help a good DUT pass with early pass criteria while we don’t let a DUT not meeting the performance requirement also pass. While it would be useful to have a functional test for high reliability BLER target for URLLC, it would not be sufficient to guarantee the performance. Hence, we propose to introduce tests with boosted SNR method as a functional test but not as a substitute to performance test.

Proposal #4: Introduce test cases with boosted SNR method as a functional test if necessary, but not as a substitute to performance test

# Conclusion

In this paper we present our evaluation and views on the test methodologies. Our observations and proposals are summarized below:

Methodology 1:

Observation #1: By increasing N, overall CL of 5-9s can be achieved with lower value of D and smaller maximum testing time, at the same time minimum testing time increases.

Observation #2: Overall CL of 5-9s can be achieved with very small D (4e-7) and testing at every error observed.

Observation #3: The BLER achieved at SNR specified in performance tests is usually lower than requirement

Proposal #1: Introduce test cases with 30KHz SCS for TDD and FDD mode to reduce test time

Proposal #2: Introduce performance requirements for testing 10-5 BLER for URLLC

Proposal #3: To achieve overall confidence level of 99.999% define statistical testing methodology based on D=4e-7 and N=1

Methodology 2:

Observation #4: The testing time for target BLER < 1e-5 is shorter than target BLER of 1e-5

Observation #5: The average testing time for 1e-5 BLER is reasonable and max testing time is smaller than that derived from statistical method

Observation #6: Method 2 would test for error floor rather than performance and would be a functional test.

Proposal #4: Introduce test cases with boosted SNR method as a functional test if necessary, but not as a substitute to performance test

# Reference

- R4-1915866, “Ad-hoc minutes for NR URLLC test feasibility”, Ericsson