3GPP TSG-RAN WG4 Meeting #94-e	R4-2000379

Electronic Meeting, 24 Feb. – 6 Mar. 2020

Source: 	Intel Corporation

Title: 					Discussion on conditional PSCell addition/change

Agenda Item: 			8.3.2.3

Document for:	Discussion

1	Introduction

Conditional PSCell addition/change has been discussed in RAN4 for several meetings but with limited progress. The related objectives were somehow updated in the last RAN plenary. In this contribution, we provide discussion on how to define RRM requirement for this topic according to the last WID [1].

2	Discussion

First of all, we would like to clarify the related scope. Here we duplicate the related objectives from [1]:

- To specify the following solutions agreed during the study phase. [RAN2/RAN1/RAN3/RAN4]: - To reduce interruption time during HO: - Dual active protocol stack based HO interruption time reduction solution (high priority); - To improve HO/SCG change reliability and robustness: - Conditional handover for NR PCell change (high priority); - Conditional handover based NR PSCell addition/change for any architecture option with NR PSCell- limit to intra SN change without MN involvement; - T312 based fast failure recovery for PCell and PSCell (similar to LTE) (high priority)

As can be observed, we are supposed to work on RRM requirement for NR PCell change – limit to intra SN change without MN involvement. This implies conditional PSCell addition is not considered in this work item. Thus no need to define any RRM requirement in RAN4.

Observation 1: RAN4 doesn’t need to define RRM requirement for conditional PSCell addition.

Regarding impact on existing specification, we only need to capture conditional NR PSCell change in TS38.133, even for EN-DC operation. That aligns with existing specification structure, i.e. requirement for PSCell change in EN-DC is only specified in TS38.133:

8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other SCell in EN- DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. The requirements for PSCell Addition delay in clause 8.9.2 shall apply.

Proposal 1: conditional NR PSCell change RRM requirements are defined in TS38.133, covering both EN-DC and NR-DC.

Then we can come to the detailed requirement. Although there is no official agreement in RAN4, it is common understanding that the whole procedure is very similar with conditional handover from RAN4 RRM perspective. Therefore we propose to derive requirement based on the existing agreements which were made in conditional handover session.

According to [2], the conditional handover delay has been agreed as:

Handover delay is no more than: DCHO = TRRC_1 + Tmeasure + TRRC_2 + Tinterrupt Where: - TRRC_1 is defined the same as existing processing time for RRC reconfiguration message containing CHO configuration - Interruption during CHO: Tinterrupt = TIU + Tprocessing+ T∆ ms Where TIU , Tprocessing and T∆ are defined same as legacy handover. - TRRC_2: Second segment of RRC processing time, starts after UE realizes the condition is met and identity of target cell is determined. During TRRC_2 UE is allowed to finish associated operations earlier and leave the source cell. - Tmeasure is specified by: - reusing event triggered reporting requirements. (i.e. referring to corresponding requirement in TS38.133 clause 9)

Based on above agreements and the existing PSCell change delay requirement in TS38.133, the conditional PSCell change delay can be formulated as follow, by simply replacing TRRC_delay with TRRC_1 + Tmeasure + TRRC_2:

TCPC = TRRC_1 + Tmeasure + TRRC_2 + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

Where:

TRRC_1 is defined the same as existing processing time for RRC reconfiguration message containing CPC configuration

Tmeasure is specified by reusing event triggered reporting requirements. (i.e. referring to corresponding requirement in TS38.133 clause 9)

TRRC_2 is the second segment of RRC processing time, starts after UE realizes the condition is met and identity of target cell is determined.

The rest parts remain the same as existing:

Tprocessing is the SW processing time needed by UE, including RF warm up period. T processing = 40 ms.

Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, T search = 0 ms. If the target

cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T ∆ = 1*Trs ms for a

known or unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up

to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated

period is defined in Table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the

target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the

same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement

object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB

transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

Proposal 2: conditional PSCell change delay is defined as:

TCPC = TRRC_1 + Tmeasure + TRRC_2 + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

3	Conclusion

In this contribution we provide further discussion on conditional PSCell change. After discussion, the following conclusions are provided:

Observation 1: RAN4 doesn’t need to define RRM requirement for conditional PSCell addition.

Proposal 1: conditional NR PSCell change RRM requirements are defined in TS38.133, covering both EN-DC and NR-DC.

Proposal 2: conditional PSCell change delay is defined as:

4	References

- RP-192534, New WID: NR mobility enhancements, Intel

- R4-1913436, Way forward on NR mobility enhancement, Intel