3GPP TSG-RAN WG4 Meeting #94-e	R4-2000381

Electronic Meeting, 24 Feb. – 6 Mar. 2020

Source: 	Intel Corporation

Title: 					RRM impact on inter-band CA in FR2

Agenda Item: 			8.15.1.10

Document for:	Discussion

1	Introduction

According to the latest WID in [1], RAN4 needs to discuss and specify RRM requirement for inter-band CA requirement for FR2 UE measurement capability of independent Rx beam and/or common beam:

- Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam and/or common beam - Cell detection/measurement requirement, - Beam management requirements, - Scell activation requirement, - Interruption requirement, - Scheduling restriction requirement Note: applies for inter-band CA for 28 GHz + 39 GHz combinations according to RF conclusions in R4-1916024.

In this contribution, we will discuss the potential RRM impact based on the latest agreements in RF session. After discussion some proposals are provided.

2	Discussion

Beam management related agreement in [1] is duplicated here for information:

- UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations. - “28GHz” stands for a band group includes n257, n258, n261 - “39GHz” stands for a band group includes n259, n260 - Beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 28 GHz or 39 GHz + 39 GHz combinations. - Alt 1: UE is assumed to have common beam management - Alt 2: UE is assumed to have independent beam management

As can be observed, RF session confirmed that UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations. However, inter-band CA for 28 GHz + 28 GHz or 39 GHz + 39 GHz combinations are still open. Thus we only focus on 28 GHz + 39 GHz combinations in our discussion. In this section we will discuss the potential RRM impact according to the scope in the corresponding objective in the WID.

- Cell detection/measurement requirement

The existing cell detection/measurement requirement can be found in TS38.133 section 4/5 for idle/inactive mode mobility and in section 9 for connected mode mobility. For idle/inactive mode, we don’t see the necessity to push UE to perform cell search/measurement on 28GHz and 39GHz simultaneously, although UE can support independent beam management. Actually, this is not really related to whether UE can support independent beam management. For instance, in FR1 we never force UE to perform cell search/measurement on multiple frequency layers simultaneously. Thus no need to update corresponding requirement in idle/inactive mode.

Observation 1: existing RRM requirements in idle/inactive mode still apply for inter-band CA in FR2 for UE which supports independent beam management.

Proposal 1: there is no impact on idle/inactive mode requirement.

Regarding cell search/measurement in connected mode, RRM core requirements in FR2 are derived by multiplying the number of samples in FR1 by a scaling factor N, where N is number of beams. N is introduced by assuming UE needs to perform Rx beam sweeping on the SMTC such that UE can keep tracking neighbor cells nearby. Supporting independent beam management for inter-band CA means beam management on each band are separately maintained. For instance, perform Rx beam sweeping in 28GHz will not impact the procedure on 39GHz. More specifically, UE can still perform RLM/BFD/L1-RSRP measurement or even data reception on 39GHz while UE is doing beam sweeping on 28GHz. However, it won’t facilitate the cell search/measurement procedure on neither 28GHz nor 39GHz, since the desired number of samples doesn’t change and we assume the number of beams doesn’t change. Thus no need to update inter-frequency cell search and measurement requirement in TS38.133 section 9.3.

Proposal 2: there is no impact on inter-frequency cell search and measurement requirement in TS38.133 section 9.3.

However, there is indeed some impact on the CSSF table. Currently only intra-band CA is mentioned in section 9.1.5. For example:

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. |  |  |  |  |  |

CSSF for FR2 inter-band CA needs to be discussed and specified.

Proposal 3: CSSF for FR2 inter-band CA needs to be discussed and specified.

- Beam management requirements

In Rel-15 UE only performs BFD/CBD on PCell. In R16 eMIMO RAN4 is discussing beam management on SCell. Some agreements were made in the last RAN4 #93 meeting, which can be found in [2]. We assume requirements for beam management on SCell will be finalized in Rel-16. Based on that we can discuss if there is any impact from inter-band CA for UE which support independent beam management. Take BFD for example, the latest agreement in [2] is duplicated here for information:

- RRM core requirement impact by BFD on SCell - The evaluation periods for SSB/CSI-RS based BFD on SCell are same as those of SSB/CSI-RS based BFD on PCell/PSCell. Detailed requirement captured in Section 8.5.3/4 shall be reused for SCell BFD. - FFS whether to introduce a sharing factor for BFD and CBD measurement period

Most likely we will have very similar requirement for SCell compared to the existing requirement on PCell. Similar with discussion above in the cell search/measurement session, a UE which support independent beam management can perform BFD on an SCell on 39GHz without any impact on the others serving cells on 28GHz. UE can do beam management/RLM/L3 measurement or even data scheduling on 28GHz. However, it won’t facilitate or delay the beam management procedure on 39GHz. Therefore, most likely we will have generic RRM requirement for beam management to cover both intra-band and inter-band CA. Note that corresponding requirement is discussing and will be introduced in R16 eMIMO work item.

Observation 2: most likely beam management requirement being discussed in R16 eMIMO work item can also apply for inter-band CA.

- SCell activation requirement

Currently SCell activation RRM requirements in FR2 in TS38.133 are split into two categories: first cell on that FR2 band or not the first active serving cell. SCell activation delay for the first category is longer compared to the second category. If it is the first cell on that FR2 band, UE needs time to search for the timing, frequency and beam information, which contribute the most to the total delay. However, if there is at least one active serving cell on that FR2 band, then UE can skip the above procedures since we assumed in Rel-15 only collocated intra-band CA is supported, which mean the timing, frequency and beam information can be acquired based on the active cell on the same band. Regarding inter-band CA in FR2, supporting independent beam management actually doesn’t facilitate or delay the SCell activation procedure. The reason is similar with above. The time for cell search, AGC, beam training doesn’t change.

Observation 3: supporting independent beam management for inter-band CA has no impact on the SCell activation RRM requirement.

- Interruption requirement

Existing interruption requirements in TS38.133 cover several procedures, e.g. SCell addition/release, SCell activation/deactivation, active BWP switching and etc. Note that all of these requirements are quite irrelevant to independent beam management. On the other hand, interruption requirements for inter-band CA in FR2 have already been captured in our specification. We don’t see any impact so far.

Observation 4: interruption requirement for inter-band CA in FR2 are already there in TS38.133.

Proposal 4: no impact on interruption RRM requirements.

- Scheduling restriction requirement

Scheduling availability in FR2 can be found in several sections in TS38.133, covering RLM, BFD, CBD, L1/L3-measurement. The key idea to allow UE to drop data transmission/reception once the UE has to perform Rx sweeping. Similarly, we also have measurement restriction specified in TS38.133 to address the collision of downlink reference signals, on one of which UE is expected to perform Rx beam sweeping. For instance, if the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, RAN4 requirements say that UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

As mentioned above, independent beam management in inter-band CA in FR2 allows UE to handle Tx/Rx beams separately on different bands. Consequently, new scheduling availability and measurement restriction shall be introduced to allow, e.g., UE to keep data Rx/Tx or measurement on one band while it is doing beam sweeping on the other band.

Proposal 5: new scheduling availability and measurement restriction needs to be specified.

3	Conclusion

In this contribution, we discussed the RRM impact of inter-band CA in FR2 with independent beam management supported. After discussion, the following conclusions are provided:

Observation 1: existing RRM requirements in idle/inactive mode still apply for inter-band CA in FR2 for UE which supports independent beam management.

Proposal 1: there is no impact on idle/inactive mode requirement.

Proposal 2: there is no impact on inter-frequency cell search and measurement requirement in TS38.133 section 9.3.

Proposal 3: CSSF for FR2 inter-band CA needs to be discussed and specified.

Observation 2: most likely beam management requirement being discussed in R16 eMIMO work item can also apply for inter-band CA.

Observation 3: supporting independent beam management for inter-band CA has no impact on the SCell activation RRM requirement.

Observation 4: interruption requirement for inter-band CA in FR2 are already there in TS38.133.

Proposal 4: no impact on interruption RRM requirements.

Proposal 5: new scheduling availability and measurement restriction needs to be specified.

4	References

- RP-193211, Revised WID: NR RRM enhancement in R16, Intel

- R4-1915850, Way forward on NR eMIMO RRM, Samsung