3GPP TSG-RAN WG4 Meeting #94-e	R4-2000408

Electronic meeting, 24th February – 6th March 2020

Source:	Ericsson

Title:	Simulation results for PRACH HST with 500km/hour

Agenda item:	8.17.2.2.2

Document for:	Discussion

# Introduction

During the meeting RAN4#93, certain agreements [1] on the PRACH high speed train scenario with velocity at 500km/h were reached, where part of them are listed here.

- PRACH format

  - For 500km/h velocity, use PRACH format A2/B4/C2

  - For 500km/h velocity, no extra requirements for PRACH format 0

    - Common understanding: PRACH format 0 with 2334Hz also shows UE with 500km/h at 1.9GHz can be supported

- Frequency offsets under AWGN for PRACH formats with short sequence length targeting 500km/h

  - Align with PUSCH maximum Doppler shift

As suggested from [1], the frequency offsets for 500km/h are aligned with the maximum Doppler shift of PUSCH, which are captured in [2] and listed below.

- Maximum Doppler shift

  - Single tap HST 500km/h

    - 15kHz SCS: 1740Hz

    - 30kHz SCS: 3334Hz

Moreover, the test preamble parameters are provided as well.

- Short sequence format

  - 15 KHz

    - {Ncs, logical sequence index, V} = {23, 0, 0}

  - 30 KHz

    - {Ncs, logical sequence index, V} = {46, 0, 0}

- Timing error tolerance for AWGN and TDLC 300-100

  - Reuse the time error tolerance for corresponding NR PRACH normal performance requirements, i.e.

| Format | SCS | AWGN | TDLC300-100 |
| --- | --- | --- | --- |
| 0 | 1.25 | 1.04us | 2.55us |
| A2,B4,C2 | 15 | 0.52us | 2.03us |
|  | 30 | 0.26us | 1.77us |

Some other assumptions agreed in [3] are provided as below,

- Number of receiver antennas: 2 Rx, 4 Rx, 8 Rx

- Test metric

  - False alarm probability: 0.1%

  - Missed detection probability: 1%

In this contribution, we will provide the simulation results for PRACH high speed train case when the velocity is 500km/h.

# Discussion

Apart from the above simulation parameters, some other test parameter assumptions are provided as below,

- Carrier frequency for velocity at 500km/h

  - 2.1GHz (SCS=15kHz)

  - 3.6GHz (SCS=30kHz)

- Total number of PRACH preambles: 64

- Number of transmitter antennas: 1 Tx

The simulation results are included in the following tables.

Table 1. ideal simulation results for 2Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -13,8 | -16,4 | -13,1 |
|  | 3334 | 30 | -13,7 | -16,4 | -13,0 |
| TDL-C 300-100 | 400 | 15 | -6,8 | -8,9 | -6,8 |
|  |  | 30 | -7,2 | -8,7 | -7,3 |

Table 2. ideal simulation results for 4Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -16,1 | -18,7 | -15,5 |
|  | 3334 | 30 | -16,1 | -18,6 | -15,5 |
| TDL-C 300-100 | 400 | 15 | -11,5 | -13,4 | -11,6 |
|  |  | 30 | -11,4 | -12,4 | -11,5 |

Table 3. ideal simulation results for 8Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -18,3 | -20,9 | -17,6 |
|  | 3334 | 30 | -18,2 | -20,8 | -17,6 |
| TDL-C 300-100 | 400 | 15 | -15,2 | -17,1 | -15,3 |
|  |  | 30 | -14,4 | -15,3 | -14,6 |

Table 4. impaired results for 2Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -11,4 | -14,1 | -10,6 |
|  | 3334 | 30 | -11,2 | -14,4 | -10,5 |
| TDL-C 300-100 | 400 | 15 | -4,7 | -6,8 | -4,7 |
|  |  | 30 | -4,7 | -6,4 | -4,8 |

Table 5. impaired results for 4Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -13,6 | -16,5 | -13,2 |
|  | 3334 | 30 | -13,9 | -16,1 | -13,5 |
| TDL-C 300-100 | 400 | 15 | -9,1 | -11,0 | -9,2 |
|  |  | 30 | -9,3 | -9,9 | -9,0 |

Table 6. impaired results for 8Rx PRACH format A2/B4/C2

| Channel | Frequency offset (Hz) | SCS (kHz) | SNR (dB) for A2 | SNR (dB) for B4 | SNR (dB) for C2 |
| --- | --- | --- | --- | --- | --- |
| AWGN | 1740 | 15 | -16,0 | -18,6 | -15,5 |
|  | 3334 | 30 | -15,8 | -18,7 | -15,6 |
| TDL-C 300-100 | 400 | 15 | -12,8 | -14,7 | -13,3 |
|  |  | 30 | -12,2 | -13,3 | -12,2 |

# Conclusion

Simulation results for PRACH HST scenario with format A2/B4/C2, when velocity is 500km/h, are provided in Section 2.

# References

| [1] | Samsung, R4-1915914, Way forward on NR HST PRACH demodulation requirement, Reno, US, November 18-22, 2019. |
| --- | --- |
| [2] | Huawei, R4-1915886, Way forward on NR HST PUSCH demodulation requirements, Reno, US, November 18-22, 2019. |
| [3] | Huawei, R4-1910126, Way forward on NR HST BS demodulation requirements, Ljubljana, Slovenia, August 26-30, 2019. |