3GPP TSG-RAN4 Meeting #94-e	R4-2000412

# Online, , 24th Feb 2020 - 6th Mar 2020

Source:	Sprint

Title:	n41 and n90 device compatibility

Agenda item:	9.27.1

Document for:	Approval

# 1	Introduction

At RAN4#93 ZTE proposed removing the SCS based channel raster from n90 [1]. The proposal and CRs were discussed and noted.

This contribution looks at the issue from the perspective of device compatibility for open market devices and roaming.

# 2	Discussion

In R4-1913239 [1] several observations where noted including:

Observation 1: SCS based channel raster is unnecessary for n90 under dynamic spectrum sharing

This is true. For DSS with LTE Band 41, the 100 kHz raster will need to be used.

In addition to this observation, we would like to make some additional observations:

Observation 2: For DSS with LTE Band 41, n90 will need to use the 7.5 kHz UL shift and 100 kHz raster.

When LTE/NR DSS is not needed, there is no advantage of using the 7.5 kHz UL uplink shift and 100 kHz raster. However, there is a serious disadvantage of using the 7.5 kHz UL shift or the 100 kHz raster when LTE/NR DSS is not needed: UEs that support n41 but not n90 will not be compatible with the 7.5 kHz UL shift or the 100 kHz raster, so they will be incompatible with such a network.

Observation 3: When DSS between NR and LTE Band 41 is not used in a network, the 7.5 kHz UL shift and 100 kHz raster would provide no benefit for an NR network, and would cause n41 UEs that do not support n90 to be incompatible with the network.

In addition, when n90 was created, it was the intention that all n90 UEs also support n41. This did not get reflected clearly in 38.101-1, which only says “a UE supporting Band n90 shall meet the requirements for Band n41.”

Observation 4: It is not explicitly stated in 38.101-1 that UEs that support n90 shall also support n41.

The reason Sprint opposed the creation of n90 and preferred to add the 100 kHz raster and 7.5 kHz UL shift to n41 was to avoid fragmenting the ecosystem. Our understanding when n90 was created was that n90 UEs would also support n41. We think that this needs to be explicitly stated in the requirements, otherwise we risk fragmenting the ecosystem.

Proposal 1: Add an explicit requirement in 38.101-1 that UEs that support n90 also shall support n41.

Since not all n41 UEs will support n90, if a network does not need the 7.5 kHz UL shift and 100 kHz raster, then it should not use them to ensure compatibility with the maximum number of UEs. n41 UEs likely will be a superset of n90 UEs, especially if Proposal 1 is approved. So, whenever DSS is not needed, the network should be configured without the 7.5 kHz UL shift and 100 kHz raster.

With the current definition of n90, it is possible for a network that is configured without the 7.5 kHz UL shift and compatible with the SCS based raster to be configured as an n90 network in SIB1, which would make it incompatible with n41 UEs that do not support n90. While it is possible that the network without the 7.5 kHz UL shift and compatible with the SCS based raster could be configured with the MFBI indicating n41 to allow for compatibility with n41 UEs, it is not mandatory that a network do so. In order to maximize compatibility with the most UEs, networks that do not need the 7.5 kHz UL shift and 100 kHz raster should declare themselves to be n41 networks. There is no reason to allow them to declare themselves to be n90 networks, given that UEs that support n90 also support n41. In order to avoid this potential incompatibility, the n90 requirements should be modified so that only the 100 kHz raster and 7.5 kHz UL shift are supported.

Proposal 2: In order to maximize device compatibility and minimize ecosystem fragmentation, RAN4 should remove the SCS based rasters from n90, and mandate that n90 only be used with the 7.5 kHz UL shift.

# 3	Conclusions

Observation 1: SCS based channel raster is unnecessary for n90 under dynamic spectrum sharing

Observation 2: For DSS with LTE Band 41, n90 will need to use the 7.5 kHz UL shift and 100 kHz raster.

Observation 3: When DSS between NR and LTE Band 41 is not used in a network, the 7.5 kHz UL shift and 100 kHz raster would provide no benefit for an NR NR network, and would cause n41 UEs that do not support n90 to be incompatible with the network.

Observation 4: It is not explicitly stated in 38.101-1 that UEs that support n90 shall also support n41.

Proposal 1: Add an explicit requirement in 38.101-1 that UEs that support n90 also shall support n41.

Proposal 2: In order to maximize device compatibility and minimize ecosystem fragmentation, RAN4 should remove the SCS based rasters from n90, and mandate that n90 only be used with the 7.5 kHz UL shift.

# 4	References

- R4-1913239. “On channel raster for n90,” ZTE Wistron Telecom AB