3GPP TSG-RAN4 Meeting #94-e	R4-2000419

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0212 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-1: Missing 70 MHz for NS_01 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n77_n78_BW-Core |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 70 MHz is missing from the bandwidhts for NS_01 in Table 6.2.3.1-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds 70 MHz for NS_01 in Table 6.2.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 70 MHz will continue to be missing as a Channel bandwidth for NS_01 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 6.2.3.1-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2 In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5,10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
| NS_21 | 6.5.3.3.12 | n30 | 5, 10 |  | Clause 6.2.3.14 |
| NS_24 | 6.5.3.3.13 | n65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.3.15-1 | Clause 6.2.3.15 |
| NS_27 | 6.5.2.3.8 6.5.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.16-1 | Table 6.2.3.16-2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_44 | 6.5.3.2 | n38 | 40 | Table 6.2.3.20-1 | Table 6.2.3.20-1 |
| NS_45 | 6.5.3.3.16 | n39 | 25, 30, 40 |  | Clause 6.2.3.19 |
| NS_46 | 6.5.3.2 | n7 | 25, 30, 40, 50 | Table 6.2.3.17-1 | Table 6.2.3.17-2 |
| NS_47 | 6.5.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.18-1 | Table 6.2.3.18-2 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: No A-MPR is applied for 5 MHz CBW where the lower channel edge is ≥ 1930 MHz,10 MHz CBW where the lower channel edge is ≥ 1950 MHz and 15 MHz CBW where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n18 | NS_01 | NS_100 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_45 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n89 | NS_01 | NS_100 |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

<End of changes>