3GPP TSG-RAN4 #94-e	R4-2000436

Online, 24th Feb – 6th Mar 2020

Title:	Condition of IBB blocker location in FR2

Source:			Anritsu Corporation

Agenda Item:			6.5.8

Document for:			Approval

# 1.	Introduction

Taking into consideration of the previous offline discussions in RAN5 #85 in Reno, in this contribution we revisit a motivation of placing an interferer for in-band blocking (IBB) test repeatedly through a whole range in the corresponding FR2 band. The final goal in this paper is to confirm that a test condition regarding a location of blocker signal is enough only at the first non-adjacent channel position (FIoffset = +/- 2*Channel BW) in FR2.

# 2.	Discussion

2.1 Requirements of blocker position

The latest requirements of IBB for FR2 in TS 38.101-2 [1] is extracted at Appendix A in this paper. As can be seen in the table, requirements of FInterferer is defined to cover a whole range in the corresponding pass band, and the test procedure is also described in TS 38.521-2 [2] to repeat placing the interferer signal throughout the in-band range.

Looking back on the previous chairman notes while the blocking requirements were discussed (e.g. [3][4]), there is only a minute that we received a comment which preferred to follow the same principle with the E-UTRA requirements. But we assume that the original motivation to place the blocker throughout the pass band is to confirm spurious responses within a UE. Figure 2.1-1 and 2.1-2 depict the major factors of spurious responses, i.e. image response and Half-IF spurious response. Here fIF represents an intermediate frequency, and fLO is a local oscillator in a UE.

Observation 1: Original motivation to place the blocker throughout the pass band is to confirm spurious responses within a UE. (e.g. Image response and Half-IF spurious response.)

However if we look into a design of the current mmWave UE frontend architecture, we can find that there is not such a case or the influence is negligible. We explain the mechanism from the next sub-clause.

Down conversion

Down conversion

Wanted signal

Interferer

LO

fIF

fLO-fIF

fLO+fIF

fLO

Image of interferer

Wanted signal

Figure 2.1-1: Image response

Down conversion

Down conversion

Wanted signal

Interferer

LO

fIF/2

fLO+fIF/2

fLO+fIF

fLO

Interferer

Wanted signal

fIF

2nd harmonic of interferer

Figure 2.1-2: Half-IF spurious response

2.2 Consideration on the mechanism of spurious responses

Figure 2.2-1 depicts an example of a design with the mmWave UE frontend architecture. Here we assume that the current UE is down converting FR2 signals to a baseband frequency by two intermediate steps.

DPX

LNA

Rx

HPA

Tx

10 GHz 
1st IF

ADC

BPF

1 GHz 
2nd IF

LO1

LO2

Mixer1

Mixer2

Figure 2.2-1: Example of FR2 NR UE Rx frontend architecture

For the image response, from a relationship between the frequency of wanted signal (around 28 GHz or 39 GHz), 1st IF (approx. 10 GHz) and LO (approx. 18 GHz or 29 GHz), a gap between wanted signal and LO is around 10 GHz. And that means only in a case the interferer is somewhere around 10 GHz away from the LO, an image of the interferer overlaps the wanted signal, which is already an out of band blocking test. To overlap the interferer on the wanted signal within the same band, considering the current FR2 bandwidth (4 GHz max with n259), 1st IF needs to be designed less than 2 GHz.

Observation 2: From the current design of the mmWave UE frontend architecture, an image of the interferer does not appear in a same FR2 band of the wanted signal.

Next with the half-IF spurious response, suppose the IF is around 10 GHz again, this influence matters with the case when the interferer is located in the middle of the LO and wanted signal frequency, i.e. 5 GHz away from wanted signal. Again this does not apply to FR2 IBB and even if with a case in which the UE is designed to have 1st IF at less than 8 GHz (which means the interferer may overlap the FR2 wanted signal), considering a typical down-conversion spurious suppression rate by a mixer, this spurious can be suppressed more than 60 dBc and thus we assume it is negligible.

Observation 3: The half-IF spurious response also does not appear in-band or can be assumed as negligible in FR2.

From the observation 2 and 3 above, we assume that the in-band blocking is severest when the blocker is located at the first non-adjacent channel position (FIoffset = +/- 2*Channel BW) and thus we propose to clarify it in TS 38.101-2. By this change we will be able to solve the issue raised by RAN5.

Proposal 1: Modify the requirement of IBB in TS 38.101-2 to place the in-band blocker only at the first non-adjacent channel position (FIoffset = +/- 2*Channel BW).

# 3. Conclusion

In this contribution we revisited the IBB requirement in FR2 to solve the testability issue raised by RAN5.

Observation 1: Original motivation to place the blocker throughout the pass band is to confirm spurious responses within a UE. (e.g. Image response and Half-IF spurious response.)

Observation 2: From the current design of the mmWave UE frontend architecture, an image of the interferer does not appear in a same FR2 band of the wanted signal.

Observation 3: The half-IF spurious response also does not appear in-band or can be assumed as negligible in FR2.

Proposal 1: Modify the requirement of IBB in TS 38.101-2 to place the in-band blocker only at the first non-adjacent channel position (FIoffset = +/- 2*Channel BW).

# 4.	References

[1] TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, V16.2.0, 2019-12

[2] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception;

Part 2: Range 2 Standalone”, V16.2.0, 2019-12

[3] R4-1708449, “On UE IBB requirement for mmW”, Qualcomm Inc., RAN4 #84, Berlin, Germany

[4] R4-1710101, “RAN4#84 Meeting Report”, MCC, RAN4 #84bis, Dubrovnik, Croatia

[5] R4-2000437, “Correction to in-band interferer offset definition in FR2”, Anritsu corp., RAN4 #94-e, Online

5. Appendix A: Extract of in band blocking requirements from TS 38.101-2 [1]

Table 7.6.2-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1. TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Clause 7.3.2, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: FIoffset is the frequency separation between the center of the channel bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |  |  |  |