3GPP TSG-RAN4  #94-e																     	R4-2000472

24th Feb – 6th Mar 2020

Source: 	Qualcomm Incorporated

Title: 	MPR, A-MPR results for PSSCH/PSCCH transmission

Agenda item:	8.4.2.1

Document for:	Approval

# Introduction

In [1] a way forward was agreed for doing PSSCH/PSCCH MPR/A-MPR simulations. This paper presents the results of these simulations and proposes MPR and AMPR back offs to be added to the TS38.101-1 specification.

# Discussion

The details of the assumptions used for these simulations are outlined in [3]. The simulation results for MPR are based on the system requirements agreed to in TR38.886 [4] and those for A-MPR are based on a 10M ETSI mask and 40M 5GAA mask submitted to the FCC both of which were agreed to by RAN4 [5,6]. Simulations were done for PSSCH/PSCCH bandwidths of 10MHz, 20MHz, 30MHz and 40MHz and SCS values of 15KHz, 30kHz and 60KHz and for sub-channel sizes and LCRB allocations as specified in [1]. For each bandwidth all possible sub-channel sizes were simulated. Based on these simulations the worst case MPR and AMPR values were found for each bandwidth and SCS. In this case the MPR and AMPR values are the back-off from the MPR0 output power.

The MRR back-off results for these simulations are presented in table 1.

|  | MPR(dB) |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | Bandwidth (MHz) |  |  |  |
|  | 10 | 20 | 30 | 40 |
| QPSK | 4.6 | 4.7 | 4.7 | 4.7 |
| 16QAM | 4.6 | 4.7 | 4.7 | 4.8 |
| 64QAM | 4.6 | 4.7 | 4.7 | 4.8 |
| 256QAM | 4.6 | 4.7 | 4.8 | 4.8 |
| Note: EVM was not considered in this analysis |  |  |  |  |

Table 1 – Max MPR for various modulations and SCS values

The ETSI 10M AMPR back-off values for various Gpostconnector gains are given in Table2

| GP Gain (dB) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 16 QAM | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 64 QAM | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 256 QAM | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| Note: EVM was not considered in this analysis |  |  |  |  |  |  |  |  |

Table 2 – ETSI 10M AMPR back-off for various Gpostconnector values

The 40M AMPR back-off values are given in table 3

|  | AMPR_40M (dB) |
| --- | --- |
| Modulation |  |
| QPSK | 8.5 |
| 16QAM | 8.3 |
| 64QAM | 9.0 |
| 256QAM | 8.3 |
| Note: EVM was not considered in this analysis |  |

Table 3 – 40M AMPR back-off for various modulations

Based on the above results the following proposals are made for MPR and AMPR as a function of BW and SCS:

|  | MPR BO (dB) |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | Bandwidth (MHz) |  |  |  |
|  | 10 | 20 | 30 | 40 |
| QPSK | 5.0 | 5.0 | 5.0 | 5.0 |
| 16QAM | 5.0 | 5.0 | 5.0 | 5.0 |
| 64QAM | 5.0 | 5.0 | 5.0 | 5.0 |
| 256QAM | 6.51 | 6.51 | 6.51 | 6.51 |
| - MPR required to meet EVM requirements |  |  |  |  |

Table 4 – Proposed MPR back-off table for various BW and SCS settings of 15K,30K and 60K

|  | ETSI 10M AMPR BO (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GP Gain (dB) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| QPSK | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 16 QAM | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 64 QAM | 3.51 | 4 | 4 | 4 | 5 | 5 | 5 | 6 |
| 256 QAM | 6.51 | 6.51 | 6.51 | 6.51 | 6.51 | 6.51 | 6.51 | 6.51 |
| - AMPR required to meet EVM requirements |  |  |  |  |  |  |  |  |

Table 5 – Proposed AMPR 10M back-off table vs Gpostconnector values for various BW and SCS settings

|  | AMPR_40M BO (dB) |
| --- | --- |
| Modulation |  |
| QPSK | 9.0 |
| 16QAM | 9.0 |
| 64QAM | 9.5 |
| 256QAM | 9.0 |

Table 6 – Proposed AMPR table for 40M emissions mask for various BW and SCS settings

Proposal 1: Use MPR back-off values given in table4 for multi-cluster PFSCH operation

Proposal 2: Use AMPR back-off values given in table5 for ESTI 10M emissions when NS33 is signaled for multi-cluster PFSCH operation

Proposal 3: Use AMPR values given in table6 for 40M emissions when FCC requirements are signaled for multi-cluster PFSCH operation

# Conclusion

Based on MPR and AMPR simulations done on the multi-cluster performance of the PFSCH the following proposals are made:

Proposal 1: Use MPR back-off values given in table4 for multi-cluster PFSCH operation

Proposal 2: Use AMPR back-off values given in table5 for ESTI 10M emissions when NS33 is signaled for multi-cluster PFSCH operation

Proposal 3: Use AMPR values given in table6 for 40M emissions when FCC requirements are signaled for multi-cluster PFSCH operation

# Reference

- Summary of e-mail discussion on NR V2X UE basic RF parameters and MPR assumptions, LG Electronics, RAN4#94-E, February 2020

- R4-1913967,”TR 38.886v0.4.0 V2X service based on NR, UE radio transmission and reception”, RAN#93, Reno USA, November 2019

- 5GAA Petition for Waiver to Allow Deployment of Cellular Vehicle-to-Everything (C-V2X) Technology in the 5.9 GHz Band; GN Docket No. 18-357.

https://ecfsapi.fcc.gov/file/104030451515194/5GAA%20Band%20Plan%20Ex%20Parte%20-%20FINAL.pdf

- R4-1915430, “V2X RF considerations”, RAN#93, Reno, USA, November 2019