3GPP TSG-RAN WG4 Meeting #94-e	R4-2000514

Online, 24 - 28 February, 2020

Source:	ZTE

Title:	Discussion on TCI state known status mismatch

Agenda item:	6.10.8.4

Document for:	Discussion

# 1	Introduction

- For the last 3 meetings, the issue of TCI state known status mismatch has been raised and discussed. In our view, most companies agreed that the problem of TCI state known condition mismatch at UE and gNB side exist, while the necessity for a solution in R16 is a bit controversial [1]. We provide our view in this paper and suggest a way to move forward.

2	TCI state known status mismatch

- In TS 38.133 [2], the TCI known status is defined as:

### 8.10.2 Known conditions for TCI state The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within [1280] ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

In the current definition, some conditions are only known by the UE, for instance the following:

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

- 	SNR of the TCI state ≥ -3dB

These conditions are not known to gNB, thus, when the gNB assumes that one TCI state is known to the UE, on the UE side the TCI state can be unknown according to the current definition.

Some conditions in the current definition of TCI known state can’t be known by the gNB.

The current definition of known TCI state is subject to different interpretations at gNB and UE side.

Based on the above observations, we think that the current definition in the specification needs some extent of clarification. Companies have provided their views on how this problem would be possibly solved in RAN4 #93 [3][4], somehow more discussions are needed to draw a final conclusion. There are also views from companies that the problem would potentially degrade system performance, but won’t break the system. Thus, a complicated solution might not be needed.

In our view, to correctly evaluate the impact of TCI state known status mismatch, companies should bring in detailed analysis and simulations. If analysis and/or simulations show that the overall system performance degrades significantly with this mismatch issue, then solutions are needed. Otherwise, since the system is not broken by this issue, no solution is needed.

Further study the impact of mismatch issue based on detailed analysis and simulations.

3	Conclusion

- Some conditions in the current definition of TCI known state can’t be known by the gNB.

The current definition of known TCI state is subject to different interpretations at gNB and UE side.

- Proposal 1: Further study the impact of mismatch issue based on detailed analysis and simulations.

References

- R4-1915937, WF on TCI state and spatial relation switch.

- 3GPP TS 38.133 NR; Requirements for support of radio resource management.

- R4-1911629

- R4-1911633