3GPP TSG-RAN4#94-e	R4-2000529

24th – 06th Mar 2020

Source: 	Nokia, US Cellular

Title: 					TP to TR 37.716-21-11: DC_(n)5AA

Agenda Item: 			9.4.2

Document for:	Approval

# Introduction

This is a TP into TR 37.716-21-11 to add DC_(n)5AA.

********************************** Start of TP ***********************************************

## 6.3	Intra-band contiguous EN-DC

### 6.3.X	DC_(n)5AA

#### 6.3.X.1	Channel bandwidths per operating band for DC

Table 6.3.X.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)5AA | - | 5, 10 | 5, 10, 15, 20 | 5, 10 | 15 | 0 |
| NOTE: Only single switched UL is supported |  |  |  |  |  |  |

#### 6.3.X.2	Configuration for DC

Table 6.3.X.2-1:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration | Single UL allowed |
| --- | --- | --- | --- | --- |
| DC_(n)5AA | - | 5A | n5A | Yes |
| NOTE: Only single switched UL is supported |  |  |  |  |

#### 6.3.X.3	Maximum output power for DC

Note that only single switched UL mode is supported for this combination hence no additional Maximum output power for DC requirement is needed.

#### 6.3.X.4	Spurious emission band UE co-existence for DC

Note that only single switched UL mode is supported for this combination hence no additional spurious emission band UE co-existence requirement is needed

#### 6.3.X.5	MSD analysis for DC

Note that only single switched UL mode is supported for this combination hence no MSD requirement is needed

#### 6.3.X.6	∆TIB and ∆RIB values

As DC_(n)5AA is single band thus relaxations are not needed

#### 6.3.X.7	MPR, AMPR requirements

Note that only single switched UL mode is supported for this combination hence no additional MPR or A-MPR requirement is needed.

********************************** End of TP ***********************************************