3GPP TSG-RAN WG4 Meeting #94-e	R4-2000560

Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.15.1.10

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on inter-band CA requirement for FR2

Document for:	Discussion

- Introduction

At the RAN #84 meeting, new WI for NR RRM enhancement in Rel-16 was approved and 6 new requirements were included [1]. In this contribution, we focus on “Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam and/or common beam” and provide our view of each item in the topic.

- Discussion

At the last RAN4 #93 meeting, the WF on FR2 inter-band downlink CA was discussed and approved that UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations [2]. According to this WF, it is roughly divided into two types of scenario: common beam scenario and different beam scenario. Additionally, the transmission point of each band is not always co-located. Therefore, the scenario is also divided into two types of scenario w.r.t transmission point: co-location and non co-location. Generally speaking, if the transmission point of each band is non co-located and UE has only common beam, UE may hardly receive signals transmitted from each transmission point simultaneously.

Observation 1: If the transmission point of each band is non co-located and UE has only common beam, UE may hardly receive signals transmitted from each transmission point simultaneously.

Proposal 1: The scenarios other than that UE has only common beam and transmission points are non co-located shall be prioritized.

Obviously, UE can simultaneously receive both signals from each transmission point simultaneously if they are co-located even if UE has only common beam. On the other hand, if UE can have independent beam, both co-location scenario and non co-location scenario are feasible and UE can simultaneously receive both signals from each transmission point. Based on above discussion, we only focus on the scenario that UE can simultaneously receive signals of each band in this contribution.

  - Cell detection/measurement requirement

Basically, intra/inter-frequency measurement requirements do not need to be changed, however CSSF should be clarified since this factor is related to simultaneous measurement among CCs. Based on the current requirements regarding CSSF, UE is assumed to have at least two cell searchers. Since we are now focusing on the UE who is capable of simultaneous reception in case of inter-band FR2 CA, the UE can measure each band parallelly by using two cell searchers if one PCell/PSCell and one SCell are assumed. If more than 3 CCs are configured, the method specifying CSSF requirements in Rel-15 is reused. More specifically, the additional value sets of CSSFoutside_gap,i shall be added in Table 9.1.5.1.1-1 and 9.1.5.1.2-1 as follows (existing values are ommitted).

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

Observation 2: UE can measure each band parallelly under the assumption that the UE is capable of simultaneous reception and has at least two cell searchers.

Proposal 2: Rel-15 Cell detection/measurement requirement shall be reused for FR2 inter-band CA. More specifically, the following additional value sets of CSSFoutside_gap,i shall be added in Table 9.1.5.1.1-1 and 9.1.5.1.2-1 of current spec.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

  - Beam management requirements

Beam management requirements is also same situation as that of cell detection/measurement requirements as mentioned in 2.1, i.e. if the UE can measure each band parallelly, Rel-15 requirements can be reused.

Proposal 3 Rel-15 beam management requirement for FR1+FR2 CA shall be reused for FR2 inter-band CA scenario as is.

  - SCell activation requirement

In the current specification, SCell activation requirements are specified in following cases [3]:

- The SCell is known and belongs to FR1

- The SCell is unknown and belongs to FR1

- The SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band

- The SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1.

Since this contribution focuses on FR2 inter-band CA, case 1 and 2 are irrelevant. In case 3, there is no specific condition for PCell or PSCell, therefore this requirement can be applicable to FR2 inter-band CA case. In case 4, the condition declares PCell or PSCell is FR1, however this assumption does not affect following delay requirement definition. Therefore, these exitsing requirements also can be applicable for FR2 inter-band CA case.

Observation 3: The current SCell activation requirements for FR2 SCell are specified regardless of PCell or PSCell frequency range.

Proposal 4: Rel-15 SCell activation requirements for FR2 SCell shall be reused for FR2 inter-band CA scenario as is.

  - Interruption requirement

In the current specification, interruption requirements are specified in following cases [3]:

- EN-DC Interruption

- SA Interruptions with Standalone NR Carrier Aggregation

- NE-DC Interruption

- NR-DC Interruption.

Each case contains several scenarios but basically it specifies interruption requirements for every CC according to each CC configuration. In case 1 and 3, interruption requirements are regardless of whether inter-band CA or intra-band CA of NR cell group. In case 2 and 4, basically the requirements are not related to the CA condition while interruptions at PSCell/SCell addition/release and SCell activation/deactivation are related to the CA condition. However since they do not specify the band combination, FR2 inter-band CA shall be included.

Observation 4: The current interruption requirement does not preclude the case of FR2 inter-band CA.

Proposal 5: Rel-15 interruption requirement shall be reused for FR2 inter-band CA scenario as is.

  - Scheduling restriction requirement

In the current specification, scheduling restriction requirements are specified in following cases [3]:

- Scheduling availability of UE during radio link monitoring

- Scheduling availability of UE during beam failure detection

- Scheduling availability of UE during candidate beam detection

- Scheduling availability of UE during intra-frequency measurements

- Scheduling availability of UE during L1-RSRP measurement

In case 1, RLM is performed only in PCell and/or PSCell, therefore the CA condition is irrelevant. In case 2, 3, and 5, if the condition which UE has to do Rx beam sweeping is met, there are scheduling restrictions. However, in this contribution, since the UE is assumed to have the capability of simultaneous measurement for each FR2 band, the scheduling restriction in each band does not affect other band. This means the existing requirement for FR1-FR2 inter-band CA and NR-DC can be reused. In case 4, since this discussion assumes that the UE has at least two cell searchers, intra-frequency measurement in each band can be done parallelly. Therefore scheduling in each band is not restricted by other band.

Observation 5: Based on the assumption that UE is assumed to have the capability of simultaneous measurement for each FR2 band, scheduling in each band is not restricted by other band.

Proposal 6: Rel-15 scheduling restriction requirement shall be reused for FR2 inter-band CA scenario as is.

- Conclusion

In this contribution, we proposed our views on inter-band CA requirement for FR2 UE measurement capability of independent Rx beam and/or common beam.

Observation 1: If the transmission point of each band is non co-located and UE has only common beam, UE may hardly receive signals transmitted from each transmission point simultaneously.

Observation 2: UE can measure each band parallelly under the assumption that the UE is capable of simultaneous reception and has at least two cell searchers.

Observation 3: The current SCell activation requirements for FR2 SCell are specified regardless of PCell or PSCell frequency range.

Observation 4: The current interruption requirement does not preclude the case of FR2 inter-band CA.

Observation 5: Based on the assumption that UE is assumed to have the capability of simultaneous measurement for each FR2 band, scheduling in each band is not restricted by other bandCCs.

Proposal 1: The scenarios other than that UE has only common beam and transmission points are non co-located shall be prioritized.

Proposal 2: Rel-15 Cell detection/measurement requirement shall be reused for FR2 inter-band CA. More specifically, the following additional value sets of CSSFoutside_gap,i shall be added in Table 9.1.5.1.1-1 and 9.1.5.1.2-1 of current spec.

Table 9.1.5.1.1-1: CSSFoutside_gap,i scaling factor for EN-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required Note 2 | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR2 inter-band CA | N/A | N/A | 1 | 2 | 2×(Number of configured SCell(s)-1) |

Proposal 3 Rel-15 beam management requirement for FR1+FR2 CA shall be reused for FR2 inter-band CA scenario as is.

Proposal 4: Rel-15 SCell activation requirements for FR2 SCell shall be reused for FR2 inter-band CA scenario as is.

Proposal 5: Rel-15 interruption requirement shall be reused for FR2 inter-band CA scenario as is.

Proposal 6: Rel-15 scheduling restriction requirement shall be reused for FR2 inter-band CA scenario as is.

References

[1] 3GPP, RP-191601, “WI of NR RRM enhancement in R16”.

[2] 3GPP, R4-1916024, Nokia, “WF on FR2 inter-band DL CA”.

[3] 3GPP, TS 38.133 V16.2.0 (2019-12), “Requirements for support of radio resource management (Release 16)”