3GPP TSG RAN WG4 Meeting #94-e	R4-2000564

Online, 24th February – 6th March 2020

Source: 		Rohde & Schwarz

Title: 		On DL channel signal power ratios and mapping for UE requirements

Agenda Item:		6.12.1

Document for:	Approval

# Introduction

[1] defines the required EPRE ratios between different channels/signals in FR1:

Table 1-1: Table C.3.1-1 from [1]

The static channel matrices H defined in [1, B.1] affect different channels and signals differently regarding their power, i.e. while the EPRE of e.g. PDSCH and SSS is equal at the matrix input it might be different at the matrix output.

As an example consider a 4 layer PDSCH transmission setup according to Figure 1-1.

Figure 1-1: Exemplary 4 layer transmission setup

with the 4 port transmission static channel matrix H given in [1, B.1.2]:

H=11jj11-j-j1-1j-j1-1-jj

Assume that PDSCH EPRE equals SSS EPRE at Ref. 1.

PDSCH will be present on all four antennas at Ref. 1, while SSS will be present on only one antenna at Ref. 1, if no beamforming is applied on SS-Block. The PDSCH signals on the 4 antennas at Ref. 1 are statistically independent, i.e. the PDSCH EPRE on an antenna at Ref. 2 will be 6 dB higher than the PDSCH EPRE on an antenna at Ref. 1. Since the SSS is only present on 1 antenna at Ref. 1 it will not experience a power gain, i.e. PDSCH EPRE and SSS EPRE on an antenna at Ref. 2 differ by 6 dB.

As a second example consider the same setup as in the previous example but now assume that SSS is present on all four antennas at Ref. 1 and still no SS-Block beamforming is applied, i.e. all four antennas at Ref. 1 carry the exact same SSS. While the PDSCH again experiences a power gain of 6 dB the SSS will now experience a power gain of 9 dB, because the H matrix adds up 4 fully correlated SSS. Thus PDSCH EPRE and SSS EPRE on an antenna at Ref. 2 differ by 3 dB.

As last example consider the same setup as in the first example but now assume that SSS is beamformed. While the PDSCH again experiences a power gain of 6 dB the SSS will now experience a power gain that depends on the beamforming vector, i.e. the difference of PDSCH EPRE and SSS EPRE on an antenna at Ref. 2 depends on the beamforming vector.

The previous examples show that it is not straightforward to ensure the EPRE ratios defined in Table 1-1 at Ref. 2.

What makes matters worse is that there are different possible ways to ensure them.

Regarding the first example one might attenuate the PDSCH on the antennas at Ref. 1 by 6 dB. This would ensure the same EPRE of SSS and PDSCH on the antennas at Ref. 2.

On the other hand one could apply an appropriate beamforming vector to SSS ensuring a 6 dB power gain of SSS (in conjunction with H). Then again the SSS EPRE and PDSCH EPRE would be equal on an antenna at Ref. 2.

A third possibility would be to send the same SSS on all four antennas and attenuate SSS by 3 dB with respect to PDSCH at Ref. 1. Then again the SSS EPRE and PDSCH EPRE would be equal on an antenna at Ref. 2.

Obviously these three approaches lead to three different testcases. One with beamformed SS-Block the other two without beamforming.

From that we conclude that the test specification is ambiguous which needs to be fixed.

# Discussion

We propose that table C.3.1-1 in [1] and corresponding tables that define EPRE ratios to be present at Ref. 2 are replaced by tables that define the EPREs of all the involved signals and channels at Ref. 1.

Moreover it needs to be specified on which antennas the SS-Block, i.e. PBCH, SSS and PSS shall be transmitted from BS in case that no beamforming is applied.

We propose that the SS-Block is mapped to one single physical antenna as long as no beamforming is applied.

This way it doesn’t experience a power gain from the channel matrix  H as long as no beamforming is applied. In terms of power gain between Ref. 1 and Ref. 2 CSI-RS behaves like a SS-Block (for which no beamforming is applied) because the CSI-RS signals on different antennas at Ref. 1 are orthogonal, i.e. separated in time and/or frequency. Hence CSI-RS EPRE won’t experience a power gain from H as well.

With an appropriate setting of the PDSCH-DMRS, PDSCH OCNG and CSI-RS EPREs w.r.t. SSS, which needs to be defined per testcase, the intended EPRE ratios at Ref. 2 can be achieved.

| Parameter | Unit | Value before applying channel matrix | Mapping to antenna before applying channel matrix |
| --- | --- | --- | --- |
| SSS transmit power | W | Test specific |  |
| EPRE ratio of PSS to SSS | dB | 0 | PBCH transmitted over Single antenna |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | PDCCH transmitted over same antenna as PBCH |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS | dB | Test specific | TC specific |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | Test specific (Note 2) | TC specific |
| EPRE ratio of PDSCH OCNG to SSS | dB | Test specific (Note 3) | TC specific |
| EPRE ratio of PDCCH OCNG to SSS | dB | 0 | Transmitted over same antenna as PBCH |
| NOTE 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on “Number of DM-RS CDM groups without data” and “DMRS Type” parameters specified for each test. NOTE 2: CSI-RS is not beamformed. Therefore in case of beamforming in general it will experience a gain from the channel matrix H which is different to the gain that a beamformed channel/signal will experience. NOTE 3: Since OCNG on different transmit antennas is always uncorrelated according to Annex A.5 it in general will experience a different power gain from the channel matrix H than PDSCH. This is because PDSCH on different transmit antennas might be correlated, e.g. in case of beamforming or Tx diversity. |  |  |  |

Table 2-1: Proposed table update

Important hint that has to be kept in mind:

If channels / signals on antennas at Ref. 1 are correlated and the static channel matrix H is replaced by a time varying fading matrix H(t) the power gains of H and H(t) are different in general.

For PDSCH we need to also consider the cases where the transmitted number of layers is smaller than the number of Tx antennas (e.g. 4x4 scenarios with two layers). In this case it also needs to be clarified on which antennas the PDSCH is transmitted. Since this varies for each requirement, this needs to be defined on a per requirement basis. We propose to introduce a precoding matrix for the transmit antennas to define the antennas which are used for transmission.

# Conclusion

As as conclusion from the discussion in this paper we make the following proposals to update the specification. A corresponding CR is provided in R4-2000565 [3].

Proposal 1: The SS-Block is mapped to one single physical antenna at Ref.1 as long as no beamforming is applied.

Proposal 2: Update the specification to include Table 3-1 with the relevant information for each requirement with regard to Ref. 1.

| Parameter | Unit | Value before applying channel matrix | Mapping to antenna before applying channel matrix |
| --- | --- | --- | --- |
| SSS transmit power | W | Test specific |  |
| EPRE ratio of PSS to SSS | dB | 0 | PBCH transmitted over Single antenna |
| EPRE ratio of PBCH to SSS | dB | 0 |  |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |  |
| EPRE ratio of PDCCH to SSS | dB | 0 | PDCCH transmitted over same antenna as PBCH |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS | dB | Test specific | TC specific |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | Test specific (Note 1) |  |
| EPRE ratio of CSI-RS to SSS | dB | Test specific (Note 2) | TC specific |
| EPRE ratio of PDSCH OCNG to SSS | dB | Test specific (Note 3) | TC specific |
| EPRE ratio of PDCCH OCNG to SSS | dB | 0 | Transmitted over same antenna as PBCH |
| NOTE 1: Value is derived from Table 4.1-1 in TS 38.214 [12] based on “Number of DM-RS CDM groups without data” and “DMRS Type” parameters specified for each test. NOTE 2: CSI-RS is not beamformed. Therefore in case of beamforming in general it will experience a gain from the channel matrix H which is different to the gain that a beamformed channel/signal will experience. NOTE 3: NOTE 3: Since OCNG on different transmit antennas is always uncorrelated according to Annex A.5 it in general will experience a different power gain from the channel matrix H than PDSCH. This is because PDSCH on different transmit antennas might be correlated, e.g. in case of beamforming or Tx diversity. |  |  |  |

Table 3-1: Table proposed from R&S

Proposal 3: Add precoding matrix for PDSCH to each requirement to clarify mapping of antenna ports to physical antennas.

# References

[1] TS 38.101-4 v15.4.0

[2] TS 38.214 V15.7.0

[3] R4-2000565, Update of DL physical channels, Rohde & Schwarz, RAN4 #94-e, February 2020