3GPP RAN WG4 Meeting #94-e	R4-2000584

Online, 24th Feb – 6th Mar, 2020

Title:	LS on CSI-RS based intra-frequency and inter-frequency Measurement definition

Release:	Release 16

Work Item:	NR_CSIRS_L3meas

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:				Xuhua Tao

E-mail Address:	taoxuhua@catt.cn

- Overall Description:

RAN4 has discussed the definitions of CSI-RS based intra-frequency and inter-frequency measurements in Rel-16 WI of “RRM requirements for CSI-RS based L3 measurement” and has reached the following conclusion on their definitions:

- When CSI-RS resource of serving cell is available

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS on the serving cell and neighbor cell is the same, and

    - the CP type of CSI-RS on serving cell and target cell is the same, and

        - It is applied for SCS = 60KHz

    - the center frequency of CSI-RS on the serving cell and neighbor cell is the same

    - the BW of CSI-RS resources in the same MO is the same

  - Otherwise, it can be defined as CSI-RS based inter-frequency measurement.

- When CSI-RS resource of serving cell is not available

  - the MOs configured for CSI-RS based RRM measurement are defined as CSI-RS based inter-frequency measurement

Actions:

To RAN WG2: RAN4 kindly asks RAN2 to take the above conclusions into consideration in future work.

- Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #94					24-28 February, 2020, Athens, Greece

TSG RAN WG4 Meeting #94bis					20-24 April, 2020, China