3GPP TSG-RAN4 Meeting #94-e	 	R4-2000587

Online, 24th Feb – 6th Mar, 2020

Source:	CATT

Title:	Motivation on further enhancement for NR RRM requirement in Rel-17

Agenda Item:	13.2.4

Document for:	Discussion

Introduction

In Rel-16, the WI of NR RRM requirement enhancement is discussed to specify a list of RRM requirements in [1], however, some of new RRM requirements which has been agreed to be specified were not captured  in Rel-16 WI due to the limitation of scope and timeline, i.e. NR SRS antenna port switching requirement and second pair of BLER for RLM. Thus, in this paper, we provide the justification of WI to specify some new RRM requirements in Rel-17.

Discussion

RLM requirement enhancement

The function of RLM is to monitor the DL signal quality of the primary cell for the purpose of determining UE out-of-sync or in-sync status. The RLM design has been extensively discussed in RAN1 during many meetings. Reference [2-18] shows the broad interest of this topic in RAN1 from different companies. One of the most important aspects discussed for RLM is whether to support multiple IS/OOS PDCCH BLER pairs for supporting RLM optimization of different service like VoNR and URLLC. Finally RAN1 agreed to support configurability between two pairs of IS/OOS BLER values for RLM purposes at NR-AH#1709 meeting. A LS was also sent to RAN4 to let RAN4 to decide the second BLER pair.

Agreements: - For a cell group, - A single IS or OOS is reported by the UE - A single IS BLER is configured for a UE at time - A single OOS BLER is configured for a UE at a time - Configurable from two pairs of values for IS/OOS BLERs - Detailed pair of values up to RAN4 to decide - FFS whether the configuration is an explicit RRC configuration or implicitly derived from other parameter - FFS the case of URLLC & mMTC - Send an LS to RAN4 capturing the above agreements, and also add: - For the two pairs of values for IS/OOS BLERs, RAN1 discussed use cases such as VoIP vs. eMBB.

During the subsequent RAN4 meetings, RAN4 indeed had some discussions on this topic. However the specification work for the second IS/OOS BLER pair was not carried out due to the heavy RAN4 work load and different understanding on the purpose of the second BLER pair. Some companies thought that it was designed for VoNR while some others thought that it was for URLLC.

While From the LS in R1-1809853, very clear background information on the purpose of the second IS/OOS BLER pair was included. It has been observed in LTE, when semi-persistent scheduling is used, the service quality can be adequate also when the SINR is below the level corresponding to a PDCCH BLER of 10%. The most prominent example is for VoLTE. In these cases, it has been observed that the UE triggers RLF also when the voice quality is quite adequate. In other words, RLF may be triggered prematurely. To avoid this situation, RAN1 introduced the additional BLER threshold values for RLM.

Observation: The second IS/OOS BLER pair is reserved for VoNR service from the primary design purpose of this feature.

We think RLM is a very important feature which impact operators’ network performance and deployment cost. We propose to start a new WI in Rel-17 to define optimized performance for RLM for VoNR as intended by this feature.

If there are interests to also investigate the requirement for URLLC as well, more than two IS/OOS BLER pairs may be needed and could be studied in Rel-17.

Proposal 1: Start a Rel-17 WI on RLM enhancement for VoNR based on the second IS/OOS BLER pair.

Proposal 2: Investigate whether more than 2 IS/OOS BLER pair is needed to support RLM optimization for other service like URLLC.

SRS antenna port switching requirement

SRS antenna switching is used to obtain full DL channel information at gNB through channel reciprocity when the UE has fewer transmitting chains than receiving chains. RAN1 has agreed related agreements for SRS antenna switching which are captured as follows:

Agreements in RAN1 #NR3: - For antenna switching for SRS transmission within a carrier, at least support 1Tx (in the case of UE with 1T2R) and 2Tx (in the case of UE with 2T4R) antenna switching scheme Agreements in RAN1 #AH1801: - For sounding different antenna ports, the antenna switching time is 15 µsec. - The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. Table: The minimum guard period between two SRS resources of an SRS resource set for antenna switching SCS(KHz); Y (symbol). 15; 1. 30; 1. 60; 1. 120; 2.

In the RAN2’s spec. TS38.331, RAN2 has introduced two signaling to report UE’s capability for TX switching impact to other bands.

BandParameters-v1540 ::=            SEQUENCE {

…

srs-TxSwitch-v1540              SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                         OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                         OPTIONAL

}                                                                           OPTIONAL

}

The interruption requirement for SRS antenna switching still hasn’t been captured in RAN4 spec. Since SRS antenna switching in one carrier will possibly influence the reception/transmission for other carriers. Therefore, RAN4 need to study the interruption requirement for SRS antenna switching.

Proposal 3: RAN4 should study the interruption requirement for SRS antenna switching.

Conclusion

In this contribution, we provide the justification to specify some enhanced RRM requirements in Rel-17.

Observation: The second IS/OOS BLER pair is reserved for VoNR service from the primary design purpose of this feature.

Proposal 1: Start a Rel-17 WI on RLM enhancement for VoNR based on the second IS/OOS BLER pair.

Proposal 2: Investigate whether more than 2 IS/OOS BLER pair is needed to support RLM optimization for other service like URLLC.

Proposal 3: RAN4 should study the interruption requirement for SRS antenna switching.

References

R4-1902032, “Way forward on R15 NR RRM scope”, Intel, RAN4#90

R1-1717038,	“RLM in NR,”	ZTE, Sanechips

R1-1717055,	“Remaining details on Radio link monitoring in NR,”	Huawei, HiSilicon

R1-1717360,	“Radio Link Monitoring for NR,”	Intel Corporation

R1-1717464,	“IMR configuration for Radio link monitoring in NR,”	Vivo

R1-1717584,	“Radio link monitoring,”	Samsung

R1-1717742,	“Discussions on the IS and OOS counting procedure,”	Spreadtrum Communications

R1-1717805,	“NR Radio Link Monitoring,”	CATT

R1-1717933,	“Discussion on Radio Link Monitoring in NR,”	LG Electronics

R1-1718061,	“Remaining details on NR radio link monitoring	Guangdong,” OPPO Mobile Telecom

R1-1718186,	“Discussion on remaining details on radio link monitoring for mobility management,”	NTT DOCOMO, INC.

R1-1718321,	“Discussion on Radio Link Monitoring,”	MediaTek Inc.

R1-1718383,	“Radio link monitoring procedure and thresholds	,”AT&T

R1-1718534,	“Radio link monitoring consideration,”	Qualcomm Incorporated

R1-1718803,	“Radio Link Monitoring in NR,”	Nokia, Nokia Shanghai Bell

R1-1718718,	“Radio link monitoring for mobility management,”	Ericsson

R1-1718794,	summary of discussion for NR Radio link monitoring, Intel

R1-1809853,	LS on second BLER pair for RLM