3GPP RAN WG4 Meeting #94-e	R4-2000589

Online, 24th February – 6th March 2020

# Agenda item:	8.8.2.1.2

Source:	CATT

Title:	Discussion on RSTD Measurement requirements

Document for:	Discussion

1 Introduction

In RAN4 #93 meeting, the core requirements of positioning RRM are discussed and a way forward is approved [1]. And following agreements on RSTD measurement have been reached.

Agreements：

- Side conditions for FR1 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot = [-13] dB

  - Reference cell: PRS Es/Iot

    - Option 1: -6 dB

    - Option 2: -3 dB

- Side conditions for FR2 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot

    - Option 1: -13 dB

    - Option 2: -10 dB

  - Reference cell: PRS Es/Iot

    - Option 1: -6 dB

    - Option 2: -3 dB

- RSTD reporting range

  - The Max/Min reported values are same as in LTE for FR1 and FR2

- Core requirements shall be applicable for:

  - All PRS BW.

  - All comb patterns.

  - All repetition factors.

- Non-DRX requirements apply for RSTD measurement period, regardless of whether and which DRX configuration is configured for the UE.

- Condition under which no gaps are needed for PRS measurements:

  - If SCSs of reference and neighbor cells’ PRS are the same and the PRS are within the active BWP, and this SCS is same as active BWP SCS, then the UE can measure without gaps and without any scheduling restriction;

  - otherwise whether the UE needs gaps/scheduling restriction or not to measure PRS is FFS

- The UE shall continue RSTD measurement after each serving cell change for:

  - intra-frequency handover,

  - inter-frequency handover,

- In this case the RSTD measurement period shall be extended. Details are FFS.

According to the agreements above, there are still some issues on RSTD measurement remaining to be discussed and agreed. In this paper, we have some discussion on the remaining issues and give our proposals.

2 Discussion

2.1 Side condition

In LTE, the side condition for RSTD requirements are defined as -13dB for neighbour cells and -6dB for serving cell in [2].

From the agreements above, side conditions for FR1 PRS RSTD measurements for neighbour cell can be -13dB and most companies have reached a consensus, but for serving cell, there are two options that are -6dB and -3dB. So in the approved link-level simulation assumption [3], there are also two Es/Iot options which are [-6,-13,-13] and [-3,-13,-13] to be simulated. In our simulation, 3 cells are configured and the Es/Iots of the 3 cells are -6dB, -13dB, -13dB respectively. From the simulation results which are shown in the another paper [4], The most measurement error of DL RSTD is smaller than 70Tc which will result in about 10m positioning error except for TDL-C channel. So with the same side condition as LTE, the accuracy required in 38.855 can be achieved.

Proposal 1：Side conditions for PRS RSTD measurements are defined as follows:

- Side conditions for FR1 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot = [-13] dB

  - Reference cell: PRS Es/Iot = [-6] dB

- Side conditions for FR2 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot = [-13] dB

  - Reference cell: PRS Es/Iot = [-6] dB

2.2 Report mapping

The report mapping of RSTD include two parts, report range and report resolution. In RAN4 #93 meeting, the RSTD reporting range are agreed to be defined as the same as in LTE, but we have not reached an agreement on the report resolution of RSTD measurement.

In RAN1 #99 meeting, some progress on the reporting granularity for NR positioning measurements and reports have been made as follows and send a LS to other groups[5].

Agreement:

- The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0.

  - Note: RAN4 can determine if -1 can be a minimum value

- RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.

According to RAN1’s agreements, the reporting resolution of RSTD is defined as T = Tc2k and k is a configured parameter, this definition can represent most companies’ options on report resolution who think the report resolution is related to PRS bandwidth. So I think we can follow RAN1’s agreement on the definition of RSTD report resolution, and according to the cell coverage, the resolution Tc is enough to meet the requirements, so minimum value of k need not to be set to -1. But the parameter k should be related to DL PRS bandwidth.

Proposal 2: The reporting resolution of RSTD is defined as T = Tc2k and k is configured according to DL PRS bandwidth with a minimum value at most 0.

2.3 UE capability

In the RAN4 #93 meeting, the definition of frequency layer is discussed, and when the definition is agreed, referencing maximum allowed layers for multiple monitoring in E-UTRA specification, UE capability that the number of frequency layer UE can monitor should be discussed and defined.

And in RAN1 #99 meeting, the following agreements on maximum numbers of frequency layers and PRS resource are reached[6]：

Agreement:

- UE can be configured for DL PRS processing according to the following table:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1,4} FFS: other values |
| Max number of TRPs per frequency layer (X2) | X2=64 |  |
| Max number of PRS resource sets per TRP (X3) per frequency layer | X3=2 | Values = {1,2} |
| Max number of Resources per PRS resource set (X4) | X4=64 | FFS: values |
| Max number of DL PRS Resources per UE (X5) | NA | FFS: values |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | FFS: values |
| Max number of Resources per frequency layer (X7) | NA | FFS: values |

From the table above, RAN1 has defined the maximum number of DL PRS resources UE can configured including number of frequency layer, number of TRPs per frequency layer and number of PRS resource sets per TRP per frequency layer, etc. I think the number of frequency layer UE can monitor can be defined as {1,2,4} which represents the capability of different type of UE. As for the number of TRPs per frequency layer, it can refer to the results of system-level simulation of PRS measurement.

Proposal 3：The UE capability that the number of frequency layer UE can monitor can be defined as {1,2,4} and the number of TRPs per frequency layer is depend on the system simulation.

2.4 Measurement period

In LTE, the RSTD measurement period requirement for intra-frequency is defined as follows [2]:

,

where

is the total time for detecting and measuring at least n cells,

is the the largest value of the cell-specific positioning subframe configuration period, defined in TS 36.211 [16], among the measured n cells including the reference cell,

is the number of PRS positioning occasions as defined in Table 8.1.2.6.1-1, where  a PRS positioning occasion is as defined in clause 8.1.2.5.1,

=  ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time, and

the n cells are distributed on up to two carrier frequencies including a serving carrier frequency and one inter-frequency carrier.

Table 8.1.2.6.1-1: Number of PRS positioning occasions within

| Positioning subframe configuration period | Number of PRS positioning occasions |  |
| --- | --- | --- |
|  | f2 Note1 | f1 and f2 Note2 |
| 160 ms | 16 | 32 |
| >160 ms | 8 | 16 |
| Note 1: When inter-frequency RSTD measurements are performed over the reference cell and neighbour cells, which belong to the FDD inter-frequency carrier frequency f2. Note 2: When inter-frequency RSTD measurements are performed over the reference cell and the neighbour cells, which belong to the serving FDD carrier frequency f1 and the FDD inter-frequency carrier frequency f2 respectively. |  |  |

From the definition of RSTD measurement period requirement in LTE, the RSTD measurement period is impacted by the factors including periodicity of PRS resource, length of measurement window, number of cells UE measured, number of PRS resource UE measured in measurement window and measurement delay on each PRS resource including sampling and processing time.

In NR, the same principle and equation as LTE can be used. The difference is the periodicity of PRS is configured more flexible as follows, so the requirement of measurement window should be re-examined.

Agreement:

The following periodicity values of DL PRS resource allocation are supported depending on SCS

- 2μ{4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively

And the number of frequency layers, number of TRPs per frequency layer and number of DL PRS UE measured should be redefined referring to the simulation results of RSTD in NR.

Proposal 4：The measurement period requirements of NR can reuse the same equation as LTE.

2.5 Scheduling restriction

In RAN4 #93meeting, the agreement is reached on the scheduling restriction under PRS based measurement as follows:

Agreement:

- UE behavior on scheduling restriction in FR2 is according to the following RAN1 agreements:

  - “In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE.”

- Impact of the above rule on the scheduling restriction on PRS based measurement requirements and the priority between data and measurement in RAN4 spec are FFS.

  - If any problem is identified by RAN4 then how to resolve it is FFS.

According the above rule on the scheduling restriction, when the DL PRS is collided with other signals, the PRS will be dropped, but this might not be always appropriate. In my opinion, if the UE is in the periodic PRS transmitting and no positioning request from UE or network, then the PRS can be dropped when collision occurred. But if UE or network has sent a positioning request, then the positioning demand should be first, that is the PRS should have higher priority than other signals in this case.

3 Summary

In this paper, the requirements of  RSTD measurement are discussed, and following proposals are given：

Proposal 1：Side conditions for PRS RSTD measurements are defined as follows:

- Side conditions for FR1 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot = [-13] dB

  - Reference cell: PRS Es/Iot = [-6] dB

- Side conditions for FR2 PRS RSTD measurements

  - Neighbor cells: PRS Es/Iot = [-13] dB

  - Reference cell: PRS Es/Iot = [-6] dB

Proposal 2: The reporting resolution of RSTD is defined as T = Tc2k and k is configured according to DL PRS bandwidth with a minimum value at most 0.

Proposal 3：The UE capability that the number of frequency layer UE can monitor can be defined as {1,2,4} and the number of TRPs per frequency layer is depend on the system simulation.

Proposal 4：The measurement period requirements of NR can reuse the same equation as LTE.

4 References

- R4-1915854, Way forward on NR Positioning RRM, Ericsson

- 3GPP TS 36.133: “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management.”

- R4-1915802, Link-level simulation assumptions for PRS RSTD and PRS RSRP, Ericsson

- R4-2000592, Link-level simulation results of DL RSTD, CATT

- R1-1913522, LS on agreements related to NR Positioning

- Chairman's Notes RAN1#99 final