3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2000611

# Online, 24th February – 6th March, 2020

Source: 	CATT

Title: 	Initial simulation results for NR HST PRACH demodulation requirement with PRACH format 0

Agenda item:	8.17.2.2.2

Document for:	Agreement

- Introduction

In the recent RAN4#93 meeting, a WF [1] on NR HST BS demodulation requirements was approved. This paper will provide our evaluation results for NR HST PRACH with format 0 for simulation alignment based on agreed simulation assumptions in WF.

- Evaluation results

The agreed simulation assumptions in WF [1] are summarized and shown in Table A-1 in Annex.

The evaluation results for PRACH format 0 with restricted set type A and B are shown in Table 2-1 and Table 2-2 respectively.

Table 2-1 Evaluation results for PRACH format 0 with restricted set type A

| Format | SCS | Restricted set type | Ncs | Logical sequence index | v | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T2R | 2.1 | AWGN | 625 | 1.04us | -13.75 | -11.75 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T2R | 2.1 | AWGN | 1340 | 1.04us | -16.35 | -14.35 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T2R | 2.1 | TDLC300-100 | 400 | 2.55us | -6.67 | -4.67 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T4R | 2.1 | AWGN | 625 | 1.04us | -16.08 | -14.08 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T4R | 2.1 | AWGN | 1340 | 1.04us | -18.76 | -16.76 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T4R | 2.1 | TDLC300-100 | 400 | 2.55us | -12.51 | -10.51 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T8R | 2.1 | AWGN | 625 | 1.04us | -18.12 | -16.12 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T8R | 2.1 | AWGN | 1340 | 1.04us | -20.53 | -18.53 |
| Format 0 | 1.25kHz | Type A | 15 | 384 | 0 | 1T8R | 2.1 | TDLC300-100 | 400 | 2.55us | -14.79 | -12.79 |

Table 2-2 Evaluation results for PRACH format 0 with restricted set type B

| Format | SCS | Restricted set type | Ncs | Logical sequence index | v | Tx/Rx | Carrier frequency(GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance | SNR @ 99% detection probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T2R | 2.1 | AWGN | 625 | 1.04us | -12.86 | -10.86 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T2R | 2.1 | AWGN | 2334 | 1.04us | -15.51 | -13.51 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T2R | 2.1 | TDLC300-100 | 400 | 2.55us | -5.38 | -3.38 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T4R | 2.1 | AWGN | 625 | 1.04us | -15.33 | -13.33 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T4R | 2.1 | AWGN | 2334 | 1.04us | -18.01 | -16.01 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T4R | 2.1 | TDLC300-100 | 400 | 2.55us | -11.81 | -9.81 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T8R | 2.1 | AWGN | 625 | 1.04us | -17.34 | -15.34 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T8R | 2.1 | AWGN | 2334 | 1.04us | -19.78 | -17.78 |
| Format 0 | 1.25kHz | Type B | 15 | 30 | 30 | 1T8R | 2.1 | TDLC300-100 | 400 | 2.55us | -14.06 | -12.06 |

- Conclusion

This paper provides evaluation results for NR HST PRACH with format 0 for simulation alignment based on agreed simulation assumptions in WF.

- References

- R4-1915871, Way forward on NR HST PRACH demodulation requirement, Samsung, RAN4#93

- R4-1910984,Discussion on HST PRACH demodulation requirement, CATT, RAN4#92bis

- R4-1913726, Discussion on HSTPRACH demodulation requirement, CATT, RAN4#93

- Annex

Table A-1 Simulation assumptions for NR HST PRACH with PRACH format 0

| Parameter | Value |
| --- | --- |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Preamble format | Format 0, |
| SCS | 1.25kHz |
| Ncs | 15 |
| v | Restricted set type A: 0 Restricted set type B: 30 |
| Logical sequence index | Restricted set type A: 384 Restricted set type B: 30 |
| Restricted set type | Type A, Type B |
| Frequency offset | AWGN: - for restricted set type A: 625Hz, 1340Hz - for restricted set type B: 625Hz, 2334Hz TDLC300-100 :400Hz |
| Propagation condition | AWGN, TDLC300-100 |
| Timing error tolerance | AWGN: 1.04us TDLC300-100 : 2.55us |
| Carrier frequency (GHz) | 2.1 |
| Test metric | SNR @ 99% detection probability |