3GPP TSG-RAN WG4 (Radio) Meeting #94-e	R4-2000673

Online, 24 February – 6 March 2020

Agenda Item:	10.3.8.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.820: BS classes for 7-24 GHz frequency range

Document for:	Approval

# 1.	Introduction

The BS classes section 7.3 in TR 38.820 [1] for 7 - 24 GHz frequency range remains to be filled in. This contribution provides a TP to fill in this section in the TR.

# 2.	Discussion

Currently, the NR BS classes is defined in TS 38.104 as follows [2]:

For BS type 1-O and 2-O, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

For BS type 1-C and 1-H, BS classes are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

Note that the BS to UE minimum coupling loss of each NR BS class is the same as that of UTRAN [3] and E-UTRA [4], while the BS to UE minimum distance of each NR BS class is calculated using the path-loss formula in TR 25.951 [5]:

Path loss [dB] = 38.25 + 20 log10(d [m])

Which is derived from the well-known free-space path-loss formula [6]:

Path loss [dB] = 20 log10(4πdf/c [m])

Where d is the BS to UE distance, c is the speed of light, and f is the carrier frequency assuming to be 2 GHz.

As the carrier frequency is one of the variables in the free-space path-loss formula, the BS to UE minimum distance of each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) will need to be calculated with the corresponding BS to UE minimum coupling loss using the relevant carrier frequency, alternatively, the BS to UE minimum coupling loss of each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) will need to be calculated with the corresponding BS to UE minimum distance using the relevant carrier frequency. For example, if the BS to UE minimum coupling loss of each NR BS class is the same as that of NR, then using the example frequencies of 10, 15 and 20 GHz, the BS to UE minimum distance of each BS class is obtained as table 1 below.

Table 1: BS to UE minimum distance [m]

| Example frequency [GHz] | Wide Area BS | Medium Range BS | Local Area BS |
| --- | --- | --- | --- |
| 10 | 7.5 | 1 | 0.4 |
| 15 | 5 | 0.7 | 0.3 |
| 20 | 3.8 | 0.5 | 0.2 |

Alternatively, if the BS to UE minimum distance of each NR BS class is the same as that of NR, then using the example frequencies of 10, 15 and 20 GHz, the BS to UE minimum coupling loss of each BS class is obtained as table 2 below.

Table 2: BS to UE minimum coupling loss [dB]

| Example frequency [GHz] | Wide Area BS | Medium Range BS | Local Area BS |
| --- | --- | --- | --- |
| 10 | 83 | 66 | 58 |
| 15 | 87 | 70 | 62 |
| 20 | 89 | 72 | 64 |

Therefore, the applicable BS classes within the 7 - 24 GHz frequency range (or each frequency sub-range) should be considered, and the corresponding BS to UE minimum coupling loss and minimum distance for each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) should be decided in the WI phase. The resultant minimum coupling loss and minimum distance for each applicable BS class should then be used in the simulation to derive the corresponding requirements.

# 3.	Text proposal

<Start of change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”

[3]	3GPP TS 38.113: “NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)”

[4]	3GPP TR 38.817-02: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”

[5]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing"

[7]	Recommendation ITU-R SM.329-12: "Unwanted emissions in the spurious domain"

[8]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010 CSV: "Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test"

[9]	Hua Wang et al., "Power Amplifiers Performance Survey 2000-Present," 2008, Available: https://gems.ece.gatech.edu/PA_survey.html

[10]	3GPP TS 37.105: “Active Antenna System (AAS) Base Station (BS) transmission and reception”

[11]	Hua Wang, Fei Wang, Huy Thong Nguyen, Sensen Li, Tzu-Yuan Huang, Amr S. Ahmed, Michael Edward Duffy Smith, Naga Sasikanth Mannem, Jeongseok Lee, Edgar Garay, Sanghoon Lee, and David Munzer, "Power Amplifiers Performance Survey 2000-Present," 2019.07, https://gems.ece.gatech.edu/PA_survey.html

[12]	Code of Federal Regulations, Title 47: Telecommunication; Part 2—Frequency Allocations and Radio Treaty Matters; General Rules and Regulations, https://www.ecfr.gov/cgi-bin/text-idx?SID=983c3dd433919e69fce5a8bd4b565cfb&mc=true&node=pt47.1.2&rgn=div5

[13]	3GPP TS 38.211: “NR; Physical channels and modulation”

[14]	3GPP TS 38.213: “NR; Physical layer procedures for control”

[15]	3GPP TS 38.214: “NR; Physical layer procedures for data”

[16]	3GPP TS 38.141-1: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”

[17]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[18]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[19]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[20]	3GPP TS 38.133: “NR; Requirements for support of radio resource management”

[21]	R4-1907593: “LS on UE feature list for NR”, RAN4#91

[22]	3GPP TR 25.942: “Radio Frequency (RF) system scenarios”

[23]	The European table of frequency allocations and applications in the frequency range 8.3 kHz to 3000 GHz, ECO, https://www.ecodocdb.dk/document/593

[24]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[25]	3GPP TS 37.104: “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

[26]	Kambiz Hadipour, Andrea Ghilioni1, Andrea Mazzanti1, Matteo Bassi1, Francesco Svelto, “A 40GHz to 67GHz Bandwidth 23dB Gain 5.8dB Maximum NF mm-Wave LNA in 28nm CMOS”, 2015 IEEE Radio Frequency Integrated Circuits Symposium

[27]	Domenico Pepe, Domenico Zito, “32 dB Gain 28 nm Bulk CMOS W-Band LNA”, IEEE Microwave and Wireless Components Letters, Vol. 25, No. 1, January 2015

[28]	Domenico Pepe1, Domenico Zito, “72 GHz CMOS LNA with Transformer-based Input Integrated Matching”, IEEE 2015

[29]	Hossein Noori, Miles Sanner, Naveen Yanduru, “A 0.8 dB NF, 4.6 dBm IIP3, 1.8 - 2.2 GHz, Low-Power LNA in 130 nm RF SOI CMOS Technology”, IEEE 2015

[30]	Joost Melai, Peter Magnée, Ivo Pouwel, Pieter Weijs, Ihor Brunets, Rob van Dalen, Anurag Vohra, Luuk Tiemeijer, Ralf Pijper, Hans Tuinhout, Nicole Wils, Nicolae Cazana, “QUBiC generation 9, a new BiCMOS process optimized for mmWave applications”, IEEE 2015

[31]	Cristina Andrei, Olof Bengtsson, Ralf Doerner, Serguei A. Chevtchenko, Wolfgang Heinrich, Matthias Rudolph, “Dynamic behaviour of a Low-Noise Amplifier GaN MMIC under input power overdrive”, Proceedings of the 45th European Microwave Conference

[32]	Recommendation ERC/REC 70-03 "Relating to the use of Short Range Devices (SRD)"

[33]	ECC, ECC Recommendation (02)/05 "Unwanted Emissions"

[34]	ERC Recommendation 74-01 "Unwanted emissions in the spurious domain" (amended 29 May 2019)

[35]		LMX2594, 15 GHz Wideband RF Synthesizer, Texas Instruments. http://www.ti.com/product/LMX2594

[36]		ADF41513 26.5 GHz, Integer N/Fractional-N, PLL Synthesizer, Analog Devices, https://www.analog.com/en/products/adf41513.html

[37] 		Staffan Ek et al., A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24-30-GHz Sliding-IF 5G Transceivers, IEEE Journal of Solid-State Circuits (Volume: 53, Issue: 7 , July 2018)

[38]	K. Raczkowski et. al. “A 9.2–12.7 GHz Wideband Fractional-N Subsampling PLL in 28 nm CMOS With 280 fs RMS Jitter”, IEEE Journal of Solid-State Circuits, vol. 50, no. 5, pp. 1203-1213, May 2015

[39]	Ximenes, G. Vlachogiannakis, R. Staszewski, “An Ultracompact 9.4–14.8-GHz Transformer-Based Fractional-N All-Digital PLL in 40-nm CMOS”, IEEE Transactions on Microwave Theory and Techniques, vol. 65, no. 11, pp. 4241-4254, Nov. 2017

[40]	N. Markulic et. al. “A Self-Calibrated 10Mb/s Phase Modulator with -37.4dB EVM Based on a 10.1-to-12.4GHz, -246.6dB-FOM, Fractional-N Subsampling PLL”, IEEE International Solid-State Circuits Conference 2016, pp. 176-177

[41]	R4-1700305, "[DRAFT] LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz"

[42]	Recommendation ITU-R SM.328-11 (2006), “Spectra and bandwidth of emissions”

[43]	Recommendation ITU-R M.1580-5 (02/2014), “Generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-2000”

[44]	Recommendation ITU-R M.2070-1 (02/2017), “Generic unwanted emission characteristic of base stations using the terrestrial radio interfaces of IMT-Advanced”

[45]	ECC Recommendation (02)05, “Unwanted emissions” (Amended 30 March 2012)

[46]	ETSI TR 101 854: “Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities”

[47]	P. Ökvist, H. Asplund, A. Simonsson, B. Halvarsson, J. Medbo and N. Seifi, "15 GHz propagation properties assessed with 5G radio access prototype," 2015 IEEE 26th Annual International Symposium on Personal, Indoor, and Mobile Radio Communications (PIMRC), Hong Kong, 2015, pp. 2220-2224.

[48]	K. Zhao et al., "Channel Characteristics and User Body Effects in an Outdoor Urban Scenario at 15 and 28 GHz," in IEEE Transactions on Antennas and Propagation, vol. 65, no. 12, pp. 6534-6548, Dec. 2017.

[49]	R4-1609122, NR RF parameters and template for WP5D, RAN4 #81

[50]	R4-168770, Way forward on UE and BS estimated NF for mm-waves and ITU-R related work, RAN4 #80bis

[51]	R4-1904127, 7-24GHz discuss implications of FR1 and FR2 architectures, RAN4 #90bis

[52]	R4-165001, UE reference architecture and other considerations at millimeter wave, RAN4 #80

[53]	R4-168076, NR UE system Noise Figure proposal, RAN4 #80bis

[54]	R4-1905846, [7-24GHz] Applicable FR1 UE Technologies, RAN4 #91

[55]	RP-170021, Reply LS to ITU-R WP5D/374 (Attachment 4.13) on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz, RAN #75

[56]		3GPP TS 25.104: “Base Station (BS) radio transmission and reception (FDD)”

[57]		3GPP TS 25.951: “FDD Base Station (BS) classification”

<Next change>

## 7.3	BS classes

The BS to UE minimum coupling loss of each NR BS class [5] is the same as that of UTRAN [56] and E-UTRA [24], while the BS to UE minimum distance of each NR BS class is calculated using the path-loss formula in TR 25.951 [57]:

Path loss [dB] = 38.25 + 20 log10(d [m])

Which is derived from the well-known free-space path-loss formula [22]:

Path loss [dB] = 20 log10(4πdf/c [m])

Where d is the BS to UE distance, c is the speed of light, and f is the carrier frequency assuming to be 2 GHz.

As the carrier frequency is one of the variables in the free-space path-loss formula, the BS to UE minimum distance of each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) will need to be calculated with the corresponding BS to UE minimum coupling loss using the relevant carrier frequency, alternatively, the BS to UE minimum coupling loss of each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) will need to be calculated with the corresponding BS to UE minimum distance using the relevant carrier frequency. For example, if the BS to UE minimum coupling loss of each NR BS class is the same as that of NR, then using the example frequencies of 10, 15 and 20 GHz, the BS to UE minimum distance of each BS class is obtained as table 7.3-1 below.

Table 7.3-1: BS to UE minimum distance [m]

| Example frequency [GHz] | Wide Area BS | Medium Range BS | Local Area BS |
| --- | --- | --- | --- |
| 10 | 7.5 | 1 | 0.4 |
| 15 | 5 | 0.7 | 0.3 |
| 20 | 3.8 | 0.5 | 0.2 |

Alternatively, if the BS to UE minimum distance of each NR BS class is the same as that of NR, then using the example frequencies of 10, 15 and 20 GHz, the BS to UE minimum coupling loss of each BS class is obtained as table 7.3-2 below.

Table 7.3-2: BS to UE minimum coupling loss [dB]

| Example frequency [GHz] | Wide Area BS | Medium Range BS | Local Area BS |
| --- | --- | --- | --- |
| 10 | 83 | 66 | 58 |
| 15 | 87 | 70 | 62 |
| 20 | 89 | 72 | 64 |

Therefore, the applicable BS classes within the 7 - 24 GHz frequency range (or each frequency sub-range) should be considered, and the corresponding BS to UE minimum coupling loss and minimum distance for each BS class within the 7 - 24 GHz frequency range (or each frequency sub-range) should be decided in the WI phase. The resultant minimum coupling loss and minimum distance for each applicable BS class should then be used in the simulation to derive the corresponding requirements.

<End of change>

# References

[1]	RP-192655, “TR 38.820, Study on 7 - 24 GHz frequency range for NR”, Huawei.

[2]		3GPP TS 38.104: "NR: Base Station (BS) radio transmission and reception".

[3]		3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD)".

[4]		3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[5]		3GPP TS 25.951: "FDD Base Station (BS) classification".

[6]	3GPP TR 25.942: "Radio Frequency (RF) system scenarios".