3GPP TSG-RAN WG4 (Radio) Meeting #94	R4-2000674

Online, 24 February – 6 March 2020

Agenda Item:	10.3.8.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.820: Update of BS receiver requirements for 7-24 GHz frequency range

Document for:	Approval

# 1.	Introduction

The TP [1] to TR 38.820 [2] for BS ICS requirements for 7 - 24 GHz frequency range was approved during RAN4#93. It has been identified that there are some issues in this approved TP. This contribution provides a TP to update TR 38.820 to rectify the identified issues.

# 2.	Discussion

In the approved TP [1], the term ‘are expected to be defined’ is used in the following statements:

- For the FR1-like frequency range, the conducted requirements should not be precluded. Hence, both conducted as well as radiated requirements are expected to be defined.

- Similar to the FR1 requirements, BS class specific requirements are expected to be defined for conducted ICS.

- BS class specific requirements are expected to be defined for OTA ICS.

The term ‘expected to be defined’ can be interpreted that RAN4 has already agreed to define such requirements. On the other hand, it is stated in section 7.2.2 of the TR that:

- However, at this stage, xFR-C and xFR-H requirements cannot be ruled out for BS operating within example frequency sub-ranges 1 and 2.

Therefore, whether both conducted as well as radiated ICS requirements are defined for each frequency sub-range should be decided in the WI phase.

Moreover, the current NR BS OTA ICS requirements in FR2 are defined agnostic to the BS class [3], and indeed the BS classes applicable to the 7 - 24 GHz frequency range (or each frequency sub-range) is yet to be concluded, hence it is pre-mature to declare BS class specific ICS requirements are expected to be defined within the 7 - 24 GHz frequency range (or each frequency sub-range).

In view of the above points, it is proposed to change the term ‘are expected to be defined’ to ‘need to be considered in the WI phase’ which is commonly used term in a FS TR. The TP is provided below, where other identified errors in the receiver spurious emissions section are also corrected.

# 3.	Text proposal

<Start of change>

#### 7.4.2.6	Receiver spurious emissions

For conducted systems, receiver spurious emissions are specified separately for FDD and TDD systems.

For conducted FDD systems, as the transmitter and the receiver are both on at the same time and can be specified on separate connectors, the receiver spurious emissions should not significantly increase the total spurious emission level. The receiver spurious emission limit is therefore lower than the transmitter spurious emission limit, so the total is approximately equal to the transmitter spurious emission level.

For OTA FDD systems, the requirement is over the air and hence the transmitter and receiver cannot be separated, so there are no receiver spurious emission requirements needed.

For OTA TDD systems in applicable regions, the receiver spurious emission requirements are the same as the Category B transmitter spurious emission requirements as discussed in subclause 7.4.1.8.

In certain regions, receiver spurious emission limit will be specified during the WI based on applicable regional regulations.

#### 7.4.2.7		In-channel selectivity

##### 7.4.2.7.1 	In-channel selectivity for FR1 and FR2

In-channel selectivity (ICS) is a measure of the receiver’s ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density.

The ICS requirement applies to FR1 (conducted and radiated requirements) and FR2 (radiated requirement).

The conducted requirement is derived similar to E-UTRA requirement which assumes that the UL signal is defined for 2 users, one being the “wanted” signal and the other one being the “interfering” signal at elevated power. Following equations are used for interfering signal and wanted signal correspondingly.

Interfering signal power level = -174dBm/Hz + 10*log10(BW) + NF + ICS

Wanted signal power level = -174dBm/Hz + 10*log10(BW) + NF + SNR + IM + 3 dB

The type of interfering signal is DFT-s-OFDM NR signal and BW is corresponding to number of RBs and SCS.

25 dBc ICS is used based on 16 dB interference-over-thermal (IOT) and 9 dB SNR considering the modulation and link level simulation.

For this conducted requirement, the modulation scheme for interfering signal is assumed as 16QAM and modulation scheme for wanted signal is assumed as QPSK.

For the OTA requirement, the OTA ICS for FR1 is based on conducted requirement and an offset of ΔminSENS is used for transferring the conducted requirement to OTA requirement. However, for FR2 requirement there is no conducted requirement to be referred to. Hence the OTA ICS for FR2 is derived using the same equation as for conducted requirement, with additional offset for the gain G:

Interfering signal power level = -174dBm/Hz + 10*log10(BW) + NF + ICS - G,

Wanted signal power level = -174dBm/Hz + 10*log10(BW) + NF + SNR + IM + 3dB - G;

14 dBc ICS is used based on the simulation result of IOT with 9 dB SNR and the worst case is applied.

For radiated requirement, the modulation scheme for interfering signal is assumed as 16QAM and modulation scheme for wanted signal is assumed as QPSK. This is the same as the conducted requirement.

##### 7.4.2.7.2	 In-channel selectivity for 7 - 24 GHz

For the FR1-like frequency range, the conducted requirements should not be precluded. Hence, both conducted as well as radiated requirements need to be considered in the WI phase.

The conducted in-channel selectivity for 7 – 24 GHz will have to be re-calculated with the following considerations:

- The wanted and interfering signal levels of the ICS requirement will have to be re-calculated to account for an updated NF values and the supported set of bandwidths. For the wanted signal, the required SNR will be subject to the simulation campaigns.

- Similar to the FR1 requirements, BS class specific requirements for conducted ICS need to be considered in the WI phase.

- Selection of FRC to be used for the ICS requirement will depend on the set of supported channel bandwidths and SCS in 7 – 24 GHz. QPSK can be reused from FR1.

The throughput threshold (i.e. 95%) to derive the required SNR for the wanted signal can be reused for 7 – 24 GHz range.

For the OTA in-channel selectivity the following was concluded:

- For the OTA ICS, reuse the QPSK for the wanted signal (subject to the supported channel bandwidths and SCS in 7 – 24 GHz).

- For the FR1-like sub-range of the 7 – 24 GHz range:

  - BS class specific requirements for OTA ICS need to be considered in the WI phase.

  - The same approach of deriving the OTA requirement from the conducted one can be reused, i.e. conducted requirement plus ΔminSENS offset for both the wanted and interfering signals.

- For the FR2-like sub-range of the 7 – 24 GHz range:

  - The interfere signal and the wanted signal will be based on specific FRC, SCS and also the OTA reference sensitivity requirements.

<End of change>

# References

[1]	R4-1916111, “TP to TR 38.820 on ICS 7-24GHz”, ZTE.

[2]	RP-192655, “TR 38.820, Study on 7 - 24 GHz frequency range for NR”, Huawei.

[3]		3GPP TS 38.104: "NR: Base Station (BS) radio transmission and reception".