3GPP TSG-RAN WG4 Meeting #94-e 	R4-2000745

# Online, Feb. 24th – March 6th, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0113 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-2: Editorial addition of CBW and CABW definitions in Abbreviations section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - “Channel Bandwidth” abbreviation CBW is used throughout the spec. but not defined in the Abbreviations section. - “Cumulative Aggregated Channel Bandwidth” abbreviation CABW is used in the spec. but not defined in the abbreviations section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CBW and CABW definitions in the Abbreviations section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The abbreviations CBW and CABW are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AoA	Angle of Arrival

BCS	Bandwidth Combination Set

BPSK	Binary Phase-Shift Keying

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier aggregation

CABW	Cumulative Aggregated Channel Bandwidth

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable NR operating band

CBW	Channel Bandwidth

CC	Component carrier

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

EIRP	Effective Isotropic Radiated Power

EIS	Effective Isotropic Sensitivity

EVM	Error Vector Magnitude

FR	Frequency Range

FWA	Fixed Wireless Access

GSCN	Global Synchronization Channel Number

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

OTA	Over The Air

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving Power

Rx	Receiver

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

TPC	Transimission Power Control

TRP	Total Radiated Power

Tx	Transmitter

UE	User Equipment

UL MIMO	Uplink Multiple Antenna transmission

<<< End of changed sections>>>