3GPP RAN WG4 Meeting #94-e		R4-2000792

Online, 24th February – 6th March 2020

Agenda item:	8.16.1.2

Source:	Apple Inc.

Title:	On definition of CSI-RS based intra-frequency and inter-frequency layers

WI:	NR_CSIRS_L3meas-Core

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

In RAN4#93, different options of the definition of CSI-RS intra- and inter-frequency MO are summarized in [1]. In general, the following metric compared between serving and target frequency layers are used to classify a target frequency layer as intra-frequency or inter-frequency.

- SCS of CSI-RS

- Center frequency of CSI-RS

- Bandwidth of CSI-RS

On top of that, whether or not CSI-RS resource are contained within UE active BWP is another consideration. In this paper, further analysis is provided on the potential way to specify the definition of CSI-RS based measurements.

# 3	Definition of CSI-RS based Intra- and inter-frequency Measurements

As illustrated in Figure 1 [1], the commonality between Option 1 and 2 is CSI-RS resources in both target and serving cells are within the active BWP and have the same SCS. The only difference is about the center frequency.

| Option 1: same center frequency, SCS and within active BWP | Option 2: different center frequency, SCS and within active BWP |
| --- | --- |

The question then becomes if the center frequency of Cell2 CSI-RS resource will result in the different L3 measurement requirements. Based on our analysis, Cell2 CSI-RS L3 measurement requirements should be the same in both Option 1 and 2. Therefore, there is no obvious motivation to differentiate inter- and intra-frequency based on center frequency for CSI-RS L3 measurement.  With this, it is proposed that

Proposal 1:

- When the SCS of CSI-RS on serving cell and target cell is the same and the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as intra-frequency measurement without gap.

- When the SCS of CSI-RS on serving cell and target cell is NOT the same or the CSI-RS resource of the target cell is NOT fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as inter-frequency measurement.

In TS38.331, it is specified that

For CSI-RS resources, the network applies the procedure as follows:

-	to ensure that all CSI-RS resources configured in each measurement object have the same center frequency, (startPRB+floor(nrofPRBs/2))

That indicates MO is specified based on center frequency of CSI-RS resource. If CSI-RS based intra-frequency measurement is defined based on Proposal 1, there can be multiple MO for intra-frequency.

Proposal 2: Multiple intra-frequency measurement objects can be configured for intra-frequency measurements.

To simplify the discussion, it is worthwhile to further restricting all CSI-RS resources configured in the same measurement object not only have the same center frequency but also have the same bandwidth.

Proposal 3: In Rel-16, all CSI-RS resources configured in the same measurement object not only have the same center frequency but also have the same bandwidth.

Meanwhile, it is specified in TS38.331, within the same MO, CSI-RS resources of the target cells can be different in terms of density, bandwidth and periodicity. From this point of view, CSI-RS resource bandwidth, density and periodity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

Proposal 4: CSI-RS resource bandwidth, density and periodicity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

# 5 Conclusion

In this contribution, the definition of CSI-RS based intra- and inter-frequency measurement are analysed. It is proposed that

Proposal 1:

- When the SCS of CSI-RS on serving cell and target cell is the same and the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as intra-frequency measurement without gap.

- When the SCS of CSI-RS on serving cell and target cell is NOT the same or the CSI-RS resource of the target cell is fully contained within the active BWP of the serving cell, the corresponding CSI-RS L3 measurement is defined as inter-frequency measurement.

Proposal 2: Multiple intra-frequency measurement objects can be configured for intra-frequency measurements.

Proposal 3: In Rel-16, all CSI-RS resources configured in the same measurement object not only have the same center frequency but also have the same bandwidth.

Proposal 4: CSI-RS resource bandwidth, density and periodicity should not be used to distinguish between CSI-RS based intra- and inter-frequency measurement.

# References

[1] R4-1910956 WF on CSI-RS based RRM measurements requirements, CATT