3GPP TSG-RAN WG4 Meeting #94-e	R4-2000807

Electronic Meeting, 24th Feb - 6th Mar 2020

Source:	ZTE Corporation

Title:	Simulation results for NR HST PUSCH

Agenda Item:	8.17.2.2.1

# Document for:	Approval Introduction

In RAN4#93, we provided some initial simulation results according to the following setup based on the way forward.

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | v = 350km/h | v = 500km/h |  |
| Transform precoding | Disabled | Disabled |  |
| Number of Tx | 1 | 1 |  |
| Number of Rx | Option 2: 2 Rx (baseline) | Option 1: 2 Rx (baseline) |  |
| Number of layers | 1 | 1 |  |
| SCS | 15kHz, 30kHz | 15kHz, 30kHz |  |
| Reference signal | DMRS type 1 with 1+1+1 | DMRS type 1 with 1+1+1 |  |
| l0 | Option 1: l0 = 3 | Option 1: l0 = 3 |  |
| symbols length | 14 | 14 |  |
| start symbol index | 0 | 0 |  |
| Time domain resource allocation type | type A | type A |  |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |  |
| MCS index | 2 and 16 (both for alignment) | 2 and 16 (both for alignment) |  |
| Maximum Doppler shift | 15kHz SCS: 1340Hz | 15kHz SCS: Option 2: 1750Hz |  |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz | 15kHz SCS: [2.1GHz |  |
| Propagation condition | HST single-tap channel model: Tunnel: Ds=300m, Dmin=2m Open space: Ds=700m, Dmin=150m |  |  |
| SCS and BW | 15kHz: 10MHz |  |  |
| Timing offset | 0 |  | 0 |
| Frequency offset | 0 |  | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |  | Disabled |
| Number of HARQ transmissions | 4 RV=[0 2 3 1] |  | 4 RV=[0 2 3 1] |
| Testing metric | SNR @70% of maximum throughput |  | SNR @70% of maximum throughput |

In this contribution, more results are provided with different starting position of DMRS symbols ( l0 = 2) but the same SCS = 15 kHz. And furthermore simulation results for SCS = 30 kHz are also provided.

# Discussion

Simulation results with different starting position of DMRS symbols ( l0 = 2) are shown Table - 1.

Compared with these results in [1], the starting position of DMRS plays very little impact on the performance.

Table-1 Simulation results with DMRS ( l0 = 2)

| UE Velocity [km/h] | OPEN |  |  |  |  | TUNNEL |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1T2R |  |  | 1T8R |  | 1T2R |  |
|  | MCS2 | MCS16 | MCS2 |  | MCS16 | MCS2 | MCS16 |
| 350 | -5.66 | 6.54 | -10.99 |  | 0.2 | -5.62 | 6.13 |
| 500 | -5.64 | 6.66 | -10.97 |  | 0.44 | -5.59 | 6.49 |

Simulations with SCS = 30 kHz and different starting positions of DMRS symbols are shown in Table – 2.

Table-2 Simulation results for SCS 30 kHz

| SCS (kHz) | Antenna Config | Velocity (km/h) | HST Channel model | MCS | DMRS l0 | SNR @70% Throughput (dB) | DMRS l0 | SNR @70% Throughput (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 1T2R | 350 | tunnel | 2 | 2 | -5.63 | 3 | -5.68 |
|  |  | 350 | open | 2 | 2 | -5.91 | 3 | -5.95 |
|  |  | 350 | tunnel | 16 | 2 | 7.34 | 3 | 7.33 |
|  |  | 350 | open | 16 | 2 | 6.65 | 3 | 6.61 |
|  | 1T8R | 350 | open | 2 | 2 | -10.97 | 3 | -10.95 |
|  |  | 350 | open | 16 | 2 | 0.96 | 3 | 1 |
|  | 1T2R | 500 | tunnel | 2 | 2 | - | 3 | -5.6 |
|  |  | 500 | open | 2 | 2 | - | 3 | -5.95 |
|  |  | 500 | tunnel | 16 | 2 | - | 3 | 8.38 |
|  |  | 500 | open | 16 | 2 | - | 3 | 7.22 |
|  | 1T8R | 500 | open | 2 | 2 | - | 3 | -10.95 |
|  |  | 500 | open | 16 | 2 | - | 3 | 1.43 |

# Reference

R4-1914188 Simulation results on HST NR PUSCH, ZTE