3GPP TSG-RAN WG4 Meeting #94-e	R4-2000827

# Online, 24 Feb. – 6 Mar., 2020

Source: 	Huawei, HiSilicon, China Unicom

Title:	UE REFSENS for 50 MHz channel bandwidth for n1

Agenda Item:	9.20.1

Document for:	Approval

1	Introduction

In last RAN plenary meeting #86, new channel bandwidth for NR band n1 was approved to be added into the WID[1].

In this contribution we provide some discussion and proposal on UE REFSENS for 50 MHz channel bandwidth.

2	Discussion

UE reference sensitivity is a band specific requirement. UE reference sensitivity for 50 MHz channel bandwidth need to be defined according to the WID [1]. The UL to DL frequency gap for n1 is 190 MHz and which might cause a small impact to RX due to UL tx spectrum regrowth for wider channel bandwidth. In LTE CA_1C, RB restriction on uplink configuration is used for 15 MHz + 15 MHz and 20 MHz + 20 MHz CA cases. 129 RB/130 RB is adopted for LTE CA cases. For 25MHz, 30 MHz and 40 MHz NR channel bandwidth, RB restriction is adopted and 128 RB for 15 KHz SCS is defined for uplink configuration. I.e. to avoid reference sensitivity degradation, the wide band spectrum regrowth is minimized by RB restriction. The Foffset/UL configuration ratio for different channel bandwidths is shown in following table.

Table 2-1 Foffset/UL configuration ratio

| Foffset/UL transmission BW |  |  |
| --- | --- | --- |
| BW | UL config |  |
| 20 | 100 | 10.5 |
| 25 | 128 | 8.2 |
| 30 | 128 | 8.1 |
| 40 | 128 | 7.9 |
| 40(CA) | 100+30 | 6.8 |
| 50 | 128 | 7.7 |

From the table, when we keep the same UL configuration as 25/30/40 MHz channel bandwidth, the Foffset/UL configuration ratio for 50 MHz channel bandwidth is 7.7 which is slightly lower than 40 MHz case but it is higher the LTE CA 20MHz+20 MHz case. Hence it is proposed to use 128 configuration for 50 MHz channel bandwidth, as the same as 25/30/40 MHz channel bandwidth.

The reference sensitivity requirement can be derived by scaling on the transmission bandwidth configuration. Table 2-2 shows the reference sensitivity limits proposed for 50 MHz. And the uplink configuration is proposed in Table 2-3.

Table 2-2: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 |  |  |  |  |  |  |  | -89.6 |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  | -89.7 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  | -89.7 |  |  |  |  |  |

Table 2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n1 | 15 |  |  |  |  |  |  |  | 1281 |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  | 641 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  | 301 |  |  |  |  |  |

Proposal 1: UE reference sensitivity for n1 50 MHz channel bandwidth is defined in Table 2-2 and 2-3

3	Conclusion

Proposal 1: UE reference sensitivity for n1 50 MHz channel bandwidth is defined in Table 2-2 and 2-3

4	References

[1] RP-192801,	Revised WID on adding new channel bandwidth in NR band n1,	China Unicom

[2] R4-1916056, WF on UE RF requirements for adding channel BW to band n1, Ericsson, Skyworks Solutions Inc., Qualcomm, Huawei