3GPP TSG-RAN WG4 Meeting #94-e	R4-2000832

Online, 24 Feb. – 6 Mar., 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 38.716-02-00 for CA_n38-n66

Agenda item:	9.2.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n38-n66 as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) RP-192615 [1].

Text Proposal

---Start of changes---

6.x	CA_n38-n66

6.x.1	Common for 1 band UL and 2 bands UL CA

6.x.1.1 Operating bands for CA

Table 6.x.1-1: CA band combination of band n38 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n38-n66 | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.x.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n38+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n38A-n66A | CA_n38A-n66A | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |

6.x.1.3	Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n38-n66.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Based on above table, there is no harmonic issue for the combination.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |

Based on above table, there is no harmonics mixing issue for the band combination of n38 and n66.

6.x.1.4	∆TIB and ∆RIB values

For CA_n38-n66, the TIB,c and RIB values are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |

Table 6.x.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |

6.x.1.5	REFSENs requirements

There is neither harmonics mixing issue nor harmonics issue for the band combination of n38 and n66.

6.x.2		Specific for 2 bands UL CA

6.x.2.1		UE co-existence studies

Table 6.x.2.1-1 gives IMD interference analysis for CA_ n38-n66 with 2 ULs.

Table 6.x.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2570 | 2620 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 790 | 910 | 4280 | 4400 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 800 | 990 | 3360 | 3530 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5990 | 6180 | 6850 | 7020 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2510 | 2770 | 5930 | 6150 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7700 | 7960 | 9420 | 9570 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1580 | 1820 | 8560 | 8800 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4220 | 4550 | 8500 | 8770 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9410 | 9740 | 11990 | 12260 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 110 | 200 | 4150 | 4440 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11130 | 11420 | 10270 | 10580 |

Based on the table 6.1.x.4-1, two-tone 4th order IMD products may fall into the own Rx Band of Band n38. However, n38 is a TDD band. Hence there is no IMD issue for the combination.

Table 6.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n38-n66 | E-UTRA Band 2，4，5，10，12，13，14，17，25, 27，28，29，30, 43，50，51，66，74，85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, x |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, x, |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE x: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. |  |  |  |  |  |  |  |

6.x.2.2		REFSENS requirements

There is no IMD issue for the band combination of n38 and n66

---End of changes---

Reference

[1]	RP-192615, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation