3GPP TSG-RAN WG4 Meeting #94-e                                                R4-2000931

# E-meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	8.1.4.10

Source:	MediaTek Inc.

Title:	Discussion on SFTD measurements towards NR-U with LBT

Document for:		Discussion

- Introduction

In RAN4-93 meeting, the requirements for inter-RAT SFTD measurements towards NR-U with LBT have been discussed as WF [1]. In this paper, the views on inter-RAT SFTD measurements for NR-U are provided, including the maximum scaling on measurement period, the corresponding UE behavior, and the reporting  delay.

- Discussion

Inter-RAT SFTD

- UE behavior upon exceeding Tmeasure_SFTD_LBT_max: UE shall stop the search

  - FFS whether UE abandons the measurement

  - Tmeasure_SFTD_LBT_max = k× Tmeasure_SFTD1, k=TBD≤10

- Reporting delay: based on that the UE shall be capable of reporting SFTD at earliest upon having received the SFTD measurement configuration, and at latest X ms after the start of a period with Y consecutively available SSBs. Values of X and Y depend on the configuration in use and are FFS.

  - Rel-15 side conditions apply

For the inter-RAT SFTD requirement, the corresponding WF [1] is attached below for reference.

Regarding the maximum scaling on inter-RAT SFTD measurement period, i.e. k, it can take the scaling in EN-DC SFTD requirement as a reference. In Table 1, the number of samples used for EN-DC SFTD measurement and the corresponding maximum scaling are provided for reference, and it can observe that the maximum scaling k is ranged from 1.6 to 2.4, where the upper bound  Tmeasure_ENDC_SFTD_LBT_max = kENDC × Tmeasure_ENDC_SFTD1. For example, with max(TDRX,TSMTC)≤40, 5 samples are provided for R15 EN-DC SFTD measurement, and the corresponding NLBT-fail,max is 7 for NR-U, thus the maximum scaling kENDC = Tmeasure_ENDC_SFTD_LBT_max /Tmeasure_ENDC_SFTD1 = (5+7)/5 = 2.4.

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling on measurement period is ranged from 1.6 to 2.4.

Table 1. Maximum number of DL LBT failures for SFTD measurements.

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition | Max. scaling |
| ENDC SFTD (8.17.2 36.133) | 5 | NLBT-fail,max | 7 | max(TDRX,TSMTC)≤40 | 2.4x |
|  | 8 |  | 5 | 40<max(TDRX,TSMTC)≤320 | 1.63x |
|  | 5 |  | 3 | TDRX > 320 | 1.6x |
| Inter-RAT SFTD (8.1.2.4.25 36.133) | 14 | Tmeasure_SFTD_LBT_max | K × Tmeasure_SFTD1 | No gap, no SS-RSRP report | K |
|  | 19 |  |  | No gap, with SS-RSRP report |  |
|  | 8 |  |  | Gap, no SS-RSRP report |  |
|  | 13 |  |  | Gap, with SS-RSRP report |  |

Over-design with a large k value is undesirable for the inter-RAT SFTD measurements, because it will significantly consume UE resources to search NR-U cells within the whole SMTC window, since the SMTC offset is unknown to UE.

The maximum scaling for inter-RAT SFTD measurement period can be similar to the scaling used for EN-DC scenario, in order to cope with the similar level of LBT failure probability. In addition, considering the inter-RAT SFTD measurement period is longer than the EN-DC SFTD measurement period, a slightly small k=2 is proposed. Regarding the exceeding of the maximum scaling, UE shall not report the SFTD measurement, since the measurement result is not available within the specified measurement period.

Observation 2: The maximum scaling for inter-RAT and EN-DC SFTD can be similar, in order to cope with similar level of LBT failure probability.

Proposal 1: For inter-RAT SFTD measurement, k is 2 as the maximum scaling. UE shall not report the SFTD measurement when it exceeds the maximum extended measurement period.

Regarding the reporting delay, it seems not necessary to specify the X, since the delay has been covered by the measurement period, where UE shall be able to determine SFTD within the measurement period. And it is also unclear why the number of consecutively available SSBs should be specified in the SFTD requirement but not in the other requirements.

Proposal 2: For the reporting delay of inter-RAT SFTD measurement, X and Y are not necessary to be specified.

- Summary

In this paper, the requirements for inter-RAT SFTD measurements towards NR-U with LBT is have been discussed. We have the following observations and proposals:

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling on measurement period is ranged from 1.6 to 2.4.

Observation 2: The maximum scaling for inter-RAT and EN-DC SFTD can be similar, in order to cope with similar level of LBT failure probability.

Proposal 1: For inter-RAT SFTD measurement, k is 2 as the maximum scaling. UE shall not report the SFTD measurement when it exceeds the maximum extended measurement period.

Proposal 2: For the reporting delay of inter-RAT SFTD measurement, X and Y are not necessary to be specified.

References

[1] R4-1915777, WF on NR-U RRM Requirements, Ericsson.