3GPP RAN4 WG Meeting #94-e	R4-2000955
Online, February 24  – March 6, 2020

Agenda item:	8.14.1.1

Source:	Intel Corporation

Title:	Rel-16 signaling solution – open issues

Document for:	Approval

# Introduction

Over the past few meetings, RAN4 has been discussing enhanced solutions to avoid radio link failures (RLFs) and connection releases in Rel-16 [1-4]. During the October RAN4 meeting, we agreed to further discuss signaling based solutions while considering potential impact to RAN4 and RAN2 specifications.

RAN4 #92Bis WF [5]:

The outcome of that discussion was the agreement that a RAN2 based signaling is needed and that P-MPR will be indicated to the network [6]. An LS informing RAN2 of the latest agreements was also approved [7].

RAN4 #93 agreements [6]:

With an agreement on the solution approach, we still need to further discuss what additional information will be indicated to the network, as well as how the report will be configured. In this paper we present our views on these two aspects.

# Discussion

In our last meeting, RAN4 agreed to focus on a RAN2 based signaling solution as an enhancement to prevent potential link failures. Furthermore, MAC-CE was deemed a suitable method for the report [6]. However, to complete the new enhanced solution we still need to address a couple of open issues.

## Open issues

### What will be signaled in the report

P-MPR has already been agreed to be signaled to the network [6]. However, additional parameters like a dynamic or preferred duty cycle were left to be further discussed. Including a dynamic duty cycle is useful as it can prevent unnecessary restriction the UL duty cycle (static), and may reduce the back off taken by the UE. In order for gNB to correctly interpret the preferred UL duty cycle included in the report, this one will be referenced to Pcmax. The power headroom (PHR) will also be included in the report, as the UE is required to report Pcmax together with the PHR [8].

Observation 1: Reporting a duty cycle preference, along with P-MPR, and PHR provides gNB with the necessary information to best determine what step to take to prevent potential link failures.

Proposal 1: In addition to P-MPR, the report should include a preferred duty cycle referenced to Pcmax and the PHR.

### Report configuration: periodic or event driven (aperiodic)

A periodic configuration for the report with a designated timer makes sense from the perspective of avoiding too many reports and aligning timing parameters [9]. However, there are specific events that should, regardless of this timer, be able to trigger the report. Because of this, it makes sense to have the report be event-driven and include a periodic timer expiration as one of the triggering events. Essentially, the report will be mostly periodic, especially if the chosen timer value is small enough (ms range). But, there will also be important designated aperiodic events. This is the same approach used in the power headroom report [10]. PHR triggering conditions can be used as guideline in our discussion.

Observation 2: The report should be event driven and include a timer expiration as one of the triggering conditions.

We do need to further discuss which specific event, or combination of events, will trigger an aperiodic report. As previously discussed in [11], P-MPR exceeding a certain dB value can be one event. This may be combined with a change in environment conditions, or headroom.

Proposal 2: RAN4 should align on and finalize the full list of triggering conditions for the report. PHR triggering conditions can be used as guideline for discussion.

Configurable parameters

Again, as with the PHR, a set of configurable parameters will be needed for our report [12]. These parameters are set by gNB and will be used in the triggering conditions. Configurable parameters may include timers, factor changes, and relevant thresholds. A small periodic timer (few ms) should be sufficiently comprehensive considering the large averaging periods in MPE (2 to 4 seconds).

### Parameter values: range and granularity

- Preferred UL duty cycle: 10 to 100% (if possible, 5% steps)

- Power headroom: Should follow the same octet format used in PHR with 2 reserved bits, the remaining 6 bits yield enough granularity headroom values (64 entries)

- P-MPR: No strong preference, can be 0 to 30 dB (1 dB steps)

While the above values make sense from our perspective, we are open to discuss.

Proposal 3: RAN4 should discuss the value, range and granularity of the reported parameters.

# Conclusions

In this contribution we discussed the remaining open issues to complete the RAN2-based signaling solution for Rel-16. The following observations and proposals were made:

Observation 1: Reporting a duty cycle preference, along with P-MPR, and PHR provides gNB with the necessary information to best determine what step to take to prevent potential link failures.

Proposal 1: In addition to P-MPR, the report should include a preferred duty cycle referenced to Pcmax and the PHR.

Observation 2: The report should be event driven and include a timer expiration as one of the triggering conditions.

Proposal 2: RAN4 should align on and finalize the full list of triggering conditions for the report. PHR triggering conditions can be used as guideline for discussion.

Proposal 3: RAN4 should discuss the value, range and granularity of the reported parameters.

# References

- RP-192653, “Revised WID on NR RF Requirement Enhancements for FR2,” Nokia, Nokia Shanghai, RP#86, December 2019

- R4-1910276, “Minutes for Ad-hoc for Rel-16 FR2 WI,” Nokia, Nokia Shanghai Bell, RAN4 #92, August 2019

- R4-1913048, “Offline meeting minutes for Rel-16 FR2 MPE,” Nokia, Nokia Shanghai Bell, RAN4 #92Bis, October 2019

- R4-20xxxxx, RAN4 #93 Meeting report, February 2020

- R4-1913057, “WF on FR2 MPE Enhancement,” Qualcomm, RAN4 #92Bis, October 2019

- R4-1916170, “WF on FR2 MPE Enhancement,” Qualcomm, RAN4 #93, November 2019

- R4-1916183, “LS on MPE enhancements,” Qualcomm, RAN4 #93, November 2019

- R4-1913230, “P-MPR and headroom reporting for MPE,” Qualcomm, RAN4 #93, November 2019

- TS38.133, version 16.2.0, September 2019

- TS38.321, version 15.8.0, January 2019

- R4-1913576, “Indication and reporting enhancements to mitigate RLF,” RAN4 #93, November 2019

- TS38.331, version 15.8.0, January 2019